Chapter Four (4)

Structural Analysis and Design

4 – 1 Introduction.

4 – 2 Factored Loads.

4 - 3 Determination of thickness of  one way rib slab.

4 – 4 Topping Design.

4 – 5  Load Calculation for Rib (2B).

4 – 6 Design of rib (2B) in the second floor slab part (B) .

4 – 7 Design of Beam (16-B) in the second floor slab part( B).

4 – 8 Determination of Thickness for Two Way Rib Slab.

4 – 9  Design Two way rib slab (R4B) .

4 –10 Design of composite Beam (B14-C).

4 –11 Design of long column (C13B).

4 –12Design of one way solid slab (2) .

4 –13 Design of stair(1).

4 –14 Design of Isolated footing.(F13)

4 –15 Design of combined footing.(F24)

4 –16 Design of strip footing.

4 –17 Design of shear wall.(SH3)in PartB.

4 –18 Design of basement wall.

4 –19 Design of Mat Foundation (Mat1).

4.1:  Introduction

In This Project, there are three types of slabs: solid slabs, one-way ribbed and two-way ribbed slabs. They would be analyzed and designed by using finite element method of design, with aid of a computer Program called " ATIR- Software" to find the internal forces, deflections and moments for ribbed slabs, and then hand calculation would be made to find the required steel for some members.

The design strength provided by a member, its connections to other members, and its cross-sections in terms of flexure, and load, shear, and torsion is taken as the nominal strength calculated in accordance with the requirements and assumptions of ACI-code.

NOTE:

* fc'  = 24 MPa .

* fy  = 420 MPa .

4 .2  : Factored Loads.

The factored loads on which the structural analysis and design is based for our project members, is determined as follows:

qu = 1.2 DL + 1.6 LL                     ACI – 318 - 05

4.3 Determination of Thickness of Slabs:

4.3.1 Determination of Thickness for One Way Rib Slab:

The structure may be exposed to different loads such as dead and live loads. The value of the load depends on the structure type and the intended use.

The overall depth must satisfy ACI Table (9.5.a):

-The maximum span length for simply support (for ribs):

hmin  =  L16=527016=329.37mm            	ACI-318-05

-The maximum span length for one- end continuous (for ribs) :

hmin  =  L18.5=548018.5=296.21mm

-The maximum span length for both -end continuous (for ribs ):

hmin  =  L21=562021=267.62mm

Take slab thickness   h=350 mm  >  329.37mm.

h =35 cm  (27cm Hollow block + 8cm Topping).

4.4 Topping Design:

4.4.1 Design of Topping for Ribbed Slab:

Topping in one way ribbed slab can be considered as a strip of 1 meter width and span of hollow block length with both end fixed in the ribs.

Table 4.1 ( Dead load calculation for topping)

| KN/m | Ơ * y* 1 | Dead load from: |
| --- | --- | --- |
| 0.69 | 0.03*23*1 | Tiles |
| 0.44 | 0.02*22*1 | mortar |
| 1.12 | 0.07*16*1 | Coarse sand |
| 2 | 0.08*25*1 | topping |
| 1 | 1*1 | partitions |
| 5.25 | ∑ |  |

Live load calculation=5*1=5 KN/m

Total factor load:

Wu = (1.2 * 5.25) + (1.6 * 5)  = 14.3 KN/m

For a one meter strip Wu = 14.3 KN/m

Assume slab fixed at supported points (ribs):

Mu = wu*l212  =  14.3*0.4212= 0.194 KN.m /m of strip width.

Mn  ≥ Mu-strength condition ,

where =0.55-for plain concrete.

= 0.42* fc'* Sm           ACI-318-05

Where  Sm for rectangular section of the slab:

Sm = bh26 = 1000 * 8026 = 1066666.67mm3.

Mn = 0.42* 24 *1066666.67*10-6 = 2.195 KN.m

Mn = 0.55 * 2.195= 1.207 KN.m

Fig. (4-1) Toping of slab

Mn = 1.207 KN.m > > Mu = 0.191 KN.m

*  No reinforcement is required by analysis. Provide As, min for slabs as shrinkage and temperature  reinforcement.

shrinkage =0.0018                                                            ACI-318-05

As =  * b * t = 0.0018 * 1000 * 80 = 144 mm2 /m strip

Try  8 with As = 50.27mm2.

Bar numbers n= AsAs8 =14450.27=2.87

Use 38/m with As=150.8mm2/m  strip or  8@300mm in both direction .

Step(s) is the smallest of :

- 3h =3*80= 240mm – control.

- 450mm.

- S= 380 (280fs )-2.5Cc=380(28023420)-2.5*20=330 mm.

S ≤ 380 (280fs )=300(28023420)=300mm.

Take 8@200mm in both directions. S = 200mm.

4.4:Load  Calculation for Rib2B:

4.4.1: One - way ribbed slab.

For the one-way ribbed slabs, the total dead load to be used in the analysis and design is calculated as follows:

Fig. (4-2) One way rib slab

The effective Flange width(be) is the smallest of :

- be ≤ ln4= (5400-800)4=1150mm.

where ln is taken here as the smallest clear span of the rib.

- be  ≤  bw +16*hf= 120+16*80=1400 .

- be ≤ center to center spacing between adjacent beams

=400+120= 520 – control .

Take be =520mm.

Fig. 4-3 (Typical section in rib slab)

Table 4.2 :Dead load calculation for Rib (2B)

| No. | material | H*ơ | Kn/m |
| --- | --- | --- | --- |
| 1 | Tile | 0.03*23*0.52 | 0.359 |
| 2 | Mortar | 0.02*22*0.52 | 0.229 |
| 3 | Sand | 0.07*16*0.52 | 0.583 |
| 4 | Topping | 0.08*25*0.52 | 1.04 |
| 5 | Rib | 0.12*0.27*25 | 0.81 |
| 6 | Block | 0.27*0.4*9 | 0.972 |
| 7 | Plaster | 0.02*22*0.52 | 0.229 |
| 8 | partition | 1*0.52 | 0.52 |
| ∑ = 4.742 KN/m |  |

Live load calculations:

LL= 5*0.52 =2.6 KN/m .

Dead load / rib: DL= 4.742 KN/m.

Live Load /rib: LL = 2.6 KN/m.

4.6 Design of Rib (2B):

Fig.(4-4) Rib location in part B .

By using ATIR program we get the envelope moment and shear diagram as the follows:-

Fig. (4 - 5) Spans  length and section of rib (2B).

Fig. (4 - 6) Moment diagram for rib (2B)-(KN.m).

Fig. (4 - 7) Shear diagram for rib (2B )-(KN).

4.6.1 Design of Rib(2B) for Positive Moment :

Assume bar diameter 12 for main positive moment reinforcement .

d= h- cover – d stirrups- db2= 350-20-10-6=314mm.

»  Mu max positive for span = +25.6 kN.m

Rn= Mubd2 =  25.6*1060.9*520*3142= 0.5548 MPa.

m =fy0.85*fc' =4200.85*24 = 20.588

ρ  = 1m(1-1-2Rn*mfy )  = 120.588(1-1-2*m2*0.5548*20.588420 )

=0.001339

As = ρ*b*d  = 0.001339*(520) *(314) = 218.7mm2

*Check for As, min :

As min = 0.25*f c'(fy) * (bw) * (d)                                         ACI-318 -05

A s min =   0.25 24420* (120) * (314)=109.88 mm2.

As min = 1.4(fy)  (bw) (d)

As   =1.4420 (120) (314)  =125.6 mm2.                             - control

As =218.7mm2 > As, min = 125.6 mm2

Use 212 with As =226.2 mm2 > As req = 218.7 mm2.   -OK

* Check for Strain:

a = As*fy0.85*fc'*b =226.2*4200.85*24*520=8.955mm

c=a0.85=8.9550.85=10.54 mm

## εs = 0.003* (d-cc) = 0.003* (314-10.5410.54)= 0.864 > 0.005

## Ok…

## 4.6.2  Design of  Rib for Negative Moment   (Rib 2B):

## The maximum negative moment at the face of support  Mu = - 21.4 kN.m

Assume bar diameter 12 for main negative moment reinforcement .

d= h- cover – dstirrups- db2= 350 – 20 – 10 - 6= 314mm.

m = 20.588

Rn= Mubd2 =  21.4*1060.9*120*3142= 2.01 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 120.588(1-1-2*2.01*20.588420 )

=0.00505

As = ρ*b*d =  0.00505*(120) *(314) = 190.3mm2

*Check for As, min :

As min = 0.25*f c'(fy) * (bw) * (d)                                         ACI-318 -05

A s min =   0.25 24420* (120) * (314)=109.88 mm2.

As min = 1.4(fy)  (bw) (d)

As   =1.4420 (120) (314)  =125.6 mm2.                             - control

As =190.3mm2 > As, min = 125.6 mm2

Use 214 with As =307.8 mm2 > As req = 190.3 mm2.   -OK

* Check for Strain:

a = As*fy0.85*fc'*b =307.8*4200.85*24*120=52.8mm

c=a0.85=52.80.85=62.12 mm

## εs = 0.003* (d-cc) = 0.003* (313-62.1262.12)= 0.012 > 0.005    Ok…

4.6.3 Design of  Rib for shear    rib (2B):

ACI – 318 – Categories for shear design:

The maximum shear force at the distance d from the face of support

Vu max = -25.8 kN

Use Ф8  with two legs

Vc = (1.1)*16*fc' *bw*d = (1.1) *16*24 *120*314*10-3 =33.842 KN.

ФVc =0.75*33.84 =25.38 KN .

Vs min =116*fc' *bw*d = 116*24 *120*314*10-3 = 11.53 KN

Vs min =13 *bw*d = 13 *120*314*10-3 = 12.56 KN  …  control …

- 12 ФVc = 12 * 25.38 = 12.69KN < Vu max = 25.8 kN

- ФVc =25.38 KN < Vu max = 25.8 kN

- ФVc =25.38 KN < Vu max = 25.8 kN < Ф(Vc + Vs min ) =0.75(33.842+12.56)=34.8                … control ….

Minimum shear reinforcement is required.

Use 2 legs 8 : Av provided =2*50=100mm2.

Sreq =3Av*fytbw =3*100*420120=1050mm.

Sreq =16Av*fytbw*fc' = 16*100*420120*24 =1143mm.

Smax    ≤       d2 = 3142=157   … control ..         Smax ≤ 600 mm .

Select 2legs Ф8 @ 150mm . < S max =157mm

4.7 : Design Of  beam (17-B) :-

Fig(4 -8 ) location of beam (17-B)

4.7.1 Load calculation of beam (17-B) :

The distribution  Dead load and live loads acting  upon the Beam2 can be difined from the support reactions of the rib1B and rib 2B .

Dead load calculation :

From  R1B :

WDL from rib 1B = 31.940.52= 61.42 KN/m.      on span 1

From  R2B :

WDL from rib 2B = 29.430.52= 56.59 KN/m.     on span 2+3

b= 0.8m   -- then the own weight of the beam and plaster :

RC beam  = 0.35*25*0.8=7 KN/m             palster =0.02*22*0.8 = 0.352 KN/m

Factor dead load = (7+0.352)*1.2 = 8.82KN/m      on all span .

Live looad calculation :

From  R1B :

WLL from rib 1B = 25.23052= 48.52 KN/m.      on span 1

From  R2B :

WLL from rib 2B = 25.930.52= 49.86 KN/m.     on span 2+3

From use atir program we find the envolop moment and shear diagram .

Fig.(4 – 9)  spans  and section of beam (17-B)

Fig. (4 - 10) Moment diagram for Beam (17-B )-(KN.m).

Fig. (4 - 11) Shear diagram for Beam ( 17-B )- (KN.m).

4.7.2: Design  of  beam for  positive moment    beam(17-B) :  Mu= 286.4KN.m       for span 1

d= h- cover – dstirrups- db2= 350 – 40 – 8 - 182 = 293mm.           ...........................

m =  20.588

Rn = Mubd2  =  286.4*1060.9*800*2932= 4.63 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 120.588(1-1-2*4.63*20.588420 )

=0.013

As = ρ*b*d =  0.013*(800) *(293) = 3047.2mm2

*Check for As, min :

As min = 0.25*f c'(fy) * (bw) * (d)                                         ACI-318 -05

A s min =   0.25 24420* (800) * (293)=683.52 mm2.

As min = 1.4(fy)  (bw) (d)

As   = 1.4420 (800) (293)  =781.33 mm2.                             - control

As = 3047.2 mm2  > As, min = 781.33 mm2           OK..

No. of bar = As reqAbar18 = 3047.2254.5  = 11.9

Use 1218 in one layer  with A =3054mm2 > As req = 3047.2 mm2.   -OK

* Check for Strain:

a = As*fy0.85*fc'*b = 3054*4200.85*24*800=78.59mm

c=aβ=85.140.85=100.16 mm

## εs = 0.003* (d-cc)  = 0.003 * (293-92.4592.45) = 0.0065 > 0.005    Ok…

Positive moment     Mu= 160.9 KN.m           for span 2

Rn = Mubd2 =  160.9*1060.9*800*2932= 2.6 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 120.588(1-1-2*2.6*20.588420 )

=0.0066

As = ρ * b * d =  0.0066 * (800) *(293) = 1547.04 mm2

*Check for As, min :

As min = 0.25*f c'(fy) * (bw) * (d)                                         ACI-318 -05

A s min =   0.25 24420* (800) * (293)=683.52 mm2.

As min = 1.4(fy)  (bw) (d)

As   =1.4420 (800) (293)  =781.33 mm2.                             - control

As =1547.04mm2  > As, min = 781.33 mm2           OK..

No. of bar = As reqAbar18 = 1547.04254.5 = 6.07

Use 718 in one layer  with A =1781.5mm2 > As req = 1547.04 mm2.   -OK

* Check for Strain:

a = As*fy0.85*fc'*b =1781.5*4200.85*24*800=45.85mm

c=aβ=45.850.85=53.94 mm

## εs = 0.003* (d-cc) = 0.003* (293-53.9453.94)= 0.013> 0.005    Ok…

Positive moment   Mu= 291 KN.m           for span 3

Rn = Mubd2 =  291*1060.9*800*2932 = 4.71 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 120.588(1-1-2*4.71*20.588420 )

=0.013

As = ρ*b*d =  0.013*(800) *(293) = 3047.2 mm2

*Check for As, min :

As min = 0.25*f c'(fy) * (bw) * (d)                                         ACI-318 -05

A s min =   0.25 24420* (800) * (293)=683.52 mm2.

As min = 1.4(fy)  (bw) (d)

As   =1.4420 (800) (293)  =781.33 mm2.                             - control

As =3047.2 mm2 > As, min = 781.33 mm2           OK..

No. of bar = As reqAbar18 = 3047.2254.5 = 11.97

Use 1318 in one layer  with A =3308.5 mm2 > As req = 3047.2 mm2.   -OK

* Check for Strain:

a = As*fy0.85*fc'*b =3308.5*4200.85*24*800=85.14mm

c=aβ=85.140.85=100.16 mm

## εs = 0.003* (d-cc) = 0.003* (293-100.16100.16)= 0.0057 > 0.005    Ok…

    - : Design of  beam for negative moments   beam (17-B)  Mu= ( -274 ) KN.m

for  support 2

Rn = Mubd2 =  274*1060.9*800*2932= 4.43 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 120.588(1-1-2*4.43*20.588420 )

=0.012

As = ρ*b*d =  0.012*(800) *(293) = 2812.8 mm2

*Check for As, min :

As min = 0.25*f c'(fy) * (bw) * (d)                                         ACI-318 -05

A s min =   0.25 24420* (800) * (293)=683.52 mm2.

As min = 1.4(fy)  (bw) (d)

As   =1.4420 (800) (293)  =781.33 mm2.                             - control

As =3187.84mm2 > As, min = 781.33 mm2           OK..

No. of bar = As reqAbar18 = 2812.8254.5 = 11.05

Use 1218 in one layer  with A =3054mm2 > As req = 2812.8 mm2.   -OK

* Check for Strain:

a = As*fy0.85*fc'*b =3054*4200.85*24*800=78.59mm

c=aβ=78.590.85=92.45 mm

## εs =  0.003* (d-cc) = 0.003* (293-92.4592.45)= 0.0065 > 0.005    Ok…

Negative Moment   Mu = 278.1  KN.m      for support  3

Rn = Mubd2 =  278.1*1060.9*800*2932= 4.5 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 120.588(1-1-2*4.5*20.588420 )

=0.012

As = ρ*b*d =  0.012*(800) *(293) = 2812.8mm2

*Check for As, min :

As min = 0.25*f c'(fy) * (bw) * (d)                                         ACI-318 -05

A s min =   0.25 24420* (800) * (293)=683.52 mm2.

As min = 1.4(fy)  (bw) (d)

As   =1.4420 (800) (293)  =781.33 mm2.                             - control

As =2812.8 mm2 > As, min = 781.33 mm2           OK..

No. of bar = As reqAbar18 = 2812.8254.5 = 11.05

Use 1218 in one layer  with A =3054mm2 > As req = 2812.8 mm2.   -OK

* Check for Strain:

a = As*fy0.85*fc'*b =3054*4200.85*24*800=78.59mm

c=aβ=78.590.85=92.45 mm

## εs = 0.003* (d-cc) = 0.003* (293-92.4592.45)= 0.0065 > 0.005    Ok…

4.7.4 Design of Beam for  shear    Beam (17-B) :

ACI – 318 – Categories for shear design: critical section of distance d= 293mm from the face of support .

Vu critical = 311.2 kN    at support 2 .

Use Ф8 with 4 legs

Vc =16 * fc' *bw*d = 16 * 24 *800*293*10-3= 191.38 KN

Check for section dimension:

Vs= Vu - Vc= 311.20.75 -191.38 = 223.55KN

Vs max = 32 * fc' *bw*d = 23 *24  *800*293*10-3 = 765.54 KN

Vs < Vs max  --- the section is large enough .

Find the maximum stirrups spacing :

Vs'= 13 * fc' *b w*d =13 *24  *800*293*10-3 =382.77 KN

Vs < Vs'  then

Smax ≤ 600mm   or     Smax ≤  d2 = 2932 =146.5 mm

Check  for  Vs min  :

Vs min  = 116 * fc' * bw* d =  116  *24  *800* 293 * 10-3 =71.77 KN

Vs min  =     13  * bw*d = 13 * 800 * 293 * 10-3 = 78.13 KN   … control …

( Vc + Vs min  ) = 0.75 ( 191.38 + 78.13)=202.13 KN

( Vc + Vs' ) =  0.75 (191.38+ 382.77)= 430.61KN

( Vc + Vs min  )  =202.13≤ Vu =311.2 ≤    ( Vc + Vs' )=430.61

Or      Vs min = 78.13  ≤ Vs =223.55  ≤  Vs' =382.77

Then it      case IV   or      Items 4

Use stirrups 2U – shape (4 legs stirrups) 8

With   Av = 4* 50.27 = 201.1 mm2 .

S req  = Avfyt*dVs =201.1*420*293*10-3223.55 = 110.7 mm

Take 2U shape (4 legs stirrups ) 8 @100mm< Sreq < Smax

S req < Smax =146.5mm

Vu critical = 310.7 kN           at support 3

Use Ф8 with 4 legs

Vc =16 * fc' *bw*d = 16 * 24 *800*293*10-3= 191.38 KN

Check for section dimension :

Vs= Vu - Vc= 310.70.75 - 191.38 = 222.88 KN

Vs max = 23 * fc' *bw*d = 32 *24  *800*293*10-3 = 765.54 KN

Vs < Vs max  --- the section is large enough

Find the maximum stirrups spacing :

Vs'= 13 * fc' *bw*d =13 *24  *800*293*10-3 =382.77 KN

Vs < Vs'  then

Smax ≤ 600mm   or     Smax ≤  d2 = 2932 =146.5 mm

Check  for  Vs min  :

Vs min  = 116 * fc' *bw*d =116 *24  *800*293*10-3 =71.77 KN

Vs min  =     13  * bw*d = 13 *800 * 293 * 10-3 =78.13 KN   … control …

( Vc + Vs min  )  = 0.75 ( 191.38 + 78.13 ) = 202.13 KN

( Vc + Vs' ) =  0.75 (191.38 + 382.77 ) = 430.61KN

( Vc + Vs min  ) =202.13 ≤ Vu =310.7  ≤    ( Vc + Vs' )=430.61

Or      Vs min = 78.13 ≤ Vs = 222.62   ≤   Vs' =382.77

Then it case IV   or    Items 4

Use stirrups 2U – shape (4 legs stirrups) 8

With   Av = 4* 50.27 = 201.1  mm2 .

S req  = Avfyt*dVs =201.1*420*293*10-3222.88 = 111.03 mm

S req <  Smax =146.5mm

Take 2U shape (4 legs stirrups ) 8 @100 mm< Sreq < Smax

4.8   Determination of Thickness for Two Way Rib Slab:

*Minimum thickness (deflection requirements): Assume the thickness for the shown ribbed slab (R4B)  . h=35.

*All Exterior and Interior beam have

rectangular section of 40,80cm

respectively and 35cm depth for both.

I b, exterior=  b*h3  12=  40*353 12 =142916.66 cm3.

I b , interior = b*h3 12 = 80*35312= 285833.33 cm3.

**Slab section for Exterior beam:

The moment of ribbed slab is the sum of

Moment of inertia of T-section ribs

Within distance ( L 2 + bw ) , bf  = 52 cm ,              Fig (4.12) Tow way rib slab location .

was  defined as one-way ribbed slab design (bf = be)

Yc= 40*8*4+35*12*17.540*8+35*12=11.66cm.

Irib= 52*11.6633-40*3.6633+12*23.3433=77682.21 cm3 .

**(Exterior beams):

Short direction L= 10.1 = 1010 cm.

Is= Irib*(L/2+bw)bf = 77682.21*(10102+40)52 =814169.31 cm3 .

Long direction  L= 10.2m = 1020cm.

Is= Irib*(L/2+bw)bf = 77682.21*( 10202+40)52 =821638.75 cm3 .

*** Slab section for (Interior beam) :

Short direction      L right = 10.1 m =1010 cm , L left = 4.7m = 470cm.

Is= I rib * Lright2+Lleft2+bwbf= 77682.21* 10102+4702+8052=1224988.7 cm3.

Long  direction      L right =10.2 m=1020 cm ,  L left = 4.6m = 460cm.

Is= I rib * Lright2+Lleft2+bwbf = 77682.21* 10202+4602+8052= 1224988.7 cm3.

f= IbIs

f1= IbIs =28533.331224988.7=0.23         Interier  – short direction.

f2= IbIs =142916.66821638.75=0.17         Exterior – long direction.

f3= IbIs =142916.66814169.31=0.17         Exterior – short  direction.

f4= IbIs =285833.331224988.7=0.23         Interior  – Long  direction.

∑fm = IbIs =0.23+0.17+0.17+0.234=0.2 = 0.2

The  minimum thickness of slab without interior beams.  = ln33=1020033=309.1mm<350mm.          or  ,…

h = ln*(0.8+fy1400)36+5β*(fm-0.2)  =  10200*(0.8+4201400)36+5*1.01*(0.2-0.2)=311.66mm>125mm  Ok

β=L, LongL,short=10.210.1=1.01

Take slab thickness h slab = 350 mm.

80 mm- topping , 270 mm concrete block.

4.8.1 Load calculation:

Table 4.3 ( Dead load calculation for two way slab )

| Material | Quality Density KN/m3 |  | W KN |
| --- | --- | --- | --- |
| Tiles | 23 | 23*0.03*052*052 | 0.186 |
| Mortar | 22 | 22*0.02*0.52*0.52 | 0118 |
| Sand | 16 | 16*0.07*0.52*0.52 | 0.3 |
| Reinforced Concrete Topping | 25 | 25*0.08*0.52*0.52 | 0.541 |
| RC. Rib | 25 | 25*0.27*0.12(0.52+0.4) | 0.74 |
| Concrete Block | 9 | 9*0.27*0.4*0.4 | 0.388 |
| Plaster | 22 | 22*0.02*0.52*0.52 | 0.118 |
| Partitions | 1 | 1*0.52*0.52 | 0.27 |
|  |  | ∑ = | 2.661 KN |

Dead Load of slab :

DL =  2.6610.52*0.52 =9.83KN/m2.

WD=  1.2 * 9.83  =11.79 KN/m2.

Live Load of Slab :

LL =   1.6 * 5  = 8 KN/m2.

W total= 11.79 + 8  = 19.79 KN/m2.

4.8.2   Moments calculations:

Ma= Ca* W * La2 * bf         and   Mb =  Cb * W * Lb2 * bf

lalb=  10.110.2=0.99

*** Negative moments at continuous edges  (Table1)    case (4)

C a , neg ( lalb = 0.99 ) = 0.0548

C b , neg  (lalb=0.99) =0.049

Ma, neg =  0.0548 *19.79 * 10.12 * 0.52 = 57.52 KN.m

Mb ,neg  = 0.049 * 19.79 * 10.22 * 0.52=52.46 KN.m

***  Positive moments:   (Table2+ Table 3):

Ca, D (lalb=0.99) = 0.0276

Ma, pos,D =  0.0276 * 11.79 * 10.12  * 0.52  = 17.26  KN.m

Ca,L ( lalb= 0.99 ) = 0.0326

Ma, pos,L= 0.0326 * 8 * 10.12  * 0.52  = 13.83 KN.m

Ma, pos = Ma ,pos ,D  + Ma ,pos ,L =  17.26 + 13.83 = 31.09kN.m

C b , D (lalb=0.99) = 0.0264

Mb, pos,D = 0.0264 * 11.79 * 10.22  * 0.52  = 16.84  KN.m

C b, L (lalb=0.99) = 0.0314

Mb, pos,L = 0.0314 * 8 * 10.22  * 0.52  = 13.59 KN.m

Mb, pos =M b ,pos ,D +M b ,pos ,L= 16.84+13.59=30.43kN.m

***  Negative moment at Discontinuous edges  = (13*positive moment )

Ma , neg = 13 * 31.09 = 10.36 KN.m

Mb , neg =  13 * 30.43= 10.14 KN.m

  - Design two way rib slab (R4B)

4.9.1   for negative moment: cont. edge

Mu = 57.52 KN.m

Assume bar diameter Ø14 for main Reinforced concrete section.

d = h – 20 -  ds  - db2  = 350 – 20 – 8 - 7  =315mm

c= 37*315=135

a = β * c  =  0.85 * 135 = 114.75 mm

Mn max = 0.85 fc'*a*b*(d- a2)

Mn max = 0.85*24*114.75*120(315-114.752)*10-6=72.36

Ø=0.82

Mu=57.52KN.m < ØMn=0.82*72.36=59.34 KN

*** Design the section as doubly concrete reinforced …

Mns = Mu-Mnc=57.520.82-59.34=10.8

d' =cover+ Ø stirrups+ Øbar2=20 +8+142=35mm

fs' =600(c-d'c) = 600(135-35 135)=444.4 > fy=420 MPa

compression steel is yielded           fs'= fy= 420MPa

As' =Mns fy-0.85fc'(d-d') = 108*106 420-0.85*24315-35=96.52

T= Cc +  Cs=0.85* fc'*a*b + As'(fy-0.85fc') =((0.85*24*114.75*120+96.5(420-0.85*24))*10-3 = 319.3KN

As= =Tfy=319.3*1013240 = 760.2 mm2

Take 222 with A s ,provided=760. 3mm2>As req=760.2 mm2 Ok

Take 212 with A's provided=226.2mm2>A's req=96.52 mm2 Ok

Check whether compression steel has yielded

ρ = Asb*d  = 760.3311*120=0.02

ρ' =A'sb*d = 226.2311*120=0.006

ρcy'= 0.85 fc'd'βd fy 600600-fy+ρ'  =0.85*24*34*085311*420*600600-420+0.006=0.02

ρ  ≥   ρcy'   compression steel is yielded ( εs ' ≥  εy)

Check for strain εs   ≥ 0.005:

a =Asfy-As'fy-0.85fc'0.85fc'* b= 760.3*420-226.2(420-0.85*24)0.85*24*120=93.5

c=  a  β = 93.430.85=109.9

εs =0.003 ( d-cc  ) = 0.003 (311-109.9109.9)=0.0055 > 0.005   Ok

4.9.2  Analysis the section rib for different two bars(2):

* **  Assume 214 for bottom bars (+ve moment )at mid span .

d =  h – 20 - ds  -  db2 =  350 – 20 – 8 – 7 =315 mm

As 214 =2 * 153.9 = 307.9 mm2  > As min = 1.4420 * 120 * 315 = 126mm2

a = As*fy0.85*fc'*b= 307.9*4200.85*24*520=12.19 mm

C=aβ== 12.190.85=14.34 mm

εs = 0.003 ( d-cc ) = 0.003 ( 315-14.3414.34 )=0.06 > 0.005   Ok

Mn=*fy*As*(d-a2)=0.9*420*307.9*(315-12.192)*10-6=35.95>Mu=31.09KN.m

** * Assume 212 for top bars (-ve moment ) at discount. edge .

d = h-20-ds-db2= 350-20-8-6=316mm

As 212 =2*113.1=226.2 mm2 > As min =1.4420*120*316=126.4mm2

a= As*fy0.85*fc'*b= 226.2*4200.85*24*120=38.8 mm

C= a  β=38.80.85=45.65 mm

εs = 0.003(d-cc) = 0.003 (316-45.6545.65)=0.017 >0.005   ok

Mn =*fy*As*(d-a2)=0.9*420*226.2*(316 -45.652)*10-6=25.1 > Mu=10.36KN.m

4.9.3  Design two way rib slab for shear  :

Case (4)

Wa = 0.51            Wb =0.49

**total load on the panel being = 10.2*10.1*19.79=2038.76KN

*Load per rib at face of long beam is

Vu,dface=0.51*2038.76*0.52/ 2*10.2= 26.5KN   in long  …  control …

Vu,d face= 0.49*2038.76*0.52/ 2*10.1= 25.7 KN   in short

The shear strength of one rib in slab is

Vc = (1.1) * 16 * fc' * bw *d = (1.1) * 16* 24  * 120 * 312 * 10-3 =33.62 KN.

ФVc =0.75 * 33.62 = 25.22 KN .

Vs min = 116*fc' *bw* d  = 116 * 24  * 120 * 312 * 10-3 = 11.46 KN

Vs min = 13 *bw*d = 13 * 120 * 312 * 10-3 = 12.48 KN     …  control …

- 12 ФVc = 12 * 25.22 = 12.61KN < Vu max = 26.5 kN

- ФVc =25.22 KN < Vu max = 26.5 kN

- ФVc =25.22 KN < Vu max = 26.5kN < Ф(Vc + Vs min ) =0.75(33.62+12.48)=34.57                … control ….     Case 3

Minimum shear reinforcement is required.

Assume  Ф8 @ 250 mm

Av min  = 116*fc' *bw*Sfyt =116*24 *120*250420=21.87mm2.

Av provided = 2*50= 100 mm2 .

S    ≤       d2 = 3122=156   … control ..         d ≤ 600 mm .

Take 2leg stirrup , U-shape 8@150mm < Smax =156mm

4.10 Design of composite Beam (B14-C)

Calculation of load for beam:

DL(slab) = 22.3 KN/m

DL(beam weight) = 3 KN/m

DL(beam concrete)= 0.25* 0.5 * 25=3.1 KN/m

Total Dead load =  22.3 + 3 + 3.1 = 28.4 KN/m

Total Live load  = 22.1 KN/m

Figure (4-13): shear and moment for composite beam

Determination width of flange (effective width):

Be = L8  = 21.37  8 = 2.7m

Be=1.8m

Determination of beam section (As):

Take   be = 1.8m           ts = 25cm           Mu = 2882.8 KN/m

a=250 mm

d=  3812 + 250 = 440.5 mm

Mn = As * Fy * (d - a2)

2882.80.9 = As * 345 * (440.5-250/2)

Asreq =0.0294 m2

Select W14*159       (A572 Grade 50)

A = 46.7 in2 =  0.0301 m2

d = 15 in =  381 mm

tw = 0.745 in = 18.9 mm

bf = 15.6 in = 396.2 mm

tf = 1.19 in = 30.2mm

Fy = 50 Ksi = 345MPa

T = C

As * Fy  = 0.85 fc *be*a

0.0301*345 = 0.85*40*1.8 *a

a = 170  mm

so the neutral axis in the slab

Cc = 0.85 fc *be * a

Cc =  0.85* 40*1800 * 170 = 10404 KN

Ts = As * Fy

Ts = 0.301 * 345 =10384.5 KN

Mn = Ts ( d2+ ts - a2)

= 10384.5 ( 381 2+ 250 - 1702)

= 3691.7 KN.m

ØMn = 0.9 * 3691.7 = 3322.5 KN.m    >  Mu = 2882.8 KN.m     OK

Figure (4-14): detail for composite beam

Connector design :

Select MC6*18

A = 5.29 in2 = 3412.9 mm2

d = 6 in = 152.4  mm

tw = 0.379 in = 9.63 mm

bf = 3.5 in = 88.9 mm

tf = 0.475 in = 12.06 mm

Qn = 0.3(tf +0.5 tw) Lc fc*Ec

Ec = 4700fc = 470040 = 29725.4 MPa

Assume  Lc = 300  mm

Qn = 0.3 (12.06 +0.5*9.63)*250*40*29725.4

Qn = 1380 KN

Cmax = 0.85fc * be * ts

= 0.85 *40* 1800 * 250

= 15300 KN

Tmax =  As *  Fy

= 0.0301*345* 103

= 10384.5 KN

N = CmaxQn = Tmax Qn  which smaller

N =10384.51380  = 7.6

Use 8 channel per half span

Welding Calculation :

Type of welding is fillet welding

Select a = 8 mm

amin = 6 mm

amax = 12.06-1.6 = 10.46 mm

te = 0.707a

= 0.707*8 = 5.656  mm

Shear action :

ØRnw = 0.75* te *0.6 * Fuw

Fuw = 70Ksi = 483 MPa

ØRnw = 0.75*5.656*0.6 * 483

= 1.23kN/mm

Qn per one channel = 10384.516 = 649 KN

L =  QnØRnw = 6491.23 = 527.6mm

Permeter of channel = 2*250 + 2*88.9 = 678mm

The welding is for all perimeter .

4 .11 Design of long column(C13B).

Figure (4-15): Location of column (C13B)

4.9.1 Load Calculation:

4.9.2 Check Slenderness Effect:

- In 0.6 m-Dirction

Lu: Actual unsupported (unbraced) length.

K: effective length factor (K= 1 for braced frame).

R: radius of gyration = 0.3 h =

Lu = 3.15 m

M1M2 =1

K=1 , According to ACI 318-2002 (10.10.6.3) The effective length factor, k, shall be permitted to be taken as 1.0.

- In 0.4 m-Dirction

Lu: Actual unsupported (un braced) length.

K: effective length factor (K= 1 for braced frame).

R: radius of gyration = 0.3 h =

Lu = 3.15  m

M1 M2=1

K=1 , According to ACI 318-02 (10.10.6.3) The effective length factor, k, shall be permitted to be taken as 1.0.

With As = 24624mm2 > As reg =2400mm2 .       ok.

- Check for spacing between the bar

S =

S = 83.2  ≥ 40

≥ 1.5db = 21        Ok

4.9.3 Design of the Tie Reinforcement:

Figure (4-16):  Reinforcement of column (C13B)

4 .12  Design of one way solid slab (2) .

Figure (4-17):  one way solid slab (2) .

Minimum slab thickness for deflection.

hmin= l24=6.2724=0.26m    take h min = 250 mm

=than-1(0.320.85)=20.63

Flight Dead load computation :

Table 4.4 :Dead load calculation for flight of solid slab(2)

| Material | KN/m3 | W kN/m |
| --- | --- | --- |
| Tiles | 23 | 23 (0.32+0.90.85 ) * 0.03 *1=0.99 |
| Mortar | 22 | 22*( 0.32+0.850.85 )*0.02*1=0.61 |
| Stair steps | 25 | 250.85*(0.32+0.852)*1=4 |
| R.C solid slab | 25 | 25*0.25*1cos20.63 =6.67 |
| Plaster | 22 | 22*0.02*1cos20.63 =0.47 |
| Total Dead Load | ∑ | 12.74 |

Landing Dead load computation;

Table 4.5 : Dead load calculation for Landing of solid slab (2)

| Material | KN/m3 | W KN/m |
| --- | --- | --- |
| Tiles | 23 | 23*0.03*1=0.69 |
| Mortar | 22 | 22*0.02*1=0.44 |
| R.C solid slab | 25 | 25*025*1=6.25 |
| Plaster | 22 | 22*0.02*1=0.44 |
| Total Dead load | ∑ | 7.82 |

*live load= LL=5 KN/m2

*Total factor load    w = 1.2*dl+1.6*LL

For flight   w =1.2*12.74+1.6*5=23.28

For landing   w = 1.2*7.82+1.6*5=17.38

Figure (4-18):  geometry, load, and moment, shear, for solid slab (2)

** Check for shear strength :

Use  14 for main reinforcement

d= 250 - 20 - 142 =223mm

Vc = (16) *24 * 1000 * 223 *10-3 = 108.047 KN.

=075  for shear.        Vc = 0.75*108.07=81.05  KN/1m strip .

Vu max  =73.4 KN/1m strip < Vc=81.05 KN/1mstrip .

The thickness of the slab is adequate enough

**Design for flexural ;

Maximum bending moment and steel reinforcement;   Mu=67.6KN

Rn= Mubd2 =  67.6*1060.9*1000*2232=  1.51  MPa.

m =fy0.85*fc' =4200.85*40 = 12.35

ρ  = 1m(1-1-2Rn*mfy )  = 112.35(1-1-2*1.51*12.35420 )=0.0036

As = ρ*b*d  = 0.0036*(1000) *(223) = 802.8 mm2

Asmin=0.0018*b*h=0.0018*1000*250= 450 mm2

As   > As min       Ok

Use 14 then    n = As As14  = 5.22          s= 1n = 15.22  = 0.19

Take 614/m     with As=923.4/m strip   or 14@150mm

Step(s) is the smallest of:

(1)   3h = 3 * 250 = 750 mm          (2)   450 mm

(3)    s = 380  (28023 * 420) -  2.5* 20 = 330 mm

(3)     s ≤ 300 ( 28023 * 420 ) = 300 mm      ….control .

S= 150mm<S max =300mm          ok

**for positive moment   Mu=27.4 KN.m

Rn= Mubd2 =  27.4*1060.9*1000*2232= 0.61 MPa.

m =fy0.85*fc' =4200.85*40 = 12.35

ρ  = 1m(1-1-2Rn*mfy )  = 112.35(1-1-2*0.61*12.35420 )=0.0014

As = ρ * b * d  = 0.0014 * ( 1000) *(223) = 312.2 mm2

As min=0.0018 * b * h  =0.0018 *1000*250=450 mm2

As < As min       notOk        Take AS min=450 mm2                  use 14

n= 450153.9 = 2.9        s= 12.9 = 0.34m         use  314/m    with As=461.7mm2

or 14@300mm                   S=300≤Smax=300mm              ok

*** Design of Negative moment :               Mu = -70.6  KN.m

Rn= Mubd2 =  70.6*1060.9*1000*2232= 1.57 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 112.35(1-1-2*1.57*12.35420 )=0.0038

As  = ρ * b * d  =  0.0038 * (1000)  * (223)  =   847.4 mm2

As min = 0.0018 * b * h = 0.0018 * 1000 * 250  = 450 mm2

As  >  As min       Ok                        use 14

n=  847.4153.9  =5.51        s= 15.51 = 0.181m         use  614/m    with As=923.4mm2

or  14@150mm                   S =150 ≤ Smax=300mm              …ok

*** As(Temperature and shrinkage )  = 0.0018 *b *h=0.0018*1000*250=450mm2.

n=AsAs14 = 450153.9 = 2.92               s= 1n = 12.92 = 0.34m

Take 314/m     with As= 461.7mm2/m strip          or 14@300mm

Step (s-for shrinkage and temperature reinforcement ) is the smallest of:

(1)    5h=5*250=1250mm                (2)   450mm     …control

S  =  300  mm < S max = 450  mm   … OK

Figure (4-19): reinforcement detail for solid slab(2)

4 .13 Design of stair(1).

Figure (4-20):  geometry of stair (1)

** minimum slab thickness for deflection (for simply supported one way solid slab)      hmin =  L20 =  5.2120 = 0.261 m =26.1cm

Take h min= 250mm.

*** Flight Dead Load computations;

= tan-1 (riserun) = tan-1( 1630 ) =28.07

Table 4.6:Dead load calculation for flight of stair (1)

| material | KN/m3 | W kN/m |
| --- | --- | --- |
| Tiles | 23 | 23( 0.16+0.350.3 )*0.03*1=1.173 |
| Mortar | 22 | 22*( 0.16+0.30.3 )*0.02*1=0.67 |
| Stair steps | 25 | 250.3*( 0.16+0.32 )*1=2 |
| R.C solid slab | 25 | 25*0.25*1cos28.07 =7.08 |
| Plaster | 22 | 22*0.02*1cos28.07 =0.49 |
| Total Dead Load | ∑ | 11.41 |

Landing Dead load computation;

Table 4.7 :Dead load calculation for landing of stair (1)

| Material | KN/m3 | W KN/m |
| --- | --- | --- |
| Tiles | 23 | 23*0.03*1=0.69 |
| Mortar | 22 | 22*0.02*1=0.44 |
| R.C solid slab | 25 | 25*0.25*1=6.25 |
| Plaster | 22 | 22*0.02*1=0.44 |
| Total Dead load | ∑ | 7.82 |

*live load= LL=5KN/m2

Figure (4-21): loads on flight of stair (1)

Figure (4-22): moment and shear diagram for flight of stair(1)

**check for shear strength (assume bar diameter 14 for main reinforcement   d= h-  20  - dp2= 250- 20 - 142 =223mm.

Vu max  at distance d from face of support    Vu max=40.9KN.

Vc =(16) 24 *1000 * 223 * 10-3 = 182.07 KN.

=0.75 _  for shear .         Vc=0.75*182.07 =136.55KN.

Vumax= 40.9 < (12) Vc = 136.552 = 68.27KN.

The thickness of the slab is adequate enough.

**** Design for flexural:        Mu=62.2KN.

Assume bar 14                   d=223mm.

Rn= Mubd2 =  62.2*1060.9*1000*2232= 1.38 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 112.35(1-1-2*1.38*12.35420 )=0.0034

As = ρ * b * d   =  0.0034 * (1000)  * ( 223) = 758.2 mm2

Asmin=  0.0018 * b * h  =  0.0018 * 1000* 250  = 450 mm2

As > Asmin       Ok                        use 14

n= 758.2153.9 = 4.93        S= 14.93 = 0.20m         use  514/m    with As=769.5mm2

or 14@200mm                   *** Step(s) is the smallest of:

(1) 3h=3*250=750mm      (2) 450mm

(3)  s=380 ( 28023 * 420 ) - 2.5*20 = 330 mm

(3)    S≤ 300(280  23 * 420) = 300 mm      control .

S= 200 mm < Smax=300 mm          ok

*** As(Temperature and shrinkage ) = 0.0018*b*h=0.0018*1000*250=450mm2.

n= AsAs14  = 450153.9 = 2.92               s = 1n  = 12.92 = 0.34m

Take 314/m     with As= 461.7 mm2 / m strip          or 14@300mm

Step (s-for shrinkage and temperature reinforcement ) is the smallest of:

(1)  5h=5*250=1250mm               (2) 450mm     …control    >300 mm  ….OK

Figure (4-23): reinforcement detail of stair (1)

***Design of landings..

Figure (4-24):  loads on landing of stair(1)

Figure (4-25): moment and shear diagram for landing of stair (1)

**check for shear strength    Vu= 15.6 KN < ( 12 ) Vc = 68.27 KN.

The thickness of the slab is adequate enough.

**for flexural :   Mu= 20KN.m

Rn= Mubd2 =  20*1060.9*1000*2232=  0.44 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 112.35(1-1-2*0.44*12.35420 )=0.0011

As =  ρ * b * d   =  0.0011  *  (1000)  * (223)  =  245.3  mm2

Asmin = 0.0018 * b * h = 0.0018 * 1000 * 250  =  450  mm2

As < Asmin      not Ok                        Take Asmin = 450  mm2 .         use 14

n = 450153.9 = 2.92        S= 12.92 = 0.342m         use  314/m    with As=769.5mm2

or 14@300mm .                  S= 300mm ≤ Smax=300mm          ok

Figure (4-26): reinforcement detail for landing of stair (1)

4 .14 Design of Isolated footing.(F13).

** load calculation :

From column :

Service dead Load  = 2391.66KN

Service live load = 1231.25KN

Soil density =18KN/m3                    soil depth =  0.7m          surcharge 5KN/m2

Allowable soil pressure =450 KN/m2

Assume footing to be about (80cm) thick

A =   force service qall,,net  =    2391.66+1231.25450-0.7*18-0.8*25-5  = 8.78 m2.

A = L2        L = A= 8.78  = 2.96 m                 Take   L=3 m.

***Depth of footing and shear design:

Pu=1.2*2391.66+1.6*1231.25=4840KN

qu= 48403*3  =  537.77  KN/m3

****Check one-way shear (Beam shear)

Figure (4-27):  one way shear (beam shear for isolated footing)

Vu at distance d from the face of support :

Vu=qu*b (  L2 - a2 – d )= 537.77*3* ( 32 - 0.62 – d )            let Vu=Vc , (=0.75)

Vc=( 16 ) fc' bw * d= ( 16 ) * 24    * 3000 * d

537.77*3*(32-0.62-d ) =0.75* (16)*24    * 3000 * d                dreq=0.561m

d aval = 800-75-20= 705mm = 0.705m > dreq =0.561 m               …. Ok

****check Tow –way shear (punching shear) :

Figure (4-28): Tow- way shear (punching shear)

Let Vu=Vc,      (=0.75)

Vu = 537.77 ( 3*3 - ( 0.6 + 0.705 )( 0.6+0.705) = 3924.09 KN

β  = 600600=1  , b0 =  2( 0.6 + 0.705 )+ 2 ( 0.6 + 0.705 ) = 5.22 m

Vc = (  16 ) (1+2β  )fc'  * b0 *d                where ( 16 )( 1 + 21) = 0.5

Vc=(  112 )*( s * d bo + 2 )  fc' * b0 *d      where (112)*( ( 40*0.7055.22 ) + 2 ) = 0.61

Vc= (13 )   fc' * b0 * d                             where  (13) = 0.333     …. Control

Vc= 0.3 * 24      *  5220 * 705 * 10-3  = 5408.62  KN

Vc  = 5408.62*0.75 = 4056.46  KN>  Vu=3924.09   KN…….OK

***** Design for flexural in both direction:

Take steel bars of 14

L= 3m           h=800mm        d= 718mm             fc' =24 MPa                  fy=420MPa

Rn= Mubd2 =  1161.58*1060.9*3000*7182= 0.83 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 120.58(1-1-2*20.58*0.83420 )=0.002

As =  ρ * b * d   =  0.002 * (3000)  *( 718)  =  4308 mm2

Asmin  =  0.0018 * b * h  = 0.0018 * 3000 * 800  =  4320 mm2

As <  Asmin      not Ok                        Take  Asmin  =  4320mm2.

Use  3014    with As=4617mm2.

S =  3000-75*2-30*1429  =  83.79 mm.

Step(s) is the smallest of :

- 3* h = 3 * 800 = 2400  mm                (2) 450mm    …control

S = 83.79 mm <  Smax=450            ………  OK

Figure (4-29) :  Top view for Isolated footing F13

Figure (4-30): Reinforcement detail of Isolated footing F13

4 .15 Design of combined footing.F24

Table 4.8 : information about combined footing F24.

| Col. | Dead load factor | Live Load fac. | Dead service | Live load service | Dim. Of column |
| --- | --- | --- | --- | --- | --- |
| C15C | 1600 | 3000 | 1333.3 | 2500 | 50*40 |
| C20C | 950 | 1650 | 593.75 | 1031.25 | 60*60 |

∑Total service = 5458.33 KN

- Footing dimensions:

** Footing dim.

∑M col1 = 0 ,    2500+1031.25 * 2.45458.33 = 1.55m

A= Pnqa.net   =  5458.33450  = 12.13 m2.

A= BL         take    L=3.6  m   =B =  AL  = 12.13 3.6 = 3.36m

Take    B=3.5 m

Figure (4-31):geometry of combined footing F24

P1u = 1.2 * 1333.33 + 1.6 * 593.75  = 2549.99 KN/m

P2u =  1.2 * 2500 + 1.6 * 1031.25 = 4650 KN/m∑(P1u+P2u)=7199.99KN

qu =  7199.99(3.6*3.5) =  571.42 KN/m2.

Figure (4-32): moment and shear diagram for combined footing F24

***One way shear (beam shear)

Assume  h=85cm  and  steel bar of 20

davg =  850 – 75 – 20 = 755mm

Vc =  0.75  * (16) * 24 * 3500 * 755 * 10-3 = 1618.19 KN

**At column(1)    pu1 = 2549.99  KN

Vu = 2549.99 - 571.42 * 3.5  (0.0 + 0.4 + 0.755 ) = 240.02 KN

**At column (2)    pu2 = 4650 KN

Vu= 4650 - 571.42 * 3.5 (0.68 +  0.6 + 0.755 ) =  580.06 KN

Vc  = 1618.19 KN > Vumax = 580.06 KN      ….OK

Two – way (punshing shear)

**At colomn(1)  pu1=2549.99KN

d=  0.7552 = 0.377m > 0.0m

as edge b0 =  2 ( 0.0 + 0.4 + 0.7552 ) +( 0.5 + 0.755 ) = 2.81 m

Vu = 2549.99 - 571.42 ( 0.0 + 0.4 + 0.7552 ) + ( 0.5 + 0.755 ) = 2106.96 KN

β=500400 = 1.25,     s =30   -- edge column .

(16) (1+21.25) = 0.43

(112) ((30* 0.7552.81) +2) = 0.83

13=  0.333---- control

Vc = 0.75 (13) * 24* 2810* 755 * 10-3  = 2598.35  KN

Vc = 2598.35 KN > Vu = 2106.96 KN     …..OK

adequate enough

**At column (2)   pu2=4650KN

d2 =  0.7552 = 0.3775m  <  0.68 m

two options punching action .

*as edge perimeter :

bo = 2 ( 0.68 + 0.6 + 0.7552 ) + ( 0.6 + 0.755 ) = 4.67    …. Control

*as interior perimeter

b0= 2 ( 0.6 + 0.755 ) + 2 ( 0.6 + 0.755 ) = 5.42 m

Vu = 4650 - 571.42 * ( 0.68 +0.6 + 0.7552 ) * ( 06 + 0.755 ) = 3366.64 KN

β = 600600 = 1.0         s =30 --- edge column .

16 (1+21) = 0.5

112 (( 30*0.7554.67 ) + 2 ) = 0.57

13 = 0.33  --- control

Vc = 0.75 * (13) 24 * 4670 * 755 * 10-3 = 4318.26 KN

Vc =  4318.26KN  >  Vumax= 3366.64 KN

OK…. The thickness  is  adequate enough .

**Flexural long –direction    Mu=-993.28KN.m

Take steel 20        d = 850 - 20 - 75- 202=745KN

Rn= Mubd2 =  993.28*1060.9*3500*7452= 0.56 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 120.59(1-1-2*20.59*0.56420 )=0.0013

As  =  ρ*b*d  = 0.0013*(3500) *(745) = 3389.75  mm2

A smin=  0.0018 * b * h = 0.001 8* 3500 * 850 = 5355  mm2

As  <  A smin       not Ok                        Take Asmin=5355 mm2.

Take 2716   with As= 5429.7mm2.

Mu=902.55KN.m  +Mu=62.49KN.m

Rn= Mubd2 =  902.55*1060.9*3500*7452= 0.52 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 120.59(1-1-2*20.59*0.52420 )=0.0013

As =  ρ * b * d  = 0.0013 * ( 3500)  * (745) = 3389.75  mm2

Asmin = 0.0018 * b * h = 0.0018 * 3500 * 850=5355 mm2

As <  Asmin      not Ok                        Take A smin = 5355mm2.

Take 2716   with As= 5429.7mm2.

**Design the flexural reinforcement in the transverse direction (transverse beams)

For column (1) under

d= 850 – 75 - 202 = 765mm

Pu1 =  2549.993.5 = 728.56

Mu= 728.562  (  3.52 -  0.52  ) 2 = 819.63 KN.m

(c+d) =( 0.40 + 0.765 2) = 0.78 m            take 0.9 m

Rn = Mubd2  =  819.63 *1060.9*900*7652 =  1.72 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 120.59(1-1-2*20.59*1.72420 ) = 0.0043

As =  ρ* b * d   =  0.0043* (900)  * (765)  =  2960.55 mm2

A smin  = 0.0018 * b * h = 0.0018 * 900 * 850 = 1377 mm2

A s  >  Asmin       Ok                        Take As = 3167.1 mm2.

Take 1120   with As= 3456.2mm2.>Asreq=2960.55mm2.

S= 900-75-11*2010   =  60.5mm < 450 mm    OK

For column (2) under

Pu1= 46503.5 = 1328.57

Mu= 1328.572 ( 3.52 - 0.62 )2 = 1396.65 KN.m

(c+d) = (0.60+0.765) =1.365             take 1.4 m

Rn= Mubd2 =  1396.65*1060.9*1400*7652= 1.89  MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 120.59(1-1-2*20.59*1.89420 ) = 0.0047

As  =  ρ*b*d  = 0.0047 * (1400)  * (765)  =  5033.7  mm2

Asmin =  0.0018 * b * h = 0.0018 * 1400 * 850 = 2142  mm2

As >  A smin       Ok                        Take  As =5033.7  mm2.

Take 1820   with As= 5653.mm2.>Asreq=5033.7mm2.

S= 1400 – 17 *2016 = 66.25 mm <  450  mm    OK

Temperature :

Asmin  =   0.0018 * 1000 * 850 = 1530 mm2.

Take 816     or   16@12.5cm <  45cm     ……OK

Figure (4-33): top veiw of combiend footing F24

Figure (4-34): reinforcement detail of combined footing F24

4 –16 Design of strip footing.

Determination of load  (from slab and weigh wall )

Dead factor      400     KN/m          live factor   120KN/m

Dead service  333.33  KN/m          live service  75 KN/m

Soil density = 18Kg/cm3.

Allowable soil pressure =  450  KN/m2

** Estimate the size of the footing and the factored net pressure .

Assume footing to be about (30cm) thick.
qa.net =  450 – 03 * 25 – 0.4 * 18 - 7= 428.3 KN/m2.

A=b*1      b=0.95        Take b=1m

**Depth footing and shear design

Pu =  1.2 * 333.33 + 1.6 * 75 =  620KN/m

qu =  6201*1 =  620  KN/m2

**  one way shear (beam shear )

Vu at distance d from the face of column :

Vu =  qu * 1 ( b2- a2 – d ) = 620 * 1 *  ( 12 – 0.252 – d )

Let Vu = Vc , ( =0.75 )

Vc =  (16) fc' *  bw * d = 0.75 * (16)*24  * 1000*d

620 * 1 * (0.5 - 0.25 2 – d ) = 0.75 * (16) * 24* 1000 * d

310  - 77.5 -  620 d = 612.37 d             d=0.1886

Assume  cover  75 mm ,and  steel  bars  of  20

h =  188 + 75 + 202 = 273 mm     the thickness is adequate enough.

Take h=30cm.             d= 300 – 75 - 202 = 215mm

**Design for  flexure :

Take steel bar   of 20                Mu  = 620 * (1*0.375) * (0.3752) = 43.59 KN.m

Rn= Mubd2 =  43.59*1060.9*1000*2152= 1.04 MPa.

ρ  = 1m(1-1-2Rn*mfy )  = 120.59(1-1-2*1.04*20.59420 )=0.0025

As  =  ρ b d = 0.0025 * 1000 * 215 = 537.5 mm2.

Asmin = 0.0018 * b * h = 0.0018 * 1000 *300 =  540 mm2

As = 537.5 mm2 <  Asmin = 540mm2     ---notOk

Use A smin       n =AsminAs12  = 540113.1  =  4.77

S= 14.77 = 0.209 m     take 12@200mm

Or   512/m       with  As =  565.5mm2  > A s.req= 540mm2.

Step (s) is the smallest of :

- 3h= 3* 300 = 900mm.       (2)  450 mm  ---- control .

S =  200mm  <   Smax = 450     —Ok

Select the minimum (temperature ) reinforcement :by code ACI

We require the  following reinforcement along length of the footing :

A smin = 0.0018 * b * h = 0.0018 * 1000 * 300 = 540 mm2.

The maximum spacing is 5h  or  450mm . Provide   512(565.5mm2)

For  shrinkage reinforcement.

** Development length of main reinforcement ;

The basic compression-development length is (ACI code)

ldcreq=0.24*Fyλfc'*db ≥ 0.043 fy * db  ≥ 200 mm

ldcreq=0.24*Fyλfc'*db=0.24*4201*24*14 =288.05 mm

ldcreq=0.043 fy * db =  0.043 * 420 * 14 = 252.84 mm

Ld available   = 300 – 75 - 2*12 =  201mm

Ld available   = 201  mm  <  ldcreq=288.05mm

Use Using hook   16*

Required length of hook    16*   16 * 1.4  =  22.4 cm

Use Hooksel. = 25 cm > Hookreq = 22.4cm

Ld =  201 + 250 = 451mm >  ldcreq=288.05      ….ok

Figure (4-35): geometry of strip footing

Figure (4-36):  reinforcement detail for strip footing

4.17 Design of  Shear Wall: Shear(3)

Figure (4-37): location of  shear wall  (3)

Figure (4-38):shear and moment for shear wall (3)

Fc  =  24 MPa

=  420 MPa.

h= 25 cm.  Shear wall thickness.

Lw=  7.7 m.  shear wall width

Hw=  21.0 m.  Building  height

Lw2=   7.72 = 3.85 m

hw2=  21 2  = 11.5 m

Story height =  3.5…………..(control)

d= 0.8 * Lw = 0.8 * 7.7 = 6.16 m

Vc2= fc*b*d4+Nu*d4*lw

Vc2= 24*0.25*6.164+0.0=1886.1kN

Vc3= fc2+lwfc+2*NuLw*bMuVu-lw2*b*d10

Mu = 24281.95 kN.m

Vc3= 242+7.724+024281.951579.7-7.752*0.25*6.1610=1101.6kN………Control

Horizontal shear reinforcement:

Vs=Vu∅- Vc

Vs=1579.10.75- 1101.6=1004.7 kN

Avhs2= VsFy.d= 1004.7420*6.16*103=0.000388

ρt=Avhs2.hmin=0.0003880.25=0.00155

Take ρt=0.0025

S ≤(lw5) = 77005= 1540 mm

S2 ≤ 3 h = 3 * 250 = 750 mm

450……… Control

Try ∅10

2*78.5*10-60.25*s.=0.0025

S = 251mm

Use ∅10  @ 25 cm c/c For Horizontal R.F.

Vertical flexture reinforcement

Use ∅16@150

Ast  =  7700150* 2 * 201.1  = 20646mm2

ω = AstLw.h. fyfc' = 206467700*250. 42024 = 0.1877

cLw =  ω+a2ω+0.85B = 0.1877+02*0.1877+0.85*0.85 = 0.171

∅Mn=∅*0.5Ast.fy.Lw. 1+PuAst.fy1-cLw

=0.75* 0.5 *20646 * 420*7700  1+01-0.171   =   24920 ………..ok

Figure (4-39): Reinforcement detail for shear (3)

4.18 Design of Basement Wall:-

4.18.1  Load Calculation:-

= 30

Soil density =  18 Kg/cm3

ko= 1-sin = 1- sin 30  =  0.5

q1= 0.5*5 = 2.5 KN

q2= 18*4*0.5 = 36 KN/m

qu1 (factored) =1.6 * 2.5 = 4   KN/m

qu2 (factored) = 1.6 * 36 = 57.6   KN/m

Figure (4-40) :  Loads  on Basement Wall

Figure (4-41):  Loads & Shear/Moment envelope for basement wall

4.18.2 Design of Wall:-

Assume h = 250 mm

d =250 – 20 – 142= 223 mm

Vud= 63.6 KN

∅Vc= ∅6fc*d*b

∅Vc= 0.75624*0.223*1=136.56 kN

∅Vc>Vu … No Shear Reinforcement is Required.

** For  Horizontal Reinforcement, Use Asmin

As min  =  0.002  *  b  *  h

Asmin =  0.002 * 1000 * 250 = 500mm2 / m = 50 cm2 / m

Use 12

500 113.1 = 4.42…..  use 5  bars

Use  12 at 20 cm c/c

4.18.2.1 Design for Bending Moment:-

Mumax = 66.8 KN/m

Kn =

Use  16

No. = 802.8/201.1= 3.99  , Use 4 bars

Use  16 at 25  cm

1/2 * 802.8 = 401.4 mm2

Use  12

No. = 401.4/113.1 = 3.54    , Use 4 Bars

Use 12 at 25 cm c/c

4.18.3 Design of Basement Footing:-

Total factored load = 75 KN

Soil density = 1 8  Kg/cm3.

Allowable soil Pressure = 450 KN/m2.

Assume footing to be about (30 cm) thick.

Footing weight = (250.3) = 7.5 KN/m2.

Soil weight above the footing =  (3.7) 18 = 66.6 KN/m2.

live load = 5  KN/m2

q a,net = 450 – 5  - 7.5- 66.6 = 375.9 KN/m2

Assume b = 1 m , h= 30 cm

d= 300 – 75 – 14 = 211 mm,    qult = 375.9/1 *1= 375.9 KN/m2

Figure (4-42): Loads & Shear/Moment envelope for basement wall

4.18.3.1 Check of One Way Shear:-

Vu = 1* (0.5 - 0.125 - 0.211 ) *  375.9  =  61.64  KN

∅Vc =  ∅6fc * d * b

∅Vc= 0.75624 * 211 * 1000 * 10-3=129.2 KN

∅Vc> Vu…..No shear R.F is required

4.18.3.2  Design of  Bending Moment:-

Mu  = 375.9 * 1 * 0.375 *  0.3752 =  26.43 KN/m

Kn =

Use  A smin

Use  14

No. =  540154  = 3.51      , Use 4 bars

Use Using hook  16*

Required length of hook  16 *   16 * 1.4  =  22.4 cm

Use Hooksel. =  25  cm  >  Hookreq = 22.4 cm

Use 14 at 25 cm c/c

In  the  other  Direction:-

Asmin =  0.0018 * b * h

Asmin =  0.0018  * 1 000 * 300 = 540  mm2

Use  14

No. = 540154= 3.51      , Use 4 bars

Use  14@ 25 cm c/c

The basic compression-development length is (ACI code)

ldcreq= 0.24 *Fyλfc'*db ≥  0.043 fy* db  ≥200mm

ldcreq= 0.24*Fyλfc'*db= 0.24*4201*24*14 = 288.05mm

ldcreq=  0.043fy*db= 0.043*420*14 = 252.84mm

Ld available =  300-75-2*14 = 197 mm

Ld available=  197  mm  <  ldcreq=288.05mm

Use Using hook  16*

Required length of hook16*16 * 1.4 =  22.4 cm

Use Hooksel. = 25 cm > Hookreq = 22.4 cm

Ld  = 197+250 = 447mm  >  ldcreq= 288.05      ….ok

Figure (4-43):reinforcement detail for basement wall

4 –19 Design of Mat Foundation(Mat1).

We got the moment in x direction and y direction after using the sap12 program as the fooling …

The load on mat  is from the shear  wall 12 and columns  C26-C

And   C30-C .

Assume h = 70 m .

And  qa = 4.5 Kg/cm3   for the soil.

Figure (4-44):(M11) moment in x axis for mat foundation

Figure (4-45):(M22)moment in y axis for mat foundation

Figure (4-46):    (M11+M22)=1100KN.m   maximum for positive.

Figure (4-47):     M11= - 600KN.m for maximum negative and

M22 = 0 for negative

Design for flexural  short direction (x direction)(M11)

Moment positive Mu= 1100KN.m

d=  700 – 75 - 20 - 202  = 595mm

Kn =

Use A s min

Use  16

No. =  5418201.1 = 26.9      ,  Use 27 bars

Use Using hook  16 *

Required length of hook  16 *    16 * 1.6  = 25.6 cm

Use Hooksel. = 35 cm > Hookreq = 25.6cm

Use    27 16

Design for flexural  short direction (x direction)(M11)

Moment Negative Mu= -600KN.m

Use Asmin

Use  16

No. = 5418201.1 = 26.9      , Use 27 bars

Use Using hook 16*

Required length of hook 16* 16 * 1.6  =  25.6 cm

Use Hooksel. = 35 cm >  Hookreq = 25.6 cm

Use    27 16

Design for flexural  long direction (y direction)(M22)

Moment Positive  Mu= 1100KN.m

d = 700 - 75 - 202 = 615mm

Kn =

Use A s min

Use  16

No. = 9387201.1 = 46.6      , Use 27 bars

Use Using hook 16 *

Required length of hook  16* 16*1.6  = 25.6 cm

Use Hooksel. =  35 cm >  Hookreq  = 25.6cm

Use    47 16

Design for flexural  long direction (y direction)(M22)

Moment Negative = 0 KN.m

Use A s min

Use  16

No. =  9387201.1 = 46.6      , Use 27 bars

Use Using hook   16 *

Required length of hook  16 *    16 * 1.6  =  25.6 cm

Use Hooksel.  = 35 cm >  Hookreq = 25.6cm

Use    47 16

Figure (4-48): top view for mat foundation mat1

Figure (4-49): reinforcement detail for mat foundation mat (1)