Appendix B

Base station source code

//Base Station Configration

#include<p18f4550.h>

#include <delays.h>

#include <stdlib.h>

#include <usart.h>

#pragma config FOSC = INTOSC_HS

#pragma config WDT = OFF

#pragma config DEBUG = ON

#pragma config LVP = OFF

// HT12D Vs PIC Mapping

#define   R_Data0       PORTBbits.RB0

#define   R_Data1       PORTBbits.RB1

#define   R_Data2       PORTBbits.RB2

#define   R_Data3       PORTBbits.RB3

#define   R_AD0         PORTBbits.RB4

#define   R_AD1         PORTBbits.RB5

#define   R_AD2         PORTBbits.RB6

#define   R_AD3         PORTBbits.RB7

#define   V_tst         PORTCbits.RC5

//HT12E Vs PIC Mapping

#define   T_Data0       PORTDbits.RD0

#define   T_Data1       PORTDbits.RD1

#define   T_Data2       PORTDbits.RD2

#define   T_Data3       PORTDbits.RD3

#define   T_AD0         PORTDbits.RD4

#define   T_AD1         PORTDbits.RD5

#define   T_AD2         PORTDbits.RD6

#define   T_AD3         PORTDbits.RD7

#define   TE            PORTCbits.RC4

void run(int x);

void sleep(int y);

char byte,byte2;

char UVal;

char RUVal;

char WUVal;

void main(void)

{

// Ports initialization

PORTA=0x00;

PORTB=0x00;

PORTC=0x00;

PORTD=0x00;

PORTE=0x00;

LATA=0x00;

LATB=0x0F;

LATC=0x00;

LATD=0x00;

LATE=0x00;

TRISA=(0b00000000);

TRISB=(0b00001111);

TRISC=(0b00000000);

TRISD=(0b00000000);

TRISE=(0b00000000);

TE=1;

// Set Nodes to Sleep Mode

// set address of N1

T_AD0 = 1;

T_AD1 = 0;

T_AD2 = 0;

T_AD3 = 0;

// write sleep command word to N1

T_Data0 =0;

T_Data1 =0;

T_Data2 =0;

T_Data3 =1;

//  Transmmit command to N1 by setting TE = 0

TE = 0;

// wait for complete transsmission 20 miro second

Delay10TCYx(100);

// reset TE

TE = 1;

// set address of N2

T_AD0 = 0;

T_AD1 = 1;

T_AD2 = 0;

T_AD3 = 0;

// write sleep command word to N2

T_Data0 =0;

T_Data1 =0;

T_Data2 =0;

T_Data3 =1;

//  Transmmit command to N1 by setting TE = 0

TE = 0;

//  wait for complete transsmission 20 miro second

Delay10TCYx(100);

// reset TE

TE = 1;

// usart configration

OpenUSART(USART_TX_INT_OFF & USART_RX_INT_OFF

& USART_ASYNCH_MODE & USART_EIGHT_BIT

& USART_CONT_RX & USART_BRGH_HIGH,25);

while(1)

{

while(BusyUSART());    //if usart busy do nothing

UVal =ReadUSART();     //read value from usart

if (UVal == 0x20)  {sleep(1);}   // call N1 sleep routin   set x=1 ,call sleep(x)

if (UVal == 0xA0)  {run(1);}    //  call N1 run   routin   //  x=1  ,call run(x)

if (UVal == 0x40)  {sleep(2);}  //  call N2 sleep routin  //   x=2  ,call sleep(x)

if (UVal == 0xC0)  {run(2);}  //    call N2 run   routin  //   x=2  ,call run(x)

}//End while(1)

}//end main

void run(int x)

{

int i;

// set node number to -->x[1,2]

if(x==1)

{

// set address on tr to node 1

T_AD0 = 1;

T_AD1 = 0;

T_AD2 = 0;

T_AD3 = 0;

//write run command on TR data port

T_Data0 =0;

T_Data1 =0;

T_Data2 =0;

T_Data3 =0;

// transmitt

TE=0;

// wait for complet transmission 20 micro seond

Delay10TCYx(100);

TE=1;

// get Data From Node 1-------------------------------------

R_AD0 = 1;

R_AD1 = 0;

//-----wait for reciption-----

Delay10TCYx(100);

//-----get TempLow,TempHigh,HumLow,HumHigh,Voltage

for(i=0;i<4;i++)

{

byte = PORTB;

byte = byte & 0x0F;

while(BusyUSART());

WriteUSART(byte); //write data To PC via Usart

}//end for

} // end if 1

if(x==2)

{

// set address on tr to node 2

T_AD0 = 0;

T_AD1 = 1;

T_AD2 = 0;

T_AD3 = 0;

//write run command on TR data port

T_Data0 =1;

T_Data1 =0;

T_Data2 =0;

T_Data3 =1;

// transmitt

TE=0;

// wait for complet transmission 20 micro seond

Delay10TCYx(100);

TE=1;

// get Data From Node 2-------------------------------------

R_AD0 = 0;

R_AD1 = 1;

//-----wait for reciption-----

Delay10TCYx(100);

//-----get TempLow,TempHigh,HumLow,HumHigh,Voltage

for(i=0;i<4;i++)

{

byte = PORTB;

byte = byte & 0x0F;

while(BusyUSART());

WriteUSART(byte); //write data To PC via Usart

}//end for

} // end if 2

}//end run

void sleep(int y)

{

if(y == 1)

{

// set nod 1 address

T_AD0 = 1;

T_AD1 = 0;

T_AD2 = 0;

T_AD3 = 0;

// write sleep command to data port

T_Data0 =1;

T_Data1 =0;

T_Data2 =0;

T_Data3 =0;

TE = 0;

// wait for 20 micro second

Delay10TCYx(100);

TE=1;

}// end if n 1

if(y == 2)

{

// set nod 2 address

T_AD0 = 0;

T_AD1 = 1;

T_AD2 = 0;

T_AD3 = 0;

// write sleep command to data port

T_Data0 =1;

T_Data1 =0;

T_Data2 =0;

T_Data3 =0;

TE = 0;

// wait for 20 micro second

Delay10TCYx(100);

TE=1;

}// end if n 2

}// end sleep

Remote station source code

// Rs

#include<p18f4550.h>

#include<delays.h>

#include<adc.h>

#include <capture.h>

#include <timers.h>

#pragma config FOSC = INTOSC_HS

#pragma config WDT = OFF

#pragma config DEBUG = ON

#pragma config LVP = OFF

// HT12D Vs PIC Mapping

#define   R_Data0       PORTBbits.RB0

#define   R_Data1       PORTBbits.RB1

#define   R_Data2       PORTBbits.RB2

#define   R_Data3       PORTBbits.RB3

//A0-A7 -->xx00000000;

#define   R_A0          PORTBbits.RB4

#define   R_A1          PORTBbits.RB5

#define   R_A2          PORTBbits.RB6

#define   R_A3          PORTBbits.RB7

#define   V_tst         PORTCbits.RC5

//HT12E Vs PIC Mapping

#define   T_Data0       PORTDbits.RD0

#define   T_Data1       PORTDbits.RD1

#define   T_Data2       PORTDbits.RD2

#define   T_Data3       PORTDbits.RD3

//A0-A7 -->xx00000000;

#define   T_A0           PORTDbits.RD4

#define   T_A1           PORTDbits.RD5

#define   T_A2           PORTDbits.RD6

#define   T_A3           PORTDbits.RD7

#define   TE             PORTCbits.RC4

//-----Node Functions get Data from sround Teprature , hummedity and Voltage value

char get_temp();

char get_hum();

char get_Vol();

int flag = 0;

void run();

void sleep();

void main()

{

char RData;

char byte;

char T,H,V;

char temp_val;

char hum_val;

char Vol_val;

ADCON1=15;

// CONF PORTS:

PORTA=0x00;

PORTB=0x00;

PORTC=0x00;

PORTD=0x00;

PORTE=0x00;

LATA=0x00;

LATB=0x00;

LATC=0x00;

LATD=0x00;

LATE=0x00;

TRISA=(0b00001111);

TRISB=(0b00001111);

TRISC=(0b00000000);

TRISD=(0b00000000);

TRISE=(0b00000000);

temp_val = get_temp();

hum_val  = get_hum();

Vol_val  = get_Vol();

while (1)

{

// Set Recever address to Bs  Transmmiter address

R_A0 = 1;

R_A1 = 0;

R_A2 = 0;//x

R_A3 = 0;//x

Delay2KTCx(20); // wait for node 1 Time slot start

// get Data from N1 recever

byte = PORTB;

byte = byte & 0x0F; // just take low nibble of PORTB

if(byte = 0x02)

{

if(flag){

// on TR --> set 0 on RC5

PORTCbits.RC5 = 1;

// on S1 and S2 --> set 0 0n RC6

PORTCbits.RC6 = 1;

}//end if flag

Delay2KTCx(20);//wait the circuit to be stable

run(); // applay run command

} //end if run

else if(byte = 0x0A)

(

if (!flag)

{

sleep();// applay sleep

} // if node in sleep mode no need to sleep it again

)// end of sleep

}// End while

}//end main

void run()

{// get Data From Sensors

char T,H,V;

char T1,H1,V1;

T=get_temp();

H=get_hum();

V=get_vol();

// set TR addess to Bs addres

// set active -->>> on

T_A0 = 0;

T_A1 = 0;

// set Data line s to 0's

T_Data0 = 0;

T_Data1 = 0;

T_Data2 = 0;

T_Data3 = 0;

TE   = 1; // Off Tanssmission

//TS_low=T_Data0-->T_Data3

T1 = T & 0x0F ;

T1 = PORTD;

TE = 0;

//Delay to complete TR

TE =1;

//TS_High=T_Data0 -->T_Data3

T1= T & 0xF0;

T1= PORTD;

TE=0;

//delay

TE=1;

//Hs_low -->T_data

H1 = H & 0x0F ;

H1 = PORTD;

TE=0;

//wait

TE=1;

//HS_high --> T_data

H1 = H & 0xF0;

H1 = PORTD;

TE=0;

// wait

TE=1;

//VOltage --> T_data

V1 = V & 0x0F;

V1 = PORTD;

TE = 0 ;

//wait

TE= 1;

}// end run

void sleep()

{

flag = 1;

// off TR --> set 0 on RC5

PORTCbits.RC5 = 0;

// off S1 --> set 0 0n RC6

// off S2 --> set 0 0n RC6

PORTCbits.RC6 = 0;

}// end sleep

char get_temp()

{

char temp;

OpenADC (ADC_FOSC_64  & ADC_RIGHT_JUST/*left shift*/& ADC_2_TAD,ADC_CH0 & ADC_INT_OFF & ADC_REF_VDD_VSS , ADC_1ANA);

ConvertADC();

while(BusyADC())

temp=ReadADC();//get high = 8 bit-->sget reg value ADRESH

temp= 5 * temp * 100/1024;

return temp;

}

char get_hum()

{

// Configure Capture1

OpenCapture1(C1_EVERY_RISE_EDGE & CAPTURE_INT_OFF);

OpenCapture2(C2_EVERY_FALL_EDGE & CAPTURE_INT_OFF);

// Configure Timer1

OpenTimer1(TIMER_INT_OFF & T1_SOURCE_INT & T1_SOURCE_CCP & T1_PS_1_1 & T1_16BIT_RW);

rise = ReadCapture1();

fall = ReadCapture2();

pulse=rise-fall;

if (pulse<0)

pulse=pulse*(-1);

f= 1/pulse;

H = 565.1 – 0.0767 * f

returen H;}

char get_Vol()

{

char Vol;

OpenADC (ADC_FOSC_64/*speed of exe*/  & ADC_RIGHT_JUST & ADC_2_TAD,ADC_CH2 /*channale num*/& ADC_INT_OFF & ADC_REF_VDD_VSS , ADC_1ANA/*Adcon1*/);

ConvertADC();

while(BusyADC());

Vol=ReadADC();

return Vol;

}