4

Chapter 4

Structural Analysis And Design

4.1 Introduction.

4.2 Design method and requirements.

4.3 Check of Minimum Thickness of Structural Member.

4.4 Design of topping.

4.5 Design of One Way-ribbed Slab (RG14).

4.6Design of Beam(BG28).

4.7 Design of Two Way Ribbed Slab.

4.8 Design of One Way Solid Slab.

4.9 Design of Two way Solid Slab .

4.10  Design of Stair.

4.11 Design of Column .

4.12 Design of shear wall.

4.13 Design of footing.

4.14 Design Steel Truss.

4.1 Introduction:

Many structures are built of reinforced concrete: bridges, buildings, retaining walls, tunnels, and others.

Reinforced concrete is logical union of two materials: plain concrete, which possesses high compressive strength but little tensile strength, and steel bars embedded in the concrete, which can provide the needed strength in tension.

Plain concrete is made by mixing cement, fine aggregate, coarse aggregate, water, and frequently admixtures.

Understanding of reinforced concrete behavior is still far from complete, building codes and specifications that give design procedures are continually changing to reflect latest knowledge.

Structural concrete can be classified into:

- Lightweight concrete with unit weight from about 1350 to 1850 kg/m3.

- Normal weight concrete with unit weight from about 1800 to 2400 kg/m3.

- Heavyweight concrete with unit weight from about 3200 to 5600 kg/m3.

4.2 Design method and requirements:

The design strength provided by a member is calculated in accordance with the requirements and assumptions of ACI _ code (318_08).

- Strength design method:

In ultimate strength design method, the service loads are increased by factors to obtain the load at which failure is considered to be occurring.

This load called factored load or factored service load. The structure or structural element is then proportioned such that the strength is reached when factored load is acting. The computation of this strength takes into account the nonlinear stress-strain behavior of concrete.

The strength design method is expressed by the following,

Strength provided ≥ strength required to carry factored loads.

NOTE:

The statically calculation and the key plans dependent on the architectural plans.

- Code :    ACI 2008

UBC

- Material :

Concrete:    B300…. For circular section

but for rectangular section () .

Reinforcement steel :   The specified yield strength of the reinforcement {fy = 420 N/mm² (MPa)}

- Factored loads:

The factored loads for members in our project are determined by:

Wu = 1.2 DL + 1.6 LL                   ACI-code-318-08(9.2.1).

4.3 Check of Minimum Thickness of Structural Member:

TABLE (4.1) — MINIMUM THICKNESS OF NONPRESTRESSED BEAMSOR ONE-WAY SLABS UNLESS DEFLECTIONS ARE CALCULATED.  (ACI 318M-11)

| Minimum thickness ( h) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Member | Simply supported | One end continuous | Both end continuous | Cantilever |
| solid one way slabs | L/20 | L/24 | L/28 | L/10 |
| Beams or ribbed one way slabs | L/16 | L/18.5 | L/21 | L/8 |

FOR RIB :

hminfor(one end)=L/18.5=6.4/18.5=35cm

hminfor(both end continuous)=L/21=5.6/21=27cm

FOR BEAM :

hminfor(both end continuous)=L/21=6.5/21=31cm

hminfor(Cantilever)=L/8=1.5/8=27cm

hminfor(one end)=L/18.5=5.8/18.5=32cm

FOR SOLID SLABS :

hminfor(one end continuous)=L/24=8/24=34cm

hminfor(both end continuous)=L/28=7.2/28=26cm

The minimum thickness will be hmin=35cm

select 35cm for rib slab with hidden beam

h=35cm(27 cm Hollow Block+8 cm Topping)

select (35cm) for solid slab.

4.4 Design of topping:

- Statically system for topping :

Consider the topping as strip of (1m) width, and span of mold length with both end fixed in the ribs.

Fig 4.1: topping load.

- Load calculations:

Dead load calculations:

| Dead load from: | δ×γ×1 | KN/m |
| --- | --- | --- |
| Tiles | 0.03×23×1 | 0.69 |
| Mortar | 0.03*22 | 0.66 |
| Coarse sand | 0.07×17×1 | 1.19 |
| Topping | 0.08×25×1 | 2 |
| Interior partitions | 2.3 | 2.3 |
|  | ∑ | 6.84 |

Table ( 4.2 ) : Dead load calculation Topping

Live load :

LL =4 KN/m2

LL =4 KN/m2×1m= 4 KN/m

Factored load :

WU = 1.2 ×6.84+ 1.6×4 =14.61 KN/m.

Mu = WuL212

Mu=14.61*0.4212=0.195 KN.mmof strip width

Check the strength condition for plain concrete, ø Mn  ≥ Mu, where ø = 0.55.

Mn = 0.42 λ fc' Sm                        (ACI 22.5.1, equation 22-2)

Sm=b.h26=1000.8026=1066666.67 mm2.

øMn =0.55×1×24 ×1066666.67 ×10-6 =1.21 KN.m

øMn>> Mu =  0.195 KN.m

No reinforcement is required by analysis. According ACI 10.5.4, provide As,min for slabs as shrinkage and temperature reinforcement.

ρshrinkage = 0.0018                                               ACI 7.12.2.1

As = ρ×b×htopping  =0.0018 ×1000×80 = 144 mm2/m.

Step (s) is the smallest of:

- 3h = 3×80 =240 mm.         control ACI 10.5.4

- 450mm.

- S =380280fs-2.5Cc=380 28023420-2.5 .20=330mm

but

S≤ 300280fs=30028023420=300mmACI 10.6.4

Take ø 8 @ 200 mm in both direction , S = 200 mm < Smax = 240 mm … OK

4.5 Design of One-Way Ribbed Slab(RG14) :

Requirements For Ribbed Slab Floor According to      ACI- (318-08) .

bw ≥ 10cm……………………………………………ACI(8.13.2)

Select bw=12cm

h ≤ 3.5*bw  ……………………………….…….. ACI(8.13.2)

Select h=32cm<3.5*12=42 cm

tf ≥ Ln/12≥50mm ………………………………….ACI(8.13.6.1)

Select tf=8cm

- Statically system and Dimensions

Fig 4.2: One Way Rib slab (RG14)

Load  calculations :

*Dead load:

| Material | W= h×γ×b(KN/M) |
| --- | --- |
| Tiles | 0.03*23*0.52= 0.359 |
| Mortar | 0.03*22*0.52= 0.343 |
| Course sand | 0.07*17*0.52= 0.619 |
| Topping | 0.08*25*0.52= 1.04 |
| R.c rib | 0.27*25*0.12=0.81 |
| Hollow block | 0.27*10*0.4= 1.08 |
| Plaster | 0.03*22*0.52= 0.343 |
| Interior partition | 2.3*0.52=1.196 |
| Total dead load | 5.79 KN/m |

Table (4.3): Dead load calculation Topping of rib 14

Dead  load /rib = 5.80KN/m

Live load =4KN/M2

Live load /rib = 4KN/m2 × 0.52m = 2.08 KN/m.

*The effective flange (be) :

1) be ≤L4=35004=875mm

2) be≤ bw +16hf=120+16*80=1400mm

3) be≤ center to center spacing between adjacent beam = 4002+4002+120=520mm

Take be=520 mm

Fig 4.3: Shear & Moment Envelope Diagram (RG14)

*Design of positive moment:

Mu = 34.4KN.m.

Assume bar diameter ø 12 for main positive reinforcement.

d =h- cover - dstirrups -db2=350-20-8-122=316 mm.

Check if  a > hf  to determine whether the section will act as rectangular or T- section,

Mnf =0.85. fc'.be.hf.(d-hf2)

= 0.85×24×520×80×316 -802×10-6=234.22KN.m

Mnf ≫Muφ=34.40.9=   38.22 KN.m  ,  the section will be designed as rectangular section with  be =520 mm.

Rn=Mu∅bd2=34.4×1060.9×520×3162=0.736Mpa.

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.736420=0.00178

As,req = ρ.b.d = 0.00178×520×316= 293.3mm2

*Check for As,min .

As,min is the maximum of :-

As,min =0.25f'cfybw.d≥1.4fybw.d

- As,min =0.2524420 120×316=110.6mm2

- As,min =1.4420120×316=126.4mm2  Control

As = 293.3mm2 > As,min = 126.4mm2

Use 2ø14 , As,provided = 307.9 mm2>  As,required = 293.3  mm2.         Ok

Check for strain:

a =As.fy0.85b fc'=307.9×4200.85×520×24=12.19mm

c=aB1=9.660.85=14.34 mm

εs=0.003d-cc=0.003316-14.3414.34=0.063>0.005                0k

*Design of negative moment:

Mu = -25.1KN.m.

Assume bar diameter ø 12 for main positive reinforcement.

d =h- cover - dstirrups -db2=350-20-8-122=316mm.

Rn=Mu∅bd2=25.1×1060.9×120×3162=2.32Mpa.

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×2.32420=0.00588

As,req = ρ.b.d = 0.00588 ×120×316 = 223 mm2

*Check for As,min .

As,min is the maximum of :-

As,min =0.25f'cfybw.d≥1.4fybw.d

- As,min =0.2524420 120×316=110.6 mm2

- As,min =1.4420120×316=126.4 mm2 Control

As,required = 223  mm2.

Use 2 ø14 , As,provided = 307.9 mm2>  As,required = 223mm2.         Ok

Check for strain:

a =As.fy0.85b fc'=307.9×4200.85×120×24=52.8 mm

c=aB1=52.80.85=66 mm

εs=0.003d-cc=0.003316-6666=0.011>0.005                0k

- Shear Design for (RG14):

Vu at distance d from  support= 32 KN

Shear strength Vc, provided by concrete for the joists may be taken 10% greater that for beams. This is mainly due to the interaction between the slab and closely spaced ribs. (ACI, 8.13.8).

Vc =1.16 λ fc'bwd=1.1624 ×120×316×10-3=34.05KN

ø Vc =0.75×34.05 =25.55 KN.

0.5 ø Vc =0.5×25.55 =12.78 KN

0.5 ø Vc< Vu< ø Vc    …………….  NO

So

vs= vu∅-vc=320.75-34.05=8.61 KN

vs,min= 116fc'bw d=11624*120*316=11.65

vs,min= 13bw d = = 13 120*316=12.64 KN…….control

vs,min= 12.64 KN >vs=8.61 KN

shear reinforcement are required.

Take vs=vs,min=12.64  KN

Use 2 leg Φ 8 .

Av =100.5 mm2

s=Av*d*fyvs

s=100.5*316*42012.64*103=1055.25 mm

smax ≤ d2=3162= 158 mm                  or                 smax ≤600 mm

Use 2 leg Φ 8  @ 150 mm .

4.6 Design of Beam(BG28) :

- Load calculations:

Load calculations for BG28:

Table Dead Load Calculations for Beam(BG28)(4.4):-

| Type | b γ h | KN/m |
| --- | --- | --- |
| Tiles | 0.03*1*23 | 0.69 |
| Mortar | 0.03*1*22 | 0.66 |
| Sand | 0.07*1*17 | 1.19 |
| Reinforced concrete | 0.35*1*25 | 8.75 |
| Plaster | 0.02*1*22 | 0.44 |
| Sum |  | 11.7 |

The distributed Dead and Live loads acting upon BG28 can be defined from the support reactions of the RG14.

- From RG14

The maximum support reaction (factored) from Dead Loads for RG14 upon BG28 is  30.43 KN . The distributed Dead Load from the RG14 on BG28:

DL =31.43/ 0.52=  60.45 KN/m

Live Load calculations: The maximum support reaction (factored) from Live Loads for RG14upon BG28 is 19.2 KN .

The distributed Live Load from the RG14 on BG28:

LL =13.26/ 0.52=  25.5 KN/m

.

Fig 4.4: BG28

‎

Fig 4.5 Loading and Moment /Shear Envelope.

- Flexural Design for (BG28) :

Determine of  Mn,max :

d=350-40-8-182=293mm

c=37d=37*293=125.57 mm

a =B.c=125.57*0.85=106.73mm

Mnmax =  0.85fc'ab( d -   a2  ) = 0.85*24*106.73*1000*(293-106.73/2 ) *10-6=  521.75 KN.m

∅ Mnmax = 0.82* 521.75 = 427.84 KN.m<  -416.5

Design as singly reinforcement

Design for positive moment :

1) Mu = 353.5 KN

Rn=Mu∅bd2=353.8×1060.9×1000×2932=4.58Mpa.

m=fy0.85fc'=4200.85×24=20.58

ρ=1m1-1-2.m.Rn420=120.581-1-2×20.58×4.58420=0.01251

As = ρ.b.d = 0.01251×1000×293 = 3667.46mm2.

Check for As,min .

As,min =0.25f'cfybw.d≥1.4fybw.d

As,min =0.2524420 1000×293=854.4 mm2

As,min =1.44201000×293=976.6 mm2 Control.

As,min=976.6 mm2< As =3667.46 mm2

Use15ø 18Bottom, As,provided = 3817.5 mm2>  As,required = 3667.46mm2.         Ok

Check spacing :

S =1000-40*2-10*2-(15×18)14=45mm>25  …    OK

Check for strain:

a =As.fy0.85b fc'=3817.5×4200.85×1000×24=78.6 mm

c=aB1=78.60.85=92.51 mm

εs=0.003d-cc=0.003293-92.592.5=0.0065>0.005                0k

2) Mu = 210.1KN.m

Rn=Mu∅bd2=210.1×1060.9×1000×2932=2.72Mpa.

m=fy0.85fc'=4200.85×24=20.58

ρ=1m1-1-2.m.Rn420=120.581-1-2×20.58×2.72420=0.00697

As = ρ.b.d = 0.00697 ×1000×293= 2044.3 mm2.

Check for As,min .

As,min =0.25f'cfybw.d≥1.4fybw.d

As,min =0.2524420 1000×293=854.4 mm2

As,min =1.44201000×293=976.6 mm2 Control.

As,min =2044.3mm2<As = 976.6 mm2

Use 9ø 18 Bottom, As,provided = 2290.5  mm2>  As,required = 2044.3 mm2.         Ok

Check spacing :

S =1000-40*2-2*10-(9×18)8=92.25 mm>25  …..       OK

Check for strain:

a =As.fy0.85b fc'=2290.5 ×4200.85×1000×24=47.15 mm

c=aB1=47.150.85=55.48 mm

εs=0.003d-cc=0.003293-55.4855.48=0.0128>0.005                0k

3) Mu = 219.6KN.m

Rn=Mu∅bd2=219.6×1060.9×1000×2932=2.84Mpa.

m=fy0.85fc'=4200.85×24=20.58

ρ=1m1-1-2.m.Rn420=120.581-1-2×20.58×2.84420=0.00731

As = ρ.b.d = 0.00731 ×1000×293= 2142.44 mm2.

Check for As,min .

As,min =0.25f'cfybw.d≥1.4fybw.d

As,min =0.2524420 1000×293=854.4

As,min =1.44201000×293=976.6 mm2 Control.

As,min=976.6  mm2 >As=2142.44 mm2

Use9ø 18 Bottom, As,provided = 2290.5 mm2>  As,required = 2142.44 mm2.         Ok

Check spacing :

S =1000-40*2-2*10-(9×18)8=92.25 mm>25 …..      OK

Check for strain:

a =As.fy0.85b fc'=2290.5 ×4200.85×1000×24=47.15 mm

c=aB1=47.150.85=55.48 mm

εs=0.003d-cc=0.003293-55.4855.48=0.0128>0.005                0k

Design for Negative moment :

1) Mu = -416.5  KN.m

Rn=Mu∅bd2=416.5×1060.9×1000×2932=5.39Mpa.

m=fy0.85fc'=4200.85×24=20.58

ρ=1m1-1-2.m.Rn420=120.581-1-2×20.58×5.39420=0.0152

As = ρ.b.d = 0.0152×1000×293= 4458.2 mm2.

Check for As,min .

As,min =0.25f'cfybw.d≥1.4fybw.d

As,min =0.2524420 1000×293=854.4

As,min =1.44201000×293=976.6 mm2 Control.

As,min = 976.6 mm2< As = 4458.2 mm2

Use 18 ø 18Top , As,provided = 4581 mm2>  As,required = 4458.2mm2.         Ok

Check spacing :

S =1000-40*2-2*10-(18×18)17=33.9mm>25 ….     OK

Check for strain:

a =As.fy0.85b fc'=4581 ×4200.85×1000×24=94.31mm

c=aB1=45.750.85=53.83 mm

εs=0.003d-cc=0.003293-94.3194.31=0.0063>0.005                0k

Design for negative moment in cantilever:

2) Mu = -199.4KN.m

Rn=Mu∅bd2=199.4×1060.9×1000×2932=2.58 Mpa.

m=fy0.85fc'=4200.85×24=20.58

ρ=1m1-1-2.m.Rn420=120.581-1-2×20.58×2.58420=0.00659

As = ρ.b.d = 0.00659×1000×293= 1930.78 mm2.

Check for As,min .

As,min =0.25f'cfybw.d≥1.4fybw.d

As,min =0.2524420 1000×293=854.4

As,min =1.44201000×293=976.6 mm2 Control.

As,min = 976.6 mm2< As = 1930.78 mm2

Use 8ø 18Top , As,provided = 2036 mm2>  As,required = 1930.78mm2.         Ok

Check spacing :

S =1000-40*2-2*10-(8×18)7=108 mm>25 …    OK

Check for strain:

a =As.fy0.85b fc'=2036 ×4200.85×1000×24=41.91mm

c=aB1=41.90.85=49.31 mm

εs=0.003d-cc=0.003293-49.3149.31=0.0148>0.005                0k

Design positive moment in  cantilever:

In this area we will put minimum steel in order to catch concrete.

Use 2 Φ 18 .

- Shear Design for (BG28):

- Vu = 378.5 KN

Vc = 16fc'bw d= =  1624*1000*293*10-3=239.22KN

Φ Vc= 0.75*239.22 =179.42 KN

vs,min= 13bw d = = 13 1000* 293*10-3= 97.66 KNcontrol

vs,min= 116fc'bw d = 116*24*1000*293*10-3=89.7KN

vs'= 13fc'bw d = 1324*1000*293*10-3=478.46

∅(vc+ vs,min⁡)<vu ≤ ∅(vc+ vs')

0.75(239.22+97.66)   <  378.5 <  0.75(239.22+478.46)

252.66   <       378.5 <     538.26  …ok

shear reinforcement are required .

Use 2 leg Φ 10 .

Av =157.1 mm2 .

Vs = Vn – Vc = 378.50.75 – 239.22 = 265.45 KN

S= Avfyt dvs=157.1*420*293265.45*1000=72.8 mm             (control)

smax ≤ d2=2932=146.5 mm                                                 or smax ≤600 mm

Use 2 leg Φ 10  @75 mm .

- Vu = 339.3 KN

Vc = 16fc'bw d= =  1624*1000*293*10-3=239.22KN

Φ Vc= 0.75*239.22 =179.42 KN

vs,min= 13bw d = = 13 1000* 293*10-3= 97.66 KNcontrol

vs,min= 116fc'bw d = 116*24*1000*293*10-3=89.7KN

vs'= 13fc'bw d = 1324*1000*293*10-3=478.46

∅(vc+ vs,min⁡)<vu ≤ ∅(vc+ vs')

0.75(239.22+97.66)   <  339.3 <  0.75(239.22+478.46)

252.66   <      339.3 <     538.26  …ok

shear reinforcement are required .

Use 2 leg Φ 10 .

Av =157.1 mm2 .

Vs = Vn – Vc = 339.30.75 – 239.22 = 213.18 KN

S= Avfyt dvs=157.1*420*293213.18*1000=90.7 mm             (control)

smax ≤ d2=2932=146.5 mm                                                 or smax ≤600 mm

Use 2 leg Φ 10  @100 mm .

- Vu = 272.7 KN

Vc = 16fc'bw d= =  1624*1000*293*10-3=239.22KN

Φ Vc= 0.75*239.22 =179.42 KN

vs,min= 13bw d = = 13 1000* 293*10-3= 97.66 KNcontrol

vs,min= 116fc'bw d = 116*24*1000*293*10-3=89.7KN

vs'= 13fc'bw d = 1324*1000*293*10-3=478.46

∅(vc+ vs,min⁡)<vu ≤ ∅(vc+ vs')

0.75(239.22+97.66)   < 272.7 <  0.75(239.22+478.46)

252.66   <      272.7 <     538.26  …ok

shear reinforcement are required .

Use 2 leg Φ 10 .

Av =157.1 mm2 .

Vs = Vn – Vc = 272.70.75 – 239.22 = 124.4 KN

S= Avfyt dvs=157.1*420*293124.4*1000=155.4 mm             (control)

smax ≤ d2=2932=146.5 mm            (control)                                  or smax ≤600 mm

Use 2 leg Φ 10  @150 mm .

4.7Design Two Way Ribbed Slab

- Determination of Thickness for Two Way Ribbed Slab:

# Assume H =35cm

# Ib1=100*35312 =357291.66 cm4

# Ib2=80*35312 =285833.3 cm4

# Ib3=100*353 12=357291.66 cm4

# Ib4=80*35312 =285833.3 cm4

# Yc= 40*8*4+35*12*1640*8+35*12 =10.81 cm

# Ir=52*10.8133-40*2.8133+21*24.1933 =78219.6 cm4

# Is1=78219.6*(7602+80)52 = 722027.07 cm4

# Is2=78219.6*(7602+80)52 =  691942.6 cm4

# Is3=78219.6*(8802+100)52 = 812280.4615 cm4

# Is4=78219.6*(8802+100)52 = 782196 cm4

# α1=285833.3691942.6 = 0.41

# α2=357291.66 812280.46 = 0.43

# α3=337291.66722027.07 = 0.46

# α4=285833.3 812280.46 = 0.36

# αfm=0.41+0.46+0.43+0.36 4 = 0.415 >0.2

# β =8.87.6 =1.12

# hmin= 8800*(0.8+4201400)36+1.15*5*(0.415-0.2) =26.06 cm

# h= 35 cm > hmin= 26.06cm

- Load Calculation:

# Table 4.5: table of two way rib calculation.

| Calculation |  | Parts of Flight | No. |
| --- | --- | --- | --- |
| 22*0.03*0.52*0.52 = 0.178 KN |  | Tiles | 1 |
| 22*0.02*0.52*0.52 = 0.119KN |  | Mortar | 2 |
| 16*0.07*0.52*0.52 = 0.303 KN |  | Sand | 3 |
| 25*0.08*0.52*0.52 = 0.541 KN |  | Topping | 4 |
| 25*0.27*0.12*(0.52+0.4) = 0.745 KN |  | Rib | 5 |
| 9*0.27*0.4*0.4 = 0.389 KN |  | Block | 6 |
| 22*0.02*0.52*0.52 = 0.119 KN |  | Plaster | 7 |
| 2.80 KN | Sum |

# Dead Load of slab:

# DL = 2.800.52*0.52 =10.36 KN/m2

# WD =1.2*10.36 =12.432 KN/m2

# LL=3  KN/m2

# WL =1.6*3=4.8 KN/m2

# W = 12.432+4.8 =17.232 KN/ m2

- Moments Calculations:

# From tables use Case (1 ): C neg = 0.0

# Ca ,dL =0.049  , ,  Cb ,dL =0.032

# Ca ,LL =0.049  , ,  Cb ,LL =0.032

# M+a,d = Ca dl  Wud   ( La)2 = 0.049 12.432  (7.6)2  0.52= 18.3 KN .m /  rib.

M+a L = Ca ll   Wul  ( La )2  = 0.049  4.8  (7.6)2  0.52= 7.06 KN .m /  rib.

Ma positive = 18.3 + 7.06 = 25.36 KN .m / rib.

Negative moment at discontinuous edges = 1/3 positive

Ma neg  =( 1/3 )  25.36  = 8.45 KN .m / rib.

M+b,d = Cb dl  Wud   ( Lb)2 = 0.032  12.432  (8.8)2  0.52= 16.01 KN .m /  rib.

M+b L = Cb ll   Wul  ( Lb )2  = 0.032  4.8  (8.8)2  0.52= 6.18 KN .m /  rib.

Mb  positive = 16.01+6.18 = 22.19 KN .m / rib.

Negative moment at discontinuous edges = 1/3 positive.

Mb neg = (1/3)  22.19 = 5.96 KN.m / rib.

- design of positive moments:

Design of Positive Moment :- (Ma=25.36KN.m)

Assume bar diameter ø 14 for main positive reinforcement

d =h- cover - dstirrups-db2=350-20-8-142=315 mm

Rn=Mu∅bd2=25.36×1060.9×520×3152=0.54Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.54420=0.00130

As, req = ρ.b.d = 0.00130 ×520×315 = 212.94 mm2

Check for As min:-

A=ACI-318 (10.5.1)

A=

A=

A=controls

Asreq = 212.94mm2 >Asmin= 126 mm2 OK

Use 2 ø12, As, provided= 226.2 mm2>As, required= 212.94mm2…  Ok

Check for strain:-

a =As.fy0.85b fc'=226.2×4200.85×520×24=8.96 mm

x=aβ1=8.960.85=10.54mm

εs=0.003d-xx=0.003315-10.5410.54=0.086>0.005                0k

Design of Positive Moment:- (Mb=22.19KN.m)

d =h- cover - dstirrups-db2=350-20-8-142=315 mm

Rn=Mu∅bd2=22.19×1060.9×520×3152=0.47Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.47420=0.001132

As, req = ρ.b.d = 0.001132 ×520×315 = 185.46 mm2

Check for As min:-

A=ACI-318 (10.5.1)

A=

A=

A=controls

Asreq = 185.46mm2 >Asmin= 126 mm2 OK

Use 2 ø12, As, provided= 226.2 mm2>As, required= 185.46mm2…  Ok

Check for strain:-

a =As.fy0.85b fc'=226.2×4200.85×520×24=8.96 mm

x=aβ1=8.960.85=10.54mm

εs=0.003d-xx=0.003315-10.5410.54=0.086>0.005                0k

- design of negative moments:

Design of Negative Moment :- (Ma=-8.43KN.m)

Assume bar diameter ø 12 for negative reinforcement

d =h- cover - dstirrups-db2=350-20-8-122=316 mm

Rn=Mu∅bd2=8.43×1060.9×120×3162=0.78 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.78420=0.002

As, req = ρ.b.d = 0.002 ×120×316 = 75.84 mm2

Check for As min:-

A=ACI-318 (10.5.1)

A=

A=

A=controls

Asreq = 75.84 < Asmin= 126.4mm2 OK

Use 2 ø10, As, provided= 157.08 mm2>As, required= 126.4mm2…  Ok

Check for strain:-

a =As.fy0.85b fc'=157.08×4200.85×120×24=26.95 mm

x=aβ1=26.950.85=31.7 mm

εs=0.003d-xx=0.003316-31.731.7=0.026>0.005                0k

Design of Negative Moment :- (Mb=-7.4KN.m)

Assume bar diameter ø 12 for negative reinforcement

d =h- cover - dstirrups-db2=350-20-8-122=316 mm

Rn=Mu∅bd2=7.4×1060.9×120×3162=0.68 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.68420=0.00164

As, req = ρ.b.d = 0.00164 ×120×316 = 62.18 mm2

Check for As min:-

A=ACI-318 (10.5.1)

A=

A=

A=controls

Asreq = 62.18 < Asmin= 126.4mm2 OK

Use 2 ø10, As, provided= 157.08 mm2>As, required= 126.4mm2…  Ok

Check for strain:-

a =As.fy0.85b fc'=157.08×4200.85×120×24=26.95 mm

x=aβ1=26.950.85=31.7 mm

εs=0.003d-xx=0.003316-31.731.7=0.026>0.005                0k

- design of shear for rib:

Maximum shear coefficient in short direction as in case (1) Wa @ m = 0.863

Wa = 0.64

The total load on the panel =7.6*8.8*17.232= 1152.4 KN

The load per rib at the face of long beam =0.64*1152.4*0.52/(2*8.8)= 21.79 KN

Vud = Vuface - Wu*bf*d = 21.79 – 17.232*0.52*0.316= 18.95 KN

The shear strength of one rib:

Vc =1.16fc'bwd=1.1624 ×120×316×10-3=34.05 KN

øVc =0.75×34.05 =25.54KN

0.5 ø Vc =0.5×25.54=12.77KN

0.5 ø Vc< Vud< ø Vc

12.77<18.95<25.54

Minimum shear reinforcement is required except for joist construction.

Vud = 17.232*0.52*(3.8 - 0.316) = 31.3 KN

# øVc =25.54 < Vud =31.3KN

Ф Vsmin≥ 0.75 () * bw * d =0 .75* ()*120*316*10-3 = 9.48 KN Controls

Ф Vsmin   ≥ 0.75 () * bw * d = 0.75*() * 120* 316*10-3 = 8.7KN

Ф (Vc+ Vsmin)= 35.021>Vu= 31.3 > ФVc = 25.54

Vs'= 13*bw*d  = 61.9 KN

Vs= VuФ-Vc = 12.92 KN

Vsmin =11.4 <Vs =12.92 < Vs’ =55.85 ………Case IV

Use 2 leg Φ 8

Av min =100 mm2

Av mins= vs dfyt

s= Avmin vs  dfyt

S=926.5mm

smax ≤ d2=2852= 142.5 mm                 or smax ≤600 mm

Use 4 leg Φ 8 @125 mm

4.8 Design of One Way Solid Slab

- Material:-

- concrete    B300                Fc' = 24 N/mm2

- Reinforcement Steel         Fy = 420 N/mm2

- Slab Thickness Calculation:-

The overall depth must satisfy ACI Table (9.5.a):

Min H  (deflection requirement):-

-For one end continuous:-

For One way solid slab, will use thickness of slab 20 cm.

- Load Calculation:-

For the one-way solid slabs, the total dead load to be used in the analysis and design is calculated as follows:-

-Load Calculation For the Horizontal Slab:- (For one Meter Strip)

Table 4.6: Dead Load Calculation of Solid Slab.

| # | material | calculation |
| --- | --- | --- |
| 1 | Tiles | 0.03*22=0.66 |
| 2 | mortar | 0.03*22=0.66 |
| 3 | Coarse sand | 0.07*16=1.12 |
| 4 | RC concrete | 0.30*25=7.5 |
| 5 | plaster | 0.02*22=0.44 |
|  | Sum | 10.38 |

Live load =5 KN/m

- Design of Positive Moment :

- Design of Positive Moment :-(Mu=25.3 KN.m)

Assume bar diameter Ф14 for main reinforcement

m =  = = 20.59

Rn =

Rn =  = 0.939(Mpa)

ρ  = (1 - )

ρ = (1 - ) = 0.00228

As=ρ * b * d = 0.00228* 100 *143= 3.3cm²

Check for As min:-

A min = =

Asreq= 3.3cm2  >Asmin= 3.6cm2          NOT  OK

Use ø 12/20cm , As,provided= 4.52cm2>As,required= 3.6cm2 ….  Ok

- Design of Positive Moment :-( Mu=27.6KN.m)

m =  = = 20.59

Rn =

Rn =  = 1.02 (Mpa)

ρ  = (1 - )

ρ = (1 - ) = 0.00245

As=ρ * b * d = 0.00245* 100 *143= 3.56cm²

Check for As min:-

A min = =

Asreq= 3.56cm2  >Asmin= 3.6cm2       Not OK

Use ø 12/20cm , As,provided= 5.65cm2≥As,required= 3.6cm2 ….  Ok

- Design of Negative Moment:

- Design of Negative Moment:- (Mu=23.4 KN.m)

m =  = = 20.59

Rn =

Rn =  = 0.868(Mpa)

ρ  = (1 - )

ρ = (1 - ) = 0.0021

As=ρ * b * d = 0.0021* 100 *173= 3.65 cm²

Check for As min:-

A min = =

Asreq= 3.65cm2  > Asmin= 3.6cm2           OK

Use ø 12/20cm , As,provided= 5.65cm2≥As,required= 3.6cm2 ….  Ok

Shrinkage and Temperature:-

→ρ=0.0018

A min =   =       (control)

Use Ф10 @ 200 mm

- Shear Design:-

Check  Whether Thickness Is Adequate For Shear:-

= 47.4 KN/ 1m strip

d = h – 20 – db = 200 – 20 – (14 /2) = 173 mm

ΦVc=

=  = 105.9 KN / 1 m strip

ΦVc= 79.42 KN > = 47.4 KN/ 1m strip

The thickness of the slab is adequate enough.

4.9 Design of Two Way Solid Slab:

- Calculate the minimum thickness slab :

hmin=22 cm

yCB6=20*120*30+80*20020*80+20+80*20=21.1cm

IbCB6=120*18.933 +    80*2033     =225575.6cm

y(CB5)=2080+40*30+80*3020*(80+40)+20*80=22cm

IbCB5=120*1833 +    80*2033     =225575.6cm

Is1=Is2=380+35*20^312=275000cm4

Is3=325+35*20^312=240000cm4

Is4=655+35*20^312=460000cm4

∝f1=∝f2=Ib66Is2=Ib56Is1=1160412.5275000=4.264

∝f3=Ib55Is3=1172751240000=4.264

∝f4=Ib53Is4=1160412.5460000=2.522

∝fm=∈∝4=2*2.522+2*4.2644=3.393

for∝fm≤0.2β=lnlonglnshort=6.8/6.3=1.07

hmin=ln*(0.8+Fy/1400)36+9B=780*(0.8+420/1400)36+9*1.07=16.3 cm

but we will select 20cm slab thickness.

- Dead load calculations:

Table(4.7) calculation of the two way solid Dead load

| Dead load from: | δ×γ | KN/m |
| --- | --- | --- |
| Tiles | 0.03×23×1 | 0.69 |
| Mortar | 0.02×22×1 | 0.44 |
| Coarse sand | 0.07×16×1 | 1.12 |
| Slab | 0.20×25×1 | 5 |
| Plaster | 0.02×22×1 | 0.44 |
| Partitions | 2*1 | 2 |
|  |  | 8.75 |

Dead load =8.75 KN/m2.

Live load = 5 KN/m2.

WuD = 1.2*Dead load = 1.2*8.75= 10.5 KN/m2.

WuL = 1.6*live load = 1.6*5 = 8 KN/m2.

Wu = 10.5+8 = 18.5 KN/m2

- Shear Design :

la/lb=0.85

Wb=0.79

Wa=0.21

- The total load on the panel being ( 7.6*6.5*19.604) = 968.487 KN

- The load at face of the long beam is (0.79×1614/(2*6.5))=58.86 KN

Assume the Φ 16

d=200-20-12\2=174mm

- Vc =(√28  *1000*174*10^-3)\6 =153.45KN1.16ʎfc'bwd=1.1628×1000×314×10-3=304.614 KN

øVc = 0.75  181.26=  115.1  KN

Vu<  ø Vc.

The thickness of the slab is adequate enough

- Flexural Design:

(la/lb=0.92)

Positive moments :

CaD=0.0318

Cla=.037

CbD=.0228

Clb=.0272

Ma+ve,Dl=Ca*W* La2 =0.0318*10.5*6.3 2= 13.25 KN.m\m

Ma+ve,Ll=Ca*W* La2 =.0374*8*6.3 2= 11.806KN.m\m

Ma+ve = Ma+ve,L + Ma+ve,D = 25.05KN.m\m

Mb+ve,D=Cb*W* Lb2 =0.0228*10.5*6.82= 11.06 KN.m\m

Mb+ve,L=Cb*W* Lb2*b =0.0228*8*6.8 2= 8.43KN.m\m

Mb+ve = Mb+ve,L + Mb+ve,D = 16.83KN.m/m

- Positive Moment:

*Mub =  16.83KN.m/m

Assume the dBar =12 mm

d =h- cover - (dBar\2) =200-20-6=174mm

Rn =

= *1000*176 =534mm2/m

=

=  534mm2≥= 360mm2/m

Use Φ 14 \ 20cm  with As=770mm2/m

*Mua =  25.05KN.m/m

Assume the dBar =12 mm

d =h- cover - (dBar\2) =200-20-6=174mm

= *1000*174 =385mm2/m

=

=  385mm2≥= 360mm2/m

Use Φ 14 \ 20cm with As=770 mm2/m

- Negative Moment:

Mua =  42.5N.m/m

Assume the dBar =12 mm

d =h- cover - (dBar\2) =200-20-6=174mm

= *1000*174 =770mm2/m

=

=  770mm2/≥= 360mm2/m

Use Φ 16 \ 20cm with As=1004.8mm2/m

Note: other moments requires areinforcement less than minimum, Use Φ 12 \ 15cm with As=678 mm2/m

fig (4-28) : Plan and section reinforcement of mat foundation

4.10 Design of Stair:

Figure4.6: Stair Plan.

- Material :-

- concrete    B300                Fc' = 24 N/mm2

- Reinforcement Steel         Fy = 420 N/mm2

- Design of Flight :-

- Determination of Thickness:-

hmin = L/20

hmin = 5.8/20 =29 cm

Take h = 30cm

The Stair Slope by θ = tan-1(15/ 30) = 26.56o

- Load Calculation:-

Dead Load For Flight For 1m Strip:-

Table 4-8: Dead Load Calculation of Flight.

| Calculation |  | Parts of Flight | No. |
| --- | --- | --- | --- |
| 27*0.03*1*(0.35+0.15/0.3 ) = 1.35 KN/m |  | Tiles | 1 |
| 22*0.02*1*(0.3+0.15/0.3 ) = 0.66 KN/m |  | Mortar | 2 |
| 25*1*(0.3+0.15/2 )/0.3 = 1.875 KN/m |  | Stair | 3 |
| 25*0.25*1 / COS 26.56 = 6.99 KN/m |  | Slab | 4 |
| 22*0.03*1 / COS 26.56o = 0.738 KN/m |  | Plaster | 5 |
| 13 KN/m | Sum |

Live Load For Landing For 1m Strip = 4*1 = 4 KN/m

Factored Load For Flight :-

WU = 1.2 ×13+ 1.6×4 =23.6KN/m

Fig 4.7: Stair Section.

R = (W*L)/2=23.6*3.31/2=42.3 KN

- Design of Shear for Flight :- (Vu=27.45 KN)

Assume bar diameter ø 14 for main reinforcement

d =h- cover -db2=300-20-142=273mm

Vu = 42.3KN

Vc = 16fc'bw d=1624*1000*273=222.9KN/m

Φ Vc = 0.75* 222.9 = 167.2KN /m

Vu = 42.3 < Φ VC = 0167.2KN /m

The thickness is enough .

- Design of Bending Moment for Flight :- (Mu=80.25KN.m)

Mu=80.25*1.5+1.6-23.6*1.3522=89.1.m KN

Rn=Mu∅bd2=89.1×1060.9×1000×2732=1.2  Mpa

m=fy0.85fc'=4200.85×24=20.59

ρ=1m1-1-2.m.Rn420=120.591-1-2×20.59×1.2420=0.00294

As,req = ρ.b.d = 0.00294 ×1000×273 = 804.2mm2

As,min= 0.0018*1000*300 = 540 mm2

Asreq >As,min=540  mm2

Asreq =804.2 mm2

Check for Spacing :-

- S = 3h = 3*300 =900 mm

- S = 380*(280/(2/3  * 420)) – 2.5*20 = 330 ≤ S = 300*(280/(2/3  * 420)) = 250mm

- S = 450 mm

S = 250  mm  ……… is control

Use ø14 @ 250 mm

- Lateral or Secondary Reinforcement For Flight :-

As,req= As,min =0.0018*1000*300= 540mm2

Use ø14@ 300 mm  , As,provided= 461.7 mm2>As,required= 450mm2…  Ok

- Design of Landing :

- Load Calculation:-

Dead Load For Landing For 1m Strip:-

Live Load For Landing For 1m Strip = 4*1 = 4KN/m

Factored Load For Landing :-

WU = 1.2 ×9.26+ 1.6×4=19.3KN/m

- System of Landing :-

R=19.3*1.72+21.15*1.9=75.45 KN

- Design of Shear:- (Vu=75.45 KN)

Assume bar diameter ø 14 for main reinforcement

d =h- cover -db2=300-20-142=273mm

Vc = 16fc'bw d= =  1624*1000*273=222.9KN

Φ* Vc = 0.75* 222.9= 1167.2  KN> Vu = 75.45  KN…... Thickness of slab is enough

- Design of Bending Moment :- (Mu=70.6KN.m)

Assume bar diameter ø 14 for main reinforcement

d =h- cover -db2=250-20-142=273mm

Rn=Mu∅bd2=70.6×1060.9×1000×2732=1.052  Mpa

m=fy0.85fc'=4200.85×24=20.59

ρ=1m1-1-2.m.Rn420=120.591-1-2×20.59×0.382420=0.002575

As,req = ρ.b.d = 0.002575×1000×273 = 703 mm2

As,min =0.0018*1000*273 = 540mm2

As,req = 703  mm2……… is control

Check for Spacing:-

- S = 3h = 3*300 =900mm

- S = 380*(280/(2/3  * 420)) – 2.5*20 = 330 ≤ S = 300*(280/(2/3  * 420)) = 200mm

- S = 450 mm

S = 200 mm  ……… is control

Use ø14 @ 200 mm

Fig 4.7: Stair Reinforcement.

4.11 Design of Column

- Material :-

- concrete    B350                Fc' = 24 N/mm2

- Reinforcement Steel        Fy = 420 N/mm2

- Load Calculation:-

Service Load:-

Dead Load =2400KN

Live Load =700 KN

Factored Load:-

PU = 1.2 ×2400+ 1.6×700 =4000 KN

- Dimensions of Column:-

Ag= 315457.4 mm2

Assume Rectangular Section

Try h = 600mm

b = 400

Selecting Longitudinal Bars:

Ast=6980mm2

Use1228 ,Ast,prov=7308mm2>Ast=6980mm2

=Ast/Ag= 0.0125

Fig 4.9:Column section and  reinforcement.

- Design of the tie reinforcement :

S ≤16  db (longitudinal bar diameter)

S≤ 48dt  (tie bar diameter).

S ≤ Least dimension.

spacing  ≤16×db=16×2.8  =44.8cm ….

spacing  ≤48×dt=48×1.0 = 48  cm

spacing  ≤ least.dim  =40 cm control

Use10@25 cm

4.12 Design of shear wall:

hw = 10.5 m , Lw = 6.5 m

d ≤ 0.8* Lw = 0.8 *6.9 = 5.2 m … control

d ≤ 0.8*hw  = 0.8 * 10.5 = 8.4 m

3.5m

2909

1656

620

358

177

296

3.5 m

3.5 m

4.10   Shear force and moment on the wall from ETABS

Lw / 2 = 3.25 m …… control

hw / 2  = 5.25 m

- Design horizontal reinforcement :

So thickness of wall is safe.

- Design for horizontal reinforcement :

= 2 Ф 10  =  158 mm2

S = 316 mm

2*79*10-6s.=0.0006225 Smax  ≤  Lw/5 =6500 /5 = 1300mm

≤ 450  mm

≤  3 * h = 3*200= 600 mm  Take s = 300 mm < s max

Select Ф10@20 cm

- Design for Vertical reinforcement:-

= 2 Ф 10  =  158 mm2

= 2 Ф 10  =  158 mm2

S=298mm

2*79*10-6s.=0.0006225 Smax  ≤  Lw/3 =6500 /3 = 2166 mm

≤ 450mm

≤  3 * h = 3*200= 600 mm

Take s = 250 mm < s max

Select Ф12 -20 cm

- Design of bending moment:

=

Select the boundary element = 960mm

Avs=

So, Boundary is not required.

4.13 Design of Footing:

- Material :-

- concrete    B350                Fc' = 24 N/mm2

- Reinforcement Steel         Fy = 420 N/mm2

- Load Calculations :-

Dead Load = 800 Kn , Live Load = 160 Kn

Total services load = 800 + 160 = 960  Kn

Total Factored  load = 1.2*800 + 1.6*160 = 1216  Kn

Column Dimensions (a*b) = 25*50 cm

Soil density = 20 Kg/cm3

Allowable Bearing Capacity = 350 Kn/m2

Fig 4.11 : Footing plan.

Assume h = 60cm

= 350 – 25*0.6 – 20*0.4 – 25*0.7 = 309.5kn/m2

- Area of Footing :-

A=Pt=960309.3= 3.2 m2

Assume Square Footing

B required = 1.79 m

Select B = 1.9 m

- Bearing Pressure :-

qu = 1216/1.9*1.9 = 336.8 Kn/m2

- Design of Footing :-

- Design of One Way Shear Strength :-

Critical Section at Distance )d( From The Face of Column

Assume h = 60cm , bar diameter ø 14 for main reinforcement      and 7.5 cm Cover

d = 600 – 75 – 14 = 511 mm

Vu = qu * B-a2-d*L

Vu = 336.8* 1.9-0.352-0.511*1.9 = 168.93Kn

- Design of Two Way Shear Strength :-

The punching shear strength is the smallest value of the following equations:-

Where:-

= Perimeter of critical section taken at (d/2) from the loaded area

= 40  for interior column

ФVc =2215 Kn<Vu=1170Kn

- Design of Bending Moment :-

Critical Section at the Face of Column

FR = qu * B-a2*L = 336.8 * 1.9-0.252 *1.9 =527.9Kn

Mu = 527.9* 0.465= 245.7Kn.m

Rn=Mu∅bd2=245.7×1060.9×1900×5112=0.54Mpa

m=fy0.85fc'=4200.85×24=20.59

ρ=1m1-1-2.m.Rn420=117.61-1-2×20.59×0.54420=0.00152

As,req = ρ.b.d = 0.00152×1900×511 = 1480mm2

As,min = 0.0018*1900*600 = 2052 mm2

As,req < As,min = 2052 mm2   ……… is control

Check for Spacing :-

S = 3h = 3*60 = 180cm

S = 380*(28023 * 420) – 2.5*75 = 192.5 cm

S = 45 cm  ……… is control

Use 17ø14, As,provided= 2618 mm2>As,required= 2052 mm2…  Ok

And In Another Direction Use 14 ø14 As,prov=2156mm2

- Development Length In Footing  :-

Tension Development Length In Footing :-

LdT req=910*Fyλfc*ψeψsψtktr+cbdb*db<  300mm

Ktr=0 Nostripes

cb=75+142=82mm  Or cb=1502=75 mm

ktr+cbdb=0+7514=5.3>2.5

ktr+cbdb=2.5

LdT req=910*4201*24*1*1*0.82.5*16=395.75 mm< 300mm

LdT available = 1900-5002 -75= 625 mm

LdT available = 625 mm >ldreq=395.054 mm…….. OK

Compression Development Length In Footing :-

LdCreq= 0.24*Fy*dB24< 0.043*Fy*dB  <200mm

LdCreq= 0.24*420*1424 = 288.05< 0.043*420*14 = 252.84<200mm

LdCreq= 288.05mm

Ldcavailable = 600 – 75 – 14 – 14 = 497mm <LdCreq= 288.0 mm …….. Ok

Lap Splice of Dowels In Column :-

Lsc = 0.071fydb = 0.071420 14 = 477.48 mm > 300 mm

Select  Lsc = 500 mm

Fig 4.12 :Footing Reinforcement Details.

4.14 Design Of  Steel  Truss

- Pos./TS/: Design Steel Truss.

Figure 4-13 shows a gross section of the components that is located over the purlin , and it is as the following :

- Surface layer of galvanized steel sheet with thickness of 0.6 mm.

- Specific kind of duct, its dimension taken from some tables depending on number of spans and the dead and live loads that can it supports.

(Figure 4- 14:Cross section of sheet metal)

Table4-9:sheet metal for 2&3spanes

Table  4-10  shows the values (type, weight ,support load) for duct that will be used to carry the live and the dead loads , consult the number and the length of spans.

- Load calculation :

1-Dead load :

1-load of galvanized steel sheet=0.61000*75=0.05kKN/m2

2-Dead load of heat insulation =0.1m*1kn/m3=0.1KN/m2

3-Sheet metal with thickness of 0.75mm =0.078KN/m2

Total dead load=0.228KN/m2

2-Snow load :

Depending on the table of the snow load which it depends on the height of the building over the sea level which is 1005m, the snow load is :

Sl=h-400400

Sl=1005-400400=1.5KN/m2

Total snow load=1.5KN/m2

Total load (qt=dead load +snow load =0.228+1.5=1.728KN/m2)

From the table (4-8) the bearing load of sheet metal is 5.16 KN/m2

Qu =5.16 KN/m2> qt=1.728 KN/m2  ….ok

Note: the members are A36 (FY=36 ksi and Fu=58 ksi)

By using the previous dead and snow loads determine the max reaction on the purlins , and then apply these values as a linear on the length of the purlin.

- Design of purlins:

Qu=1.5(1.2D.L+1.6S)=1.5(1.2*0.228+1.6*1.5)=   4  KN/m2

-Design of moment

1.1 -12.8 -11.4 -11.1 -9.4 -8. -8.1 -10.2 -8.9 -8.9 -10.2 -8.9 -8.9 -9.4 -8.1 -8. -12.8 -11.1 -11.4 1.1 9.4 4.1 5.3 4.9 5.3 4.1 9.4 1.16 1.48 1.17 1.08 1.16 1.19 1.19 1.16 1.08 1.17 1.48 1.16 2.2 3.3 3.03 2.47 2.75 2.75 2.75 2.75 2.75 2.75 2.47 3.03 3.3 2.2

(Figure 4-15:Moment envelope for purlins)

MU=11.4 kn.m =11.44.448*1000*125.4=101kip.in

Øb Mb ≥ Mu

0.9*36*Zx=101kip.in   Zx=3.1 in3

Select HSS 3×3×38Zx=3.25in3

bt=5.6,ht=5.6

Check compact:

λp=1.12EFy=1.122800036=31.3

λr=1.40EFy=1.402800036=39.1

bt=5.6<λp=31.3    , ….compact section

- -Design of shear:

8.2 11.1 10.3 10.5 10.7 9.9 12.8 -12.8 -9.9 -10.7 -10.5 -10.3 -11.1 -8.2 8.7 -13.3 11.6 -10.4 10.8 -11.2 11. -11. 11.2 -10.8 10.4 -11.6 13.3 -8.7

(Figure 4-16:Shear envelop for purlins(

vU=12.8  kn =12.84.448*1000=2.87kip

Øv vb ≥ vu

0.9*0.6*fy*d*tw ≥ 2.87kip

0.9*0.6*36*3*38 = 21.87kip>2.87 kip…..OK

- Design the member of the truss :

(Figure 4- 17:Truss system)

- The truss consist of four types of members :

- The vertical member (v)

These member are under compression Force :

| NO .of member | Value of compression force |  |
| --- | --- | --- |
|  | Kn | Kips |
| V1 | 19.37 | 4.355 |
| V2(max) | 91.90 | 20.662 |
| V3 | 55.44 | 12.46 |
| V4 | 20.90 | 4.699 |
| V5 | 10.46 | 2.352 |
| V6 | 0.91 | 0.20 |

Table4-11:Vertical  member forces

- The Diagonal member (D)

These member are under tension force :

| NO .of member | Value of tension force |  |
| --- | --- | --- |
|  | Kn | Kips |
| D1(max) | 227.14 | 51.067 |
| D2 | 150.37 | 33.807 |
| D3 | 85.77 | 19.284 |
| D4 | 31.64 | 7.114 |
| D5 | 14.0 | 3.148 |

Table4-12:diagonal member forces

- The top member (T)

These member are under compression Force :

| NO .of member | Value of compression force |  |
| --- | --- | --- |
|  | Kn | Kips |
| T1 | 4.90 | 1.102 |
| T2 | 193.68 | 43.545 |
| T3 | 313.88 | 70.569 |
| T4 | 379.76 | 85.382 |
| T5(max) | 402.69 | 90.537 |

Table4-13:Top member forces

- The bottom  member (B)

These member are under Tension Force :

| NO .of member | Value of tension force |  |
| --- | --- | --- |
|  | Kn | Kips |
| B1 | 187.85 | 42.235 |
| B2 | 310.55 | 69.821 |
| B3 | 377.95 | 84.974 |
| B4(max) | 402.02 | 90.386 |
| B5 | 391.94 | 88.119 |

Table4-13:bottom member forces

Design of tension member:

1st) Diagonal members :

Max. value of tension =227.14 Kn (51.067 Kips)

Check :

- Tensile  yielding:

Pu=Φ*FY*Ag

Ag=51.067 /0.9*36 = 1.58 in2

Try L3*2*3/8 with Ag=1.73 in2

- Tensile rupture strength :

Øt×Pn=0.75×fu×Ae

Øt×Pn=0.75×58×(0.85×1.73)=63.97 Kips < 51.067 Kips…ok

2nd) Bottom members :

Max. value of tension =402.02 Kn (90.386 Kips)

- Tensile  yielding:

Pu=Φ*FY*Ag

Ag=90.386 /0.9*36 = 2.79 in2

Try W6*12 with Ag = 3.55 in2

- Tensile rupture strength :

Øt×Pn=0.75×fu×Ae

Øt×Pn=0.75×58×(0.85×3.55)=131.3 Kips < 90.386 Kips…ok

- Design of compression member:

1st) vertical  members :

Max. value of compression =91.90 Kn (20.662Kips)

Take section member

(Try L3*3*3/8 with Ag=2.11 in2  )

Ag=2.11 in2,rx=0.91 in ,ry=0.91 in .

Length of the member =1.5m(4.92 ft)

-Determine of the reduction factor for slender "Unstiffened element ":

λ=bt=338=8

0.45EFy=0.752900036=12.77

bt=338=8<12.77 …..Qs=1.0

Q=Qa×Qs=1.0

Lrx=4.92*120.91=64.88

0<  L rx<80

k×lr=72+0.75*  Lrx=72+0.75*12*4.920.91=120.66

4.71EQs*fy=4.71290001*36=133.68>120.66

fe=π2×Eklr2=π2×29000(120.66)2=19.66ksi

Fcr=Q×0.658Qfyfe×fy

Fcr=1.0×0.6583619.66×36=16.73 kips

ØPn=Ø×fcr×Ag

ØPn=0.9×16.73×2.11=31.8kips>20.662kips…..ok

Note : use L3*3*3/8  for both of diagonal and vertical members .

2nd) top  members :

Max. value of compression =402.69 kn (90.537 kips)

Assume Klr=75

4.712900036=133.68

fe=π2×Eklr2=π2×29000(75)2=50.88ksi

Fcr=Q×0.658Qfyfe×fy

Fcr=1×0.6581*3650.88×36=26.77kips

Klr=1*12*4.92r=75→r=0.787

Ag=puØFcr=90.537 0.9*26.77=3.76in2

Use W8*15 with Ag=4.44 in2

- Design of weld:

The calculation of weld based on the following :

- Fillet weld is used.

- The plates are A36(fy=36 ksi,Fu=58 ksi)

- The plat thickness is (t=0.5 in)

- The electrodes having FExx=70 ksi

- The shielded metal arc welding (SMAW) is used.

1st) Design of weld between the vertical member and the Gusset plate in the corners of the truss:

The section of the vertical member is angle (L3*3*3/8  ) , Ag=2.11 in2 ,y=0.884.

The value of Max. compression  in the vertical member is Vu=20.662Kips.

(Figure 4-18:) weld between vertical member and  gusset plate)

Max. weld size (amax)=t-116=38-116=516 in

Min . Weld size(amin)=316 in

Use weld size (a)=14in

- Design strength of weld :

Ø×Rnw= Ø× te×0.6×FExx

Ø×Rnw=0.75×0.707×14×0.6×70=5.57 kips

- Design strength of base material :

Ø×Rn= Ø× (0.6×fy)×t=1.0×0.6×36×38= 8.1 kips >5.57 kips….ok

Or

Ø×Rn= Ø× (0.6×fu)×t=0.75×0.6×58×38= 9.79kips >5.57 kips….ok

f1=5.57*3=16.71 kips

f2=20.662-16.71=3.952 kips

lw2=f2Ø×Rnw=3.9525.57=0.71in ……use 1.0 in

2nd) Design of weld between the diagonal member and the gusset plate:

- The section of the diagonal member is angel ( L3*3*3/8 )

- For the vertical member use the same size and dimension of weld for the previous vertical member.

The value if Max. Tension in the diagonal member is Tu = 51.1 kip.

(Figure 4- 191:weld between diagonal member and  gusset plate)

Max. weld size (amax)=t-116=38-116=516 in

Min = Weld size(amin)=316 in

Use weld size (a)=14in

- Design strength of weld :

Ø×Rnw= Ø× te×0.6×FExx

Ø×Rnw=0.75×0.707×14×0.6×70=5.57 kips

- Design strength of base material :

Ø×Rn= Ø× (0.6×fy)×t=1.0×0.6×36×38=8.1kip >5.57 kip….ok

Or

Ø×Rn= Ø× (0.6×fu)×t=0.75×0.6×58×38=9.79kip >5.57 kip….ok

F3=3*5.57=16.71 kips

∑M at F1=0

=16.71*1.5+F2*3-51.1*(3-0.884)=0

F2=27.69 kips

F1=51.1-16.71-27.69=6.7 kips

lw1=f1Ø×Rnw=6.75.57=1.21in ……use 1.5 in

lw2=f2Ø×Rnw=27.695.57=4.97in ……use 5 in

Check for rupture

L=(5+1.5)2=3.25

U=1-xl=1-0.8843.25=0.728

ØtPn=0.75*fu*Ae

ØtPn=0.75*58*0.728*2.11=66.82 kips>51.1 kips…..ok

3rd) Design of weld between the bottom member and the gusset plate:

(Figure 4- 20:weld between gust plate and bottom member)

The section of the bottom member is angel (W6*12)

11 /2.54= 4.33 in

Ru=(Rv+Ry)2*(Rh+RX)2

Rv=PyL=0

Rh=PxL=20.66214.76*2=0.7 kip/in

Ip=2*14.76212=535.93 in3

Rx=M*YIp=0….y=0

Ry=M*xIp=20.662*(4.33  2535.93=0.1

Ru=(0+0.1)2+(0.7+0)2=0.71 kip/in

Ø*Rnw=Ru

0.75*(0.707a)*0.6*70=0.71……a=0.032 in

Take a=216in

- Pos./CS/: Design of Steel Truss Column.

(4-21)System and Loading:

Section:

we choose square section for this column and its dimension is 40*40 cm

.

(Figure 4- 22: Steel truss Column System )

- Design:

By using ETABS program design  we define the section and enter the load and  get the design.

We use 12 Ø16 .

(Figure 4- 23:Steel Truss Column Design)

- Pos./F/: Design of Foundation.

- materials:

- loading:

-vertical load from truss:

Pu=150KN

-Lateral load (wind load):

Wind load=2.04kn/m

The value of moment from wind load :

PW.L =5.1*0.8*0.5=2.04KN/m at the column

M=2.04*8*4=65.3 kN.m

- Design:

By using Atir program design we define the section and enter the load and get the

design.

(Figure 4-24 :Steel  foundation plan)

(Figure 4-25:Steel foundation reinforcment )

`