4

Chapter Four

Structural Analysis and Design

4-1 Introduction.

4-2 Factored Load.

4-3 Determination of thickness.

4-4 Load Calculation.

4-5 Design of Topping.

4-6 Design of One Way Rib Slab.

4-7 Design of Two Way Rib Slab.

4-8 Design of Beam.

4-9 Design of Column.

4-10 Design of Footing.

4-11 Design of Stair.

4-12 Design of Solid Slab of the stair roof.

4.13 Design of Shear Wall.

4-1 Introduction

Concrete is a construction material composed of cement (commonly Portland cement) as well as other cementations materials such as fly ash and slag cement, aggregate (generally a coarse aggregate such as gravel, limestone, or granite, plus a fine aggregate such as sand), water, and chemical admixtures. The word concrete comes from the Latin word "concretus", which means "hardened" or "hard".

Concrete solidifies and hardens after mixing with water and placement due to a chemical process known as hydration. The water reacts with the cement, which bonds the other components together, eventually creating a stone-like material. Concrete is used to make pavements, architectural structures, foundations, motorways/roads, bridges/overpasses, parking structures, brick/block walls and footings for gates.

In This Project, there are three types of slabs: solid slabs, one-way ribbed and two-way ribbed slabs. They would be analyzed and designed by using finite element method of design, with aid of a computer Program called " ATIR- Software" to find the internal forces, deflections and moments for ribbed slabs, and then hand calculation would be made to find the required steel for some members.

The design strength provided by a member, its connections to other members, and its cross-sections in terms of flexure, and load, shear, and torsion is taken as the nominal strength calculated in accordance with the requirements and assumptions of ACI-code.

In this project we use concrete type B300 with compressive strength 
for circular section and  for rectangular section, and reinforcement steel with specified yield strength {fy = 420 N/mm² (MPa)}.

4.2 Factored Loads :-

According to ACI-Code-318-11, the minimum thickness of nonprestressed beams or one way slabs unless deflections are computed as follow:

Wu = 1.2 DL + 1.6 LL                   ACI-code-318-11(9.2.1)

4.3 Determination of Thickness

4.3.1 Determination of Thickness for One Way Rib Slab:

According to ACI-Code-318-05, the minimum thickness of nonprestressed beams or one way slabs unless deflections are computed as follow:

| Minimum thickness( h) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Member | Simply supported | One end Continuous | Both end continuous | Cantilever |
| solid one way slabs | L/20 | L/24 | L/28 | L/10 |
| Beams or ribbed one way slabs | L/16 | L/18.5 | L/21 | L/8 |

Table (4.1): Check of Minimum Thickness of Structural Member.

-The maximum span length for one- end continuous (for ribs):

hmin=L18.5=68018.5=36.75cm

-The maximum span length for both -end continuous (for ribs ):

hmin=L21=50021=23.8cm

-The maximum span length for cantilever (for ribs):

hmin =L8=2108=26.25cm//

Take slab thickness   h=35 cm.)  deflection will be checked (

h =35cm (27cm Hollow block + 8cm Topping).

4.3.2 Determination of Thickness for Two Way Rib Slab:

**In Tow way rib we use kalkal block instead of concrete block with dimension (50*20) cm.

=11700(0.8+4121400)36+5×1.125×(0.315-0.2)=34.94 cm

Select  hmin=40 cm

4-4 –Load Calculation

4.4.1: One - way ribbed slab.

For the one-way ribbed slabs, the total dead load to be used in the analysis and design is calculated as follows:

Fig. (4-1) One way rib slab

Calculation of the total dead load for one way rib slab is shown in the following table:

| Calculation |  | Density | Parts of Rib | No. |
| --- | --- | --- | --- | --- |
| 0.12*0.27*25 = 0.81 KN/m |  | 25 | Rib | 1 |
| 0.08*0.52*25 = 1.04 KN/m |  | 25 | Top Slab | 2 |
| 0.03*0.52*22 = 0.3432 KN/m |  | 22 | Plaster | 3 |
| 0.27*0.4*10 = 1.08 KN/m |  | 10 | Block | 4 |
| 0.07*0. 52*17 = 0.6188 KN/m |  | 17 | Sand Fill | 5 |
| 0.03*0. 52*23 = 0.3588 KN/m |  | 23 | Tile | 6 |
| 0.03*0. 52*22 = 0.3432 KN/m |  | 22 | Mortar | 7 |
| 2.3*.52 = 1.196 KN/m |  |  | partition | 8 |
| 5.79 KN/m |

Table (4 – 2) Calculation of the total dead load for one way rib slab.

Nominal Total Dead Load:

D.L. total = 0.81+1.04+0.3432+1.08+0.6188+0.3588+0.3432+1.196   = 5.79KN/m of rib

Live load = 5 * 0.52 = 2.6 KN/m of rib

4.4.2: Two-way ribbed slab :

Calculation of the total dead load for two way rib slab "Kalkal (50*20)  is shown in the following table:

Dead load:

| Calculation |  | Density | Parts of Rib | No. |
| --- | --- | --- | --- | --- |
| 0.030.540.64 22 =0.2281 |  | 22 | Tiles | 1 |
| 0.020.540.6422 =0.1521 |  | 22 | Mortar | 2 |
| 0.070.540.6416 =0.3871 |  | 16 | Coarse Sand fill | 3 |
| 0.080.540.6425 = 0.6912 |  | 25 | Topping | 4 |
| 0.32(0.4+0.64)0.1425=1.1648 |  | 25 | Concrete Rib | 5 |
| 0.060.50.4 9 = 0.108 |  | 9 | Block | 6 |
| 0.150.260.50.4 =0.0078 |  | 0.15 | Kalkal | 7 |
| 0.020.640.5422 =0.1521 |  | 22 | Plaster | 8 |
| 0.540.641.5=0.5184 |  |  | partition | 9 |
| 3.4096 KN/m |

Table ( 4 – 3) Calculation of the total dead load for two way rib slab.

Nominal Total Dead Load = 3.4096  kN/rib

WuD=1.2×3.4096/(0.54*0.64) =11.84 KN

WuL =1.6  5 = 8 kN/m2

4.5 Design of Topping

Dead load calculation:

| Calculation | Material | No. |
| --- | --- | --- |
| 0.03*23*1= 0.69 KN/m | Tile | 1 |
| 0.03*22*1= 0.66 KN/m | mortar | 2 |
| 0.07*17*1= 1.19 KN/m | Coarse sand | 3 |
| 0.08*25*1 = 2 .0 KN/m | topping | 4 |
| 2 *1 =2 KN/m | Interior partitions | 5 |
| 6.84 KN/m | Sum |  |

Table (4 – 4) Calculation of the total dead load on topping

Live load = 5 *1 KN/m

Wu = 1.2DL + 1.6L  =  (1.2 * 6.84) + (1.6 * 5) = 16.808 KN/m. (Total Factored load)

Fig. (4-2) Toping of slab

Assume slab fixed at supported points (ribs):

Fig 4.3: Topping load and moment diagram.

Mu =

= = 0.216 KN.m /m of strip width .

(M)n > Mu [Strength Condition, where  = 0.55] for plane concrete
         ACI-318-14   (22-5.1)

Sm=== 1066666.67 mm3

2.194 KN.m

Mn = 0.55 * 2.194= 1.2 KN.m

Mn = 1.2 KN.m > Mu = 0.216  KN.m

No reinforcement required by analysis. According to ACI 10.5.4, provide A(s, min) for slabs as shrinkage and temperature reinforcement.

According to ACI 7.12.2.1,  shrinkage = 0.0018

Asmin =  * b * h = 0.0018 * 1000 * 80 = 144mm2 /1m

Try bars Ф 8 with As = 50.26mm2

No. of Ф8 = AsreqAbar = 14450.26 = 2.86 → Spacing(S) = 12.86 = 0.348m = 348 mm

≤ 380 (280fs) – 2.5 * Cc   ≤ 300 (280fs)

S =380*

S =300*

Not more than: Smax = 450 mm

Use  8 / 20 cm, with As provided = 251mm2/1m both directions.

4.6 Design of One Way Rib Slab

Fig. (4 – 4) Rib 26

- /Material :-

- concrete    B300                Fc' = 24 N/mm2

- Reinforcement Steel         fy = 420 N/mm2

By using ATIR program we get the envelope moment and shear diagram as the follows:-

Figure (4-5): Rib  geometry.

Figure (4-6) : Rib Section

Figure (4-7) : loading of Rib 26

Figure (4-8) : Moment Envelop of rib26.

Figure (4-9) : Shear Envelop of rib26.

Design of flexure :-

d= h- cover – (db/2) = 350 – 20 – 8 = 322 mm

Mu(+) = 37.2 kN.m

Check if  a  hf

Mnf = 0.85 fc' b hf (d  ( hf / 2 ) = 0.85 * 24 * 520 * 80 ( 322 – 80/2) */ 10 -6 = 239.3 KN.m

→The section will be designedas rectangular section with b = 520 mm .

Design of Negative moment of rib 18:

# Maximum negative moment = 31.2 kN.m

Mn = 31.2 / 0.9 = 34.7 kN.m

=  = =  20.2

# Rn = = = 2.79Mpa

ρ = (1 - )

ρ = (1 - ) = 0.007312

As = 0.007312 (120) (322) = 282.5

………….the larger is control

282.5>

# of bars = As/ As bar = 282.5/153.93 =2bars                                * Note AФ14 = 153.93 mm²

As provided=307.87 mm2

Select 2 Ф 14mm .

- Check for yielding

Tension = compression

As * fy = 0.85 ** b * a

OK

Design of Positive moment of rib 18

# Maximum positive moment is= 37.2kN.m

Mn = 37.2 / 0.9 = 41.3 kN.m

=  = = 20.2

# Rn = = = 0.766Mpa

ρ = (1 - )

ρ = (1 - ) = 0.0019

As = 0.0019 (520) (322) = 317.38

317.38>

# of bars = As/ As bar = 317.38/201.06 = 2 bars                                * Note AФ16 = 201.06 mm²

As providing =402.12 mm2

Select 2 Ф 12mm .

- Chick for yielding

Tension = compression

As * fy = 0.85 * * b * a

Design of shear:

Vu=25.9

Ф Vc =  Ф * bw * d

=   0.75 *  0.12 * 0.322*1000

= 23.66 KN

  - * Ф Vc = 1.1*23.66 = 26.026 KN.

Check  for  items:-

1/   Vu ≤ Ф Vc/2

25.9 ≤   13.013                      (   X    )

2/   Ф Vc/2 ≤ Vu ≤ Ф Vc

13.013  ≤ 25.9   ≤   23.66      (   X    )

3/   Ф Vc ≤ Vu ≤ Ф Vc + Ф Vsmin

23.66 ≤ 25.9 ≤   33.32            (   √    )

Ф Vsmin  ≥  0.75 () * bw * d   = 0 .75*()*0.12 *0.322 *1000= 9.66 KN.    (control)

≥  0.75 (  * bw * d  = 0.75 *  * 0.12 * 0.322 * 1000 = 8.87 kn.

Ф Vsmin = 9.4  KN.

So Case (3) satisfy

Take Av = 2 Ф 8 = 2 * 50

Av/ s =  Vs/fy * d

2*50/ s = 2.98 /322*412        s = 445 mm

S  ≤  d/2 =  161 mm

≤ 600 m.

Use Ф 8  @ 15 cm c/c

4-7 - Design of Two way rib slab :-

Figure (4-10): Two way ribbed slab

- Material :-

- concrete    B300                Fc' = 24 N/mm2

- Reinforcement Steel         fy = 420 N/mm2

- Dead Load        DL = 3.4096  kN/rib = 9.9 KN/m2        WD = 1.2*9.9 = 11.88 KN/m2

- Live load                          LL  = 5 KN/m2                    WL = 1.6*5 = 8 KN/m2

- W = DL + LL = 11.88+8 = 19.88 KN/m2.

Design of two way ribbed slab:

Design for shear :

Case 1 : discontinuous

La = 10.4 m – short direction .

Lb = 11.7 m – long direction .

m= la / lb = 10.4/11.7 = 0.89

Wa=0.612 – max

Wb=0.388

Total Load=10.4*11.7*19.88= 2419 KN.

Load at short beam=0.612×2419×0.522×10.4=37.0KN

Vud=37.0 -19.88×0.52×0.362=33.26 KN

∅Vc=1.1×0.756×24×140×362×10-3=34.14

12∅Vc=17.1 <Vu<∅Vc

No Need Shear reinforcement .

Design for positive moment :

Case1

La/Lb = 10.4/11.7 = 0.89

CaD=0.0498

CaL=0.0498

CbD=0.0284

CbL=0.0284

Ma+ve=[Cadl.W.La2]

At short direction:

Ma+ve=[0.0498×11.88×10.4 2+0498×8×10.42]×0.54= 57.82 KN.m

At Long direction:

Mb+ve=[0.0284×11.88×11.7 2+0.0284×8×11.72]×0.64= 49.46 KN.m

Design of positive moment at short direction:

d = 400 – 20 – 8 – 20/2 = 362 mm

Mu=57.82 Kn.m

Rn=Mu∅×b×d2=57.820.9×540×3622=0.91

M=4120.85×24=20.20

ρ=120.20×1-1-2×0.91×20.20412=2.26×10-3

As=540×362×2.26×10-3=441.8 mm2

So use 220.

Design of positive moment at long direction:

d = 400 – 20 – 8 – 20/2 = 362 mm

Mu=49.46 Kn.m

Rn=Mu∅×b×d2=49.460.9×640×3622=0.66

M=4120.85×24=20.20

ρ=120.20×1-1-2×0.66×20.20412=1.6×10-3

As=640×362×1.6×10-3=370.688 mm2

So use 218.

Figure (4-11): Two way ribbed slab reniforcement

4.8 Design of Beam

Fig. (4 – 12) Beam 78 .

- Material :-

- concrete    B300                Fc' = 24 N/mm2

- Reinforcement Steel         fy = 420 N/mm2

- Load Calculations:-

Dead Load Calculations for Beam(B 78):-

The distributed Dead and Live loads acting upon B0-3  can be defined from the support reactions of the R26.

The maximum support reaction from Dead Loads for R26 upon B0-3  is 21.33 KN, The distributed Dead Load from the R26 on B78

DL =(21.33/ 0.52) = 41.02 KN / m

Self weight of beam = 6.72 KN / m

Live Load calculations for Beam (B78):-

The maximum support reaction from Live Loads for R26 upon B 78 is 11.61KN . The distributed Live Load from the Rib 26 on B78.

LL =11.61/ 0.52=  22.33 KN/m.

By using ATIR program we get the envelope moment and shear diagram as the follows:-

Figure (4-13) : Beam  Geometry

Figure (4-14) : Load of beam

Figure (4-15) : Moment Envelop for Beam

Figure (4-16) : Shear Envelop for Beam

Design of flexure:-

4.8.1. Design of Positive moment:-

= 148.4 KN .m

C =   d =   * 290= 124.29  mm .

a = B *c = 0.85 * 124.29 = 105.65 mm .

max = 0.85* fc * a * b * (d- a/2)

= 0.85 * 24 * 0.10565 * 0.8 * (0.290 – 0.10565/2) = 409.0 kn.m

Ф Mn = 0.82 *409.0 = 335.38 kn .m

Ф Mn  ≥  148.4 kn.m

The  section is singly

Mn = 148.4 / 0.9 = 164.9kN.m

=  = = 20.2

# Rn = = = 2.45 Mpa

ρ = (1 - )

ρ = (1 - ) = 0.00635

As = 0.00635 (800) (290) = 1474.23

1474.23>

# of bars = As/ As bar = 1474.23/254.5 = 6 bars                                * Note AФ18 = 254.5 mm²

As providing =1527 mm2

Select 6Ф 18mm .

•	Check for yielding

Tension = compression

As * fy = 0.85 *  b * a

ok

4.8.2.Design of negative moment:-

= 167 KN .m

Mn = 167  / 0.9 =185.6 kN.m

=  = = 20.2

# Rn = = = 2.76 Mpa

ρ = (1 - )

ρ = (1 - ) = 0.00723

As = 0.00723(800) (290) = 1676.5

1676.5>

# of bars = As/ As bar = 1676.5/254.5 = 7 bars                                * Note AФ18 = 254.5 mm²

As providing =1781.5 mm2

Select 7 Ф 18 mm .

•	Check for yielding

Tension = compression

As * fy = 0.85 *  b * a

Ok

4.8.3.Design of shear

Vu = 206.6KN

Ф Vc =  Ф * bw * d

=   0.75 *  1* 800 *290

= 142.1 KN

Ф Vc + (2/3) Ф * bw * d  =  142.1 + 94.7 = 236.8 KN .

236.8 > vu= 206.6 KN.      the dimension is big enough .

Check  for  items:-

1/   Vu ≤ Ф Vc/2       (   X    )

2/   Ф Vc/2 ≤ Vu ≤ Ф Vc  (   X    )

3/   Ф Vc ≤ Vu ≤ Ф Vc + Ф Vsmin   (   √    )

Ф Vsmin  ≥  0.75 () * bw * d   = 0 .75*()*800 *290 =58 KN.    (control)

≥  0.75 (  * bw * d  = 0.75 *  * 290 * 800  = 53.3 kn.

Ф Vsmin = 58 KN.

So item (3) satisfy

Take Av = 4 Ф 10 = 4* 78.5

Av/ s =  Vs/fy * d

4*78.5/ s = 58 /290*412

S  ≤  d/2 =145 mm

≤ 600 mm.

Select  S= 10.0 cm

Use Ф 10 (4legs) @ 10.0cm c/c

4.9 Design of column

Select column (C85) for design in basement  floor.

Check Slenderness Effect:

- In 0.25m-Dirction

Lu: Actual unsupported (un braced) length.

K: effective length factor (K= 1 for braced frame).

R: radius of gyration = 0.3 h =

Lu = 3.8  m

M1/M2 =1

K=1 , According to ACI 318-02 (10.10.6.3) The effective length factor, k, shall be permitted to be taken as 1.0.

- In 0.5m-Dirction

EI=0.4Ec Ig1+βd……………………ACI 318-2002Eq.10-15

Ec=4700fc'=470024=23025.20 Mpa

βd=1.2DLPu=281.22470.66=.598

Ig=b×h312=0.5×0.25312=0.000651

EI=0.4×23025.20×0.0006511+.598=3.75Mpa

Pcr=π2EI(KLu)2=3.142×3.75(1×3.8)2=2.56KN

δns=Cm1-Pu0.75Pc=11-470.660.75×2.56×1000=1.32>1

Design of the Reinforcement:

Figure (4-17) : columns section

4.10 Design of Footing

- Material :-

- concrete    B300                Fc' = 24 N/mm2

- Reinforcement Steel         Fy = 420 N/mm2

- Load Calculations :- (From Column Group 2)

Dead Load = 234.35Kn , Live Load = 118.4Kn

Total services load = 234.35+118.4 = 352.75Kn

Total Factored  load = 1.2*234.35+ 1.6*118.4 = 470.66Kn

Column Dimensions (a*b) = 50*25 cm

Soil density = 18 Kg/cm3

Allowable Bearing Capacity = 400Kn/m2

Assume h = 50cm

= 400 – 18*0.40– 25*0.5 – 5= 375.3kn/m2

- Area of Footing :-

A=Pt=352.75375.3= 0.94 m2

Assume Square Footing

Select B = 1.10m

- Bearing Pressure :-

qu = 470.66/1.10*1.10= 389Kn/m2

- Design of Footing :-

- Design of One Way Shear Strength :-

Critical Section at Distance )d( From The Face of Column

Assume h = 50cm , bar diameter ø 14 for main reinforcement      and 7.5 cm Cover

d = 500 – 75 – 20 = 405 mm

Vu = qu * B-a2-d*L

Vu =389* 1.1-0.252-0.405*1.1 = 8.558Kn

- Design of Two Way Shear Strength :-

The punching shear strength is the smallest value of the following equations:-

Where:-

= Perimeter of critical section taken at (d/2) from the loaded area

= 40  for interior column

ФVc =1547.6 KN<Vu=462.7KN

- Design of Bending Moment :-

Critical Section at the Face of Column

Mu = 389*1.1*0.425*0.425/2 = 38.6Kn.m

Rn=Mu∅bd2=38.6×1060.9×1100×4052=0.24 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.24420=0.00057

As,req = ρ.b.d = 0.00057×1100×405 = 256.1 mm2

As,min = 0.0018*1100*500 = 990 mm2

As,req=As,min = 990 mm2   ……… is control

Check for Spacing :-

S = 3h = 3*500 = 1500mm

S = 380*(28023 * 420) – 2.5*75 = 192.5 mm

S = 500 mm

S = 500 mm  ……… is control

Use 7ø14 in Both Direction, As,provided= 1077.6 mm2>As,required= 990mm2…  Ok

Check for strain:-

a =As.fy0.85b fc'=1077.6×4200.85×1100×24=20.17 mm

c =aB1=20.170.85=23.73 mm

εs=0.003d-cc=0.003405-23.7323.73=0.048>0.005 ……0k

- Design of Dowels :-

Load Transfer In Footing :-

A1 = 0.5*0.25 = 0.125 m2

A2 = 1.1*1.1 = 1.21 m2

…………….

No Need For Dowels

As,min = 0.005 * Ac =  0.005 * 500 * 250 = 625 mm2

Use 8ø20, As,provided= 2512 mm2>As,required= 600 mm2…  Ok

Fig 4.18 : Foundation Reinforcement.

4.11 Design of Stair

Fig 4.19: Stair Plan.

- Material :-

- concrete    B300                Fc' = 24 N/mm2

- Reinforcement Steel         Fy = 420 N/mm2

Figure (4-20): Stair diagram

Load Calculation & Design of stair :-

hmin = L/28

hmin = 6.84/28 = 24.4 cm

Take h = 25 cm

The Stair Slope by θ = tan-1(17/ 30) = 29.56o

Fig 4.20: Stair Section.

Dead Load For Flight For 1m Strip:-

| Calculation |  | Parts of Flight | No. |
| --- | --- | --- | --- |
| 23*0.03*1*(0.33+0.17/0.3 ) = 1.15 KN/m |  | Tiles | 1 |
| 22*0.03*1*(0.3+0.17/0.3 ) = 1.04 KN/m |  | Mortar | 2 |
| 25*0.5*0.17*1 = 2.13 KN/m |  | Stair | 3 |
| 25*0.25*1 / cos 29.56 = 7.19 KN/m |  | R.C | 4 |
| 22*0.02*1 / cos 29.56o = 0.51 KN/m |  | Plaster | 5 |
| 12.02 KN/m | Sum |

Table ( 4.5 ): Dead Load Calculation of Flight.

Live Load For Landing For 1m Strip = 5*1 = 5 KN/m

Factored Load For Flight :-

WU = 1.2 ×12.02+ 1.6×5 =22.42 KN/m

| Calculation |  |  | Parts of Landing |  | No. |
| --- | --- | --- | --- | --- | --- |
| 22*0.03*1= 0.66 KN/m |  |  | Tiles |  | 1 |
| 22*0.03*1= 0.66 KN/m |  |  | Mortar |  | 2 |
| 25*0.25*1= 6.25KN/m |  |  | R.C |  | 4 |
| 22*0.02*1= 0.44 KN/m |  |  | Plaster |  | 5 |
| 8.01KN/m | Sum |  |

Dead Load For landing For 1m Strip:-

Table ( 4.6 ): Dead Load Calculation of landing.

- Load Calculation:-

Dead Load For Landing For 1m Strip = 8.01 KN/m

Live Load For Landing For 1m Strip = 5*1 = 5 KN/m

Stair reinforcement Design of one meter strip :- (for fligh)

The value of  Vu  at the center of support = (22.42*3)/2= 33.63KN .

d= 250-20-7 = 223 mm .

ᴓvc = (0.75/6) *24 * 1000*223= 136.56 KN > 33.63 KN ……ok .

Figure (4-21): Load distribution on stairs

Mu max = (33.63*(6.84/2))-(8.81*1.6*(1.6+3/2))-(22.42*3/2*3/4=57.37  KN.m

d = 250-20-7 = 223  mm

A = 0.00315*(100)* (22.3) = 7.024  cm

Use 5Φ 14 @ 25 cm  c/c   with As =7.696  cm2 > 7.024 cm2

Min reinforcment :-

A= 0.0018 * 1000 * 250 = 4.5 cm     Use4 Φ 12 @ 20 cm  c/c

As 7,648 > 4.5  cm2

Design of landing:-

Design of S2:-

Mu = (18.5*(3.6/2))-(8.81*1.5/*(1.5+0.6/2)17.61*0.6/2*0.6/4= 18.63KN.m

d = 250-20-7 = 223  mm

A = 0.001*(100)* (22.3) = 2.23  cm

Use 4Φ 14 @ 25 cm  c/c   with As =6.16  cm2 > 4.91 cm2

Min reinforcment :-

A= 0.0018 * 1000 * 250 = 4.5 cm     Use4 Φ 12 @ 25 cm  c/c

As 4.91 > 4.5  cm2

4-12-Design of solid slab of the stair roof :-

Determination of load:

Dead load=.2*25*1= 5 KN/m2

Live load = 1.5 KN/m2

qu= (1.2*5)+(1.6*1.5) = 8.4 KN/m

h= 20 cm

d = h-2-.5 = 20-2-.5= 17.5 cm

Mu=(qu*l2)/8 = 8.4*6.22/8 = 40.36KN.m

A = 0.0036*(100)* (17.5) = 6.3 cm

Min reinforement:

As min =0.0018*b*h = 0.0018*100*20 = 3.6 cm2

Select As =3.6 cm2

Use  Φ14@ 20 cm c/c  with As =7.7 cm2> 6.3 cm2

Longelir  reinforcement for one meter strip:

As = 0.0018*100*20 = 3.6 cm2

Use Φ 12@ 25 cm with As =4.52 cm2 > 3.6 cm2

Top reinforcement :

Use Φ 8@ 15 cm in   both direction

Design of shear reinforcement :

Vu max =qu*L/2 =(8.4*6.2)/2= 26.04 KN

Φ Vc ≥   Vu max

Φ Vc =

Φ Vc =

107.1 >  26.04 KN/m

No Shear reinforcement is required .

4.13 Design of Shear Wall

- Material and Sections:- (From Shear Wall 1)

- concrete    B300                Fc' = 24 N/mm2

- Reinforcement Steel         Fy = 420 N/mm2

- Shear Wall Thickness       h = 15 cm

- Shear Wall Width             Lw = 3.6 m

- Shear Wall Height            Hw = 26.4 m

- Design of Horizontal Reinforcement:-

The critical Section is the smaller of:

∅Vnmax=∅56fc'hd

=0.75*0.83*24*150*2880 =1317.4 KN>Vu =700 KN

# Vc is the smallest of :

1- Vc=16fc'hd=1624*150*2880=352.7 KN  …….. Control

2- Vc=0.27fc'hd+Nud4lw=0.2724*150*2880+0=571.4 KN

3- Vc=0.05fc+lw0.1fc'+0.2NulwhMuVu-lw2hd

21099.6-16339.43.95=Mu-16339.43.95-3.4⟹Mu=16.881 KN.m

MuVu-lw2=16.881205.1-3.62=-1.78 ≤ 0.00 …….. Neglected

Vu = 700  KN

∅*Vc = 0.75*352.7 = 264.5  KN

Vs,min =* bw * d =  * 150 * 2880 = 232.3 KN ……….. Control

Vs,min = * bw * d =  * 150 * 2880 = 244KN

∅*(Vc + Vs,min) = 0.75*(352.7 + 232.3) = 740.75 KN

Avh = 2*π*1024 = 157.1 mm2

157.1Sh=0.75

Sh= 200 mm

- Maximum spacing is the least of :

= 3600/5 = 720 mm

3*h = 3*150 = 450mm

360 mm ……. Control

Use ϕ10/150 mm for two layers

- Design of Vertical Reinforcement:-

AvvSv=[0.0025+0.52.5-hwLwAvhSh*h-0.0025]*150

AvvSv=[0.0025+0.52.5-26.43.6157.1150*150-0.0025]*150

AvvSv=0.350

Select ∅ 16 in Two Layer

Avh = 2*π*1624 = 402  mm2

402Sv=0.350

Sv= 401  mm

- Maximum spacing is the least of :

= 3600/3 = 1200 mm

3*h = 3*150 = 450 mm

450 mm ……. Control

Use ϕ16/150 mm for two layers