4

Chapter Four

Structural Analysis and Design

4-1 Introduction.

4-2 Check of Minimum Thickness of Structural Member.

4-3 Design of Topping.

4-4 Design of One Way Rib Slab.

4-5 Design of  Beam.

4-6 Design of  Column.

4-7 Design of  Stairs.

4-8  Design of  Isolated Footing.

4-9 Design of Basement Wall.

4-10 Design of  Shear Wall.

4-11 Design of  Retaining Wall.

4-1 | Introduction

Reinforced concrete (RC) is a versatile composite and one of the most widely used materials in modern construction. Concrete is a relatively brittle material that is strong under compression but less so in tension. Plain, unreinforced concrete is unsuitable for many structures as it is relatively poor at with standing stresses induced by vibrations, wind loading and so on.

To increase its overall strength, steel rods, wires, mesh or cables can be embedded in concrete before it sets. This reinforcement, often known as rebar, resists tensile forces. By forming a strong bond together, the two materials are able to resist a variety of applied forces, effectively acting as a single structural element.

Reinforced concrete can be precast or cast-in-place (in situ) concrete, and is used in a wide range of applications such as; slab, wall, beam, column, foundation, and frame construction.

4-1-1 Concrete and its Classifications:

Plain concrete is made by mixing cement, fine aggregate, coarse aggregate, water, and frequently admixtures, Structural concrete can be classified into:

- Lightweight concrete with a unit weight from about 1350 to 1850 ( kgm3)   produced from aggregates of expanded shale, clay, slate, and slag.

- Normal‐weight concrete with a unit weight from about 1800 to 2400 ( kgm3)    produced from the most commonly used aggregates— sand, gravel, crushed stone.

- Heavyweight concrete with a unit weight from about 3200 to 5600 ( kgm3) produced from such materials such as barite, limonite, magnetite, ilmenite, hematite, iron, and steel punching or shot. It is used for shielding against radiations in nuclear reactor containers and other structures.

4-1-2 Compressive  strength of  concrete:

The strength of concrete is controlled by the proportioning of cement, coarse and fine aggregates, water, and various admixtures. The most important variable is (w/c) ratio.

Concrete strength ( fc' ) – uniaxial compressive strength measured by a compression test of a standard test cylinder (150 mm diameter  by 300 mm high) on the 28th day–ASTM C31, C39. In many countries, the standard test unit is the cube (200 x 200 x 200 mm).

The concrete strength depends on the size and shape of the test specimen and the manner of testing. For this reason the cylinder (∅ 150mm by 300 mm high) strength is 80%  of the  150 mm  cube strength and 83% of the  150 mm  cube strength, figure (4-1) demonstrate relation between cylinder and cube concrete test.

Figure (4-1) relation between cylinder and cube concrete test.

4-1-3 Modulus of Elasticity of concrete:

The modulus of elasticity of concrete varies, unlike that of steel, with strength. A typical stress-strain curve for concrete in compression is shown. The initial modulus (tangent at origin), the tangent modulus (at 0.5 fc'  ), and the secant modulus are noted. Usually the secant modulus at from 25 to 50%  of the compressive strength fc' is considered to be the modulus of elasticity. For normal weight concrete, shall be permitted to be taken as  Ec=4700fc'   (Map), figure (4-2) demonstrate stress-strain curve of concrete.

Figure (4-2) stress-strain curve of concrete.

4-1-4 Strength Design method (Ultimate strength method):

In the strength design method, the service loads are increased by factors to obtain the load at which failure is considered to be “imminently”. This load is called the factored load or factored service load. The structure or structural element is then proportioned such that the strength is reached when the factored load is acting. The computation of this strength takes into account the nonlinear stress-strain behavior of concrete.

The strength design method may be expressed by the following:

Strength provided ≥ [strength required to carry factored loads]

Where the "strength provided" (such as moment strength) is computed in accordance with the provisions of a building code, and the "strength required" is that obtained by performing a structural analysis using factored loads.

4-1-5 Load Factors U and strength reduction Factor ∅ :

According to (ACI 318-11 9.2.1) the factor U for overload is given:

U=1.4D

U=1.2D+1.6L+0.5 ( Lr or S or R )

U=1.2D+1.6L+0.5 ( Lr or S or R )

U=1.2D+1.0W+1.0 L+ 0.5( Lr or S or R )

U=1.2D+1.0E+1.0 L+ 0.2 S

U=0.9D+1.0W

U=0.9D+1.0E

Where:

D :dead load .

L :live load.

Lr :roof live load.

S :snow load.

R :rain load.

W :Wind load.

E :Earthquake load.

The factor ∅ (under strength factor) according to ACI demonstrated in figure (4-3).

Figure (4-3) values of understrength factors related to strength condition.

4-1-6 General considerations:

- ACI 318-11 Building code will be used in this project.

- UBC-97 code will be used for lateral loads.

- Ultimate strength design method will be used during the analysis and design of this project.

- The compressive strength of concrete for all structural elements is B300 which equals to  fc'=24 Mpa .

- Yield strength of reinforcing rebar's  fy=420 Mpa .

4-2 | Check of Minimum Thickness of Structural Member:

It will be determined according to (ACI 318-11) to achieve deflection requirements, Figure (4-4) provided minimum thickness from code.

We take the longest beam and rib, then we compare between them in table (4-1).

| Supporting type | min. thickness equation | Rib &Beam | No. of span | min. thickness |
| --- | --- | --- | --- | --- |
| One end continuous | L18.5 | B-R1 | 6 | 5.0518.5=27 cm |
| Both end continuous | L21 | B,B7 | 6 | 7.5321=35 cm |

Table (4-1) Determination of thickness for ribs and beams from maximum values of cases.

The thickness of slab provided from (ACI 318-11) to achieve requirements of deflection, depends on the Flexural stiff ness of slab, by manual calculation comes about  
hmin=35 cm.

So, select Slab thickness h=35 cm 27 cm Hollow Block+8 cm  Topping.

4-3 | Design of Topping:

4-3-1 Load calculations:

Topping in One way ribbed slab can be considered as a strip of 1 meter width and span of hollow block length with both end fixed in the ribs, Table (4-2) shows Load calculations on topping.

| Dead Load Form | Thickness δ (m) | Unit weight γ (kNm3) | γ×δ×1 (kNm) |
| --- | --- | --- | --- |
| Tiles | 0.03 | 23 | 0.03×23=0.69 |
| Mortar | 0.03 | 22 | 0.03×22=0.66 |
| Coarse Sand | 0.07 | 17 | 0.07×17=1.19 |
| Topping | 0.08 | 25 | 0.08×25=2 |
| Dead loads |  |  | 4.54 (kNm) |

Table (4-2) Dead Load calculations on topping.

Live load calculations=4×1=4  ( kNm)

4-3-2 Factored Load:

Total Factored Load:

wu=1.2 4.54+1.64=11.85  (kNm)

Figure (4-5) Topping statically system.

Mu=wuL212=11.85 ×0.4212=0.158  kN.m ..for (1m)of strip width

Strength condition for plain concrete:

∅Mn≥ Mu     ( ACI  22.5.1)  …. where  ∅=0.55

Mn=0.42 λ  fc' Sm     WhereSm for rectangular section of the slab : Sm=bh26=1000×8026

Mn=0.42× 24 ×1000×8026=2.19 kN.m

0.55×2.19=1.2 kN.m≫ Mu=0.158  kN.m

NO Reinforcement is required by analysis, According to ( ACI  10.5.4)  ., provide As.min for slabs as shrinkage and temperature reinforcement.

ρshrinkage=0.0018       According to (ACI 7.12.2.1)

As= ρbt=0.0018 ×1000×80=144 mm2   for 1m strip

Step (s,max) is the smallest of:

- 3h = 3×80 =240 mm         control

- 450mm.

- S =380280fs-2.5Cc=380 28023420-2.5 .20=330mm

Take∅ 8 @ 200 mm   in both direction, S = 200 mm <Smax=240 mm ..OK

4-4 | Design of One Way Rib Slab

4-4-1 Determination of geometry:

Requirements for Ribbed Slab (T-Beam Consideration According to ACI- 318-11).

bw≥10 cm…. select   bw=12 cm

h≤3.5bw=3.5×12=42cm .. select  h=35 cm

tf≥Ln12≥50 mm…. select tf=8cm

Figure (4-6) shows typical section of rib with overall slab thickness h=35cm.

4-4-2 Load calculations for Rib:

fc'=24 Mpa   ,  fy=420 Mpa    , Ll=2  (kNm2)

| Dead Load Form | Thickness δ (m) | Unit weight γ (kNm3) | γ×δ×1 (kNm) |
| --- | --- | --- | --- |
| Tiles | 0.03 | 23 | 0.03×23×0.52=0.359 |
| Mortar | 0.03 | 22 | 0.03×22×0.52=0.343 |
| Coarse Sand | 0.07 | 17 | 0.07×17×0.52=0.619 |
| Topping | 0.08 | 25 | 0.08×25×0.52=1.04 |
| RC Rib | 0.27 | 25 | 0.27×25×0.12=0.81 |
| Hollow Block | 0.27 | 10 | 0.27×10×0.40=1.08 |
| Plaster | 0.03 | 22 | 0.03×22×0.52=0.343 |
| Dead loads /rib |  |  | 4.59 (kNm) |

Table (4-3) Dead Load calculations of rib.

Live load calculations =4×0.52=2.08  ( kNm) / rib

Figure (4-7) shows the location of rib in slab drawing plan.

Figure (4-7) location of Rib (B-R1) in slab drawing plan.

Check for chosen effective flange width (be) According to (ACI 318-11)

be is the smallest of :

- be≤L4= 18204=455 mm  .. OK…. where Lis the smallest clear span of the rib.

- be≤bw+16hf=120+1680=1400 mm.

- be≤center to center between adjacent beams=400+120=520 mm.

4-4-3 Structural analysis of Rib (B-R1):

Figure (4-8) shows rib geometry and loads.

The envelope shear and moment diagrams (for all load combinations). Using the structural analysis and design program (Atir 12) we obtain the Envelope Moment diagram for Rib(B-R1).

Figure (4-9) Moment envelop diagram.

4-4-4 Design Rib for Flexure (B-R1):

Design of (B-R1) for positive moments

Assume bar diameter ∅ 12 for main positive reinforcement.

d=h-cover-dstirrups-db2=350-20-10-122=314 mm

The maximum positive moment in all spans of Rib21  Mu=+19.4  kN.m

Check if a>hf:

Mnf=0.85 fc'bhfd-hf2=0.8524.520.80.314-802=232.53 kN.m

Mnf=232.53kN.m≫ Mu∅=19.40.9=21.6 kN.m ….. a<hf

The section will be designed as rectangular section with b=520 mm.

Rn=Mu∅bd=19.4×1060.9∙520∙3142=0.420 MPa  ,     m=fy0.85 fc'=4200.85∙24=20.58

ρ=1m1-1-2Rnmfy =120.581-1-2∙0.420∙20.58420 =0.001010

As=ρbd=0.001010∙520∙314=164.91 mm2

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙120∙314=109.88 mm2

As,min=1.4420 ∙120∙314=125.6  mm2 -control

As=164.91  mm2> As,min=125.6  mm2….. select  As=Areq=164.91  mm2

Use 2∅12 with  As=2.26 cm2 > As,req=1.6491 cm2

Check for strain:

a=Asfy0.85 fc'b=226∙4200.85∙24∙520=8.95 mm

c=aβ1       ,    β1=0.85

c=8.950.85=10.53 mm

εs=0.003d-cc=0.003314-10.5310.53=0.086>0.005  -OK

Note that:

Moments equals or less than  Mu=+19.4  kN.m  , use  2∅12 for each rib span.

So that other positive moments Mu=+14.2  ,+12.3  kN.m <Mu=+19.4  kN.m   have reinforcement of 2∅12 for each rib span.

Design of (Rib B-R1) for negative moments:

The maximum positive moment in all spans Mu=-14.4  kN.m

Assume bar diameter ∅ 12 for main negative reinforcement.

d=h-cover-dstirrups-db2=350-20-10-122=314 mm

Rn=Mu∅bd=14.4×1060.9∙520∙3142=0.312 MPa

m=fy0.85 fc'=4200.85∙24=20.58

ρ=1m1-1-2Rnmfy =120.581-1-2∙0.312∙20.58420 =0.000749

As=ρbd=0.000749∙520∙314=122.26 mm2

Check for As,min :

As,req=122.26 mm2<As,min= 125.6  mm2….  select  As=As,min=125.6  mm2

Use 2∅12 with  As=2.262 cm2 > As,min=1.256 cm2

Check for strain:

a=Asfy0.85 fc'b=226.2∙4200.85∙24∙120=38.8 mm

c=aβ1       ,    β1=0.85

c=38.80.85=45.64 mm

εs=0.003d-cc=0.003314-45.6445.64=0.0176>0.005  -OK

Note that:

Moments equals or less than  Mu=-14.4  kN.m  , use  2∅12 for each rib span.

So that other negative moments:

Mu=-10.3  ,-11.2 ,-2.9, -8.1   kN.m <Mu=-14.4  kN.m   Have reinforcement of 2∅12 for each rib span.

4-4-5 Design Rib for Shear:

4-4-5-1 Design procedure for shear:

Design of cross section subjected to shear and Flexure:

∅Vn≥Vu

Where:

Vu :factored shear force at the section .

Vn :the nominal shear strength ;

Vn= Vc+Vs

Vc : the nominal shear strenght provided by concrete.

Vc=16 λ  fc'  bwd        , λ=1.0  for normal weight concrete.

Vs : the nominal shear strenght provided by shear reinforcement stirrups.

Vs=Av∙fy∙ds

- Shear conditions and cases (Items):

Figure (4-10) Shear diagonal.

Check for dimensions:

According to (ACI), Vs  shall not be taken  grater than  Vs,max= 23 fc'  bwd   , So :

If   Vs>  Vs,max   the section must be enlarged (Dimensions are not enough) where:

Vs=Vn-Vc=Vu∅-Vc

Case I:

Vu≤ 12 ∅Vc                  -NO shear reinforcement is requered.

Case II:

12 ∅Vc<Vu≤∅Vc      -Minimum shear reinforcement is requered ( Av,min)

Av,min= 116 ∙ fc'∙ bw sfyt     ≥ 13 ∙bw sfyt        , Or in the form:

Av,minSthe maximum of   ≥ 13 ∙bw fyt

≥116 ∙ fc' . bw fyt

Here:

Smax≤ d2      or     Smax≤ 600 mm

Where:

S    :step of stirrups spacing between stirrups.

fyt :yield stress of stirrups .

Case III:

∅Vc<Vu≤∅(Vc+ Vs,min)

Av,minS=Vs,minfyt d  →       Vs,min= Av,minSfyt d

Then, Vs,min  is the maximum of:

Vs,min= 116 ∙ fc' bw∙d      ,       Vs,min= 13 bw d

Minimum shear reinforcement is provided (Av,min) with:

Smax≤ d2      or     Smax≤ 600 mm

Case IV:

∅Vc+ Vs,min<Vu≤ ∅Vc+ Vs'         -stirrups are required

Where:     Vs,min<Vs≤Vs' ,      Vs=Vn-Vc=Vu∅-Vc ,    Vs'=13 fc' bw d

AvS=Vsfyt d

Here:

Smax≤ d2      or     Smax≤ 600 mm

Case V:

∅Vc+ Vs'<Vu≤ ∅Vc+ Vs,max         -stirrups are required

Where:   Vs'<Vs≤Vs,min ,      Vs=Vn-Vc=Vu∅-Vc  ,  Vs'=13 fc' bw d

Vs,max= 23 fc'  bwd      And      AvS=Vsfyt d

Here:

Smax≤ d2      or     Smax≤ 300 mm

The shear envelops of Rib that we consider to design it (B-R1) is:

Figure (4-11) Shear envelop diagram.

The maximum shear force at the distance d from the face of support Vu=19.7 kN.

According to (ACI) Shear strength, VC provided by concrete for the ribs may be taken 10% greater than that for beams. This is mainly due to the interaction between the slab and the closely spaced ribs.

Vn=Vu∅

VC=1.1×16  fc'  bwd= 1.1×16 24 ∙120∙314=33.84 kN

∅VC=0.75∙33.4=25.38 kN

12∅VC= 25.382=12.69 kN <Vu=19.7kN<∅VC=25.38 kN      - Case II

So, Minimum shear reinforcement is required.

Av,minSthe maximum of   ≥ 13 ∙bw fyt=13  ×120420=95.23 ×10-3   -controled

≥116 ∙ fc' . bw fyt= 116×24×120420=87.48 ×10-3

Try  ∅8 stirrup 2 legs with  Av,∅8=100.53 mm2

100.55S=95.23 ×10-3    →      S=1055.86 mm

But:

Smax≤ d2=3142=157 mm      or     Smax≤ 600 mm

Use  ∅8 @ 15 cm < Smax=15.7 cm

4-5 | Design of Beam (B,B7)

Figure (4-7) shows the location of rib in slab drawing plan.

`Figure (4-12) location of Beam (B, B7) in slab drawing plan.

4-5-1 Beam (B,B7) geometry:

Figure (4-13) shows the geometry of beam that considered to design and it’s statically system with section of (80×35 cm)  Hidden beam:

Figure (4-13) Beam (B,B7) geometry.

4-5-1 Load calculations for Beam:

There are 3 sources of load that accts on beam.

- The own weight of the beam and the weights of floor layers within the beam width as a uniform (dead load).

- The load that comes from rib (B-R1) as a uniform (dead and live loads).

- The load that comes from two way ribbed slab as a uniform (dead and live loads).

The following is the description for each one:

4-5-1-1 | the own weight of beam:

Table (4-4) shows the own weight of the beam and the weights of floor layers within the beam width:

| Dead Load Form | Thickness δ (m) | Unit weight γ (kNm3) | γ×δ×1 (kNm) |
| --- | --- | --- | --- |
| Tiles | 0.03 | 23 | 0.03×23×0.8=0.552 |
| Mortar | 0.03 | 22 | 0.03×22×0.8=0.528 |
| Coarse Sand | 0.07 | 17 | 0.07×17×0.8=0.592 |
| RC Beam | 0.35 | 25 | 0.35×25×0.8=8.75 |
| Plaster | 0.03 | 22 | 0.03×22×0.8=0.528 |
| service DL |  |  | 10.95 (kNm) |

Table (4-4) Dead Load calculations of Beam.

4-5-1-2 | the load that comes from rib (B-R1):

Its equals the reactions of (B-R1) divided on reputational unit (0.52 m), Figure (4-14) shows the reactions of rib (B-R1) on beam (B,B7):

Figure (4-14) Reactions of rib (B-R1).

So that:

service DL=22.360.52=43 kNm         ,        service  LL=10.810.52=20.76 kNm

So, the load input to analysis seems like in Figure (4-15):

4-5-2 Design for Flexure of Beam:

The envelope moment diagrams (for all load combinations). Using the structural analysis and design program (Atir 12), in Figure (4-16):

Figure (4-16) Envelope moment diagram for Beam (B,B7).

Design of positive moments:

Assume bar diameter ∅ 22 for main positive reinforcement.

d=h-cover-dstirrups-db2=350-40-10-222=289 mm

The maximum positive moment in all spans of (B,B7)  Mu=+284.6  kN.m

Rn=Mu∅bd=284.6×1060.9∙800∙2892=4.733 MPa

m=fy0.85 fc'=4200.85∙24=20.58

ρ=1m1-1-2Rnmfy =120.581-1-2∙4.733∙20.58420 =0.01301

As=ρbd=0.01301∙800∙289=3008.14 mm2

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙800∙289=674.2 mm2

As,min=1.4420 ∙800∙289=770.67  mm2 -control

As,req=3008.14 mm2>As,min= 770.67  mm2      -OK

Use 8∅22  with  As=30.41 cm2 > As,req=30.0814 cm2

Check for strain:

a=Asfy0.85 fc'b=3041∙4200.85∙24∙800=78.26 mm

c=aβ1       ,    β1=0.85

c=78.260.85=92.07 mm

εs=0.003d-cc=0.003289-92.0792.07=0.00642>0.005   - OK

Check for bar placement:

Sb=800-40×2-10×2-8×227=74.86  mm >25 mm    -OK

Design of positive moment  Mu=+269.9  kN.m

Also assume bar diameter ∅ 22

d=h-cover-dstirrups-db2=350-40-10-222=289 mm

Rn=Mu∅bd=269.9×1060.9∙800∙2892=4.49 MPa

m=20.58

ρ=1m1-1-2Rnmfy =120.581-1-2∙4.68∙20.58420 =0.01223

As=ρbd=0.012839∙800∙289=2827.4 mm2

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙800∙289=674.2 mm2

As,min=1.4420 ∙800∙289=770.67  mm2 -control

As,req=2827.4 mm2>As,min= 770.67  mm2     -OK

Use 8∅22  with  As=30.41 cm2 > As,req=28.274 cm2

Check for strain:

a=Asfy0.85 fc'b=3041∙4200.85∙24∙800=78.26 mm

c=aβ1       ,    β1=0.85

c=78.260.85=92.07 mm

εs=0.003d-cc=0.003289-92.0792.07=0.00642>0.005     -OK

Check for bar placement:

Sb=800-40×2-10×2-8×227=74.86 mm >25 mm    -OK

Design of positive moment  Mu=+248.2  kN.m

Also assume bar diameter ∅ 22

d=h-cover-dstirrups-db2=350-40-10-222=289 mm

Rn=Mu∅bd=248.2×1060.9∙800∙2892=4.13 MPa

m=20.58

ρ=1m1-1-2Rnmfy =120.581-1-2∙4.13∙20.58420 =0.0111

As=ρbd=0.0111∙800∙289=2566.7 mm2

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙800∙289=674.2 mm2

As,min=1.4420 ∙800∙289=770.67  mm2 -control

As,req=2566.7 mm2>As,min= 770.67  mm2     -OK

Use 7∅22  with  As=26.609 cm2 > As,req=25.667 cm2

Check for strain:

a=Asfy0.85 fc'b=2660.9∙4200.85∙24∙800=68.48 mm

c=aβ1       ,    β1=0.85

c=68.480.85=80.56 mm

εs=0.003d-cc=0.003289-80.5680.56=0.00776>0.005     -OK

Check for bar placement:

Sb=800-40×2-10×2-7×226=91 mm >25 mm    -OK

Design of positive moment  Mu=+228.2  kN.m

Also assume bar diameter ∅ 22

d=h-cover-dstirrups-db2=350-40-10-222=289 mm

Rn=Mu∅bd=228.2×1060.9∙800∙2892=3.795 MPa

m=20.58

ρ=1m1-1-2Rnmfy =120.581-1-2∙3.795∙20.58420 =0.010082

As=ρbd=0.010082∙800∙289=2330.96 mm2

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙800∙289=674.2 mm2

As,min=1.4420 ∙800∙289=770.67  mm2 -control

As,req=2330.96 mm2>As,min= 770.67  mm2

take As,req= 2330.96 mm2

Use 7∅22  with  As=26.6093 cm2 > As,req=23.3096 cm2

Check for strain:

a=Asfy0.85 fc'b=2660.93∙4200.85∙24∙800=68.48 mm

c=aβ1       ,    β1=0.85

c=68.480.85=80.56 mm

εs=0.003d-cc=0.003289-80.5680.56=0.00776>0.005     -OK

Check for bar placement:

Sb=800-40×2-10×2-7×226=91 mm >25 mm    -OK

Design of positive moment  Mu=+198.6  kN.m

Also assume bar diameter ∅ 22

d=h-cover-dstirrups-db2=350-40-10-222=289 mm

Rn=Mu∅bd=198.6×1060.9∙800∙2892=3.30 MPa

m=20.58

ρ=1m1-1-2Rnmfy =120.581-1-2∙3.30∙20.58420 =0.00862

As=ρbd=0.00862∙800∙289=1992.9 mm2

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙800∙289=674.2 mm2

As,min=1.4420 ∙800∙289=770.67  mm2 -control

As,req=1992.9 mm2>As,min= 770.67  mm2

take As,req= 1992.9 mm2

Use 6∅22  with  As=22.808 cm2 > As,req=20.1075 cm2

Check for strain:

a=Asfy0.85 fc'b=2280.8∙4200.85∙24∙800=58.7 mm

c=aβ1       ,    β1=0.85

c=58.70.85=69.06 mm

εs=0.003d-cc=0.003289-69.0669.06=0.00955>0.005     -OK

Check for bar placement:

Sb=800-40×2-10×2-6×225=113.6 mm >25 mm    -OK

Design of positive moment  Mu=+154.3  kN.m

Also assume bar diameter ∅ 22

d=h-cover-dstirrups-db2=350-40-10-222=289 mm

Rn=Mu∅bd=154.3×1060.9∙800∙2892=2.566 MPa

m=20.58

ρ=1m1-1-2Rnmfy =120.581-1-2∙2.566∙20.58420 =0.00655

As=ρbd=0.006550∙800∙289=1514.4 mm2

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙800∙289=674.2 mm2

As,min=1.4420 ∙800∙289=770.67  mm2 -control

As,req=1514.4 mm2>As,min= 770.67  mm2

take As,req=1514.4 mm2

Use 4∅22  with  As=15.205 cm2 > As,req=15.1182 cm2

Check for strain:

a=Asfy0.85 fc'b=1520.5∙4200.85∙24∙800=39.13 mm

c=aβ1       ,    β1=0.85

c=39.130.85=46.04 mm

εs=0.003d-cc=0.003289-46.0446.04=0.0158>0.005     -OK

Check for bar placement:

Sb=800-40×2-10×2-4×223=204 mm >25 mm    -OK

Design of Negative moment  Mu=-338.8  kN.m

Assume bar diameter ∅ 20 for main negative reinforcement.

d=h-cover-dstirrups-db2=350-40-10-202=290 mm

Rn=Mu∅bd=388.8×1060.9∙800∙2902=5.59 MPa

m=20.58

ρ=1m1-1-2Rnmfy =120.581-1-2∙5.59∙20.58420 =0.01591

As=ρbd=0.01591∙800∙290=3692.6 mm2

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙800∙290=676.5 mm2

As,min=1.4420 ∙800∙290=773.3  mm2 -control

As,req=3692.6 mm2>As,min= 773.3  mm2      -OK

Use 12∅20  with  As=37.69 cm2 > As,req=36.926 cm2

Check for strain:

a=Asfy0.85 fc'b=3769∙4200.85∙24∙800=97 mm

c=aβ1       ,    β1=0.85

c=970.85=114.1 mm

εs=0.003d-cc=0.003290-114.1114.1=0.00462>0.004   -OK

Check for bar placement:

Sb=800-40×2-10×2-12×2011=41.82 mm >25 mm    -OK

Design of Negative moment  Mu=-348.3  kN.m

Assume bar diameter ∅ 20 for main negative reinforcement.

d=h-cover-dstirrups-db2=350-40-10-202=290 mm

Rn=Mu∅bd=348.3×1060.9∙800∙2902=5.75 MPa

m=20.58

ρ=1m1-1-2Rnmfy =120.581-1-2∙5.75∙20.58420 =0.01648

As=ρbd=0.01648∙800∙290=3825.2 mm2

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙800∙290=676.5 mm2

As,min=1.4420 ∙800∙290=773.3  mm2 -control

As,req=3825.2 mm2>As,min= 773.3  mm2      -OK

Use 13∅20  with  As=40.8407 cm2 > As,req=38.252 cm2

Check for strain:

a=Asfy0.85 fc'b=4084.07∙4200.85∙24∙800=105.1 mm

c=aβ1       ,    β1=0.85

c=105.10.85=123.65 mm

εs=0.003d-cc=0.003290-123.65123.65=0.00404>0.004   -OK

Check for bar placement:

Sb=800-40×2-10×2-13×2012=36.67 mm >25 mm    -OK

Design of Negative moment  Mu=-286.4  kN.m

Assume bar diameter ∅ 20 for main negative reinforcement.

d=h-cover-dstirrups-db2=350-40-10-202=290 mm

Rn=Mu∅bd=286.4×1060.9∙800∙2902=4.73 MPa

m=20.58

ρ=1m1-1-2Rnmfy =120.581-1-2∙4.73∙20.58420 =0.01300

As=ρbd=0.01300∙800∙290=3016.3 mm2

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙800∙290=676.5 mm2

As,min=1.4420 ∙800∙290=773.3  mm2 -control

As,req=3016.3 mm2>As,min= 773.3  mm2      -OK

Use 10∅20  with  As=31.416 cm2 > As,req=30.163 cm2

Check for strain:

a=Asfy0.85 fc'b=3141.6∙4200.85∙24∙800=80.85 mm

c=aβ1       ,    β1=0.85

c=80.850.85=95.12 mm

εs=0.003d-cc=0.003290-95.1295.12=0.0052>0.004   -OK

Check for bar placement:

Sb=800-40×2-10×2-10×209=55.56 mm >25 mm    -OK

Design of Negative moment  Mu=-210.5  kN.m

Assume bar diameter ∅ 20 for main negative reinforcement.

d=h-cover-dstirrups-db2=350-40-10-202=290 mm

Rn=Mu∅bd=210.5×1060.9∙800∙2902=3.48 MPa

m=20.58

ρ=1m1-1-2Rnmfy =120.581-1-2∙3.49∙20.58420 =0.00915

As=ρbd=0.00915∙800∙290=2122.8 mm2

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙800∙290=676.5 mm2

As,min=1.4420 ∙800∙290=773.3  mm2 -control

As,req=2122.8 mm2>As,min= 773.3  mm2      -OK

Use 7∅20  with  As=21.991 cm2 > As,req=21.228 cm2

Check for strain:

a=Asfy0.85 fc'b=2199.1∙4200.85∙24∙800=56.6 mm

c=aβ1       ,    β1=0.85

c=56.60.85=66.59 mm

εs=0.003d-cc=0.003290-66.5966.59=0.01>0.004   -OK

Check for bar placement:

Sb=800-40×2-10×2-7×206=93.3 mm >25 mm    -OK

Design of Negative moment  Mu=-224.6  kN.m

Assume bar diameter ∅ 20 for main negative reinforcement.

d=h-cover-dstirrups-db2=350-40-10-202=290 mm

Rn=Mu∅bd=224.6×1060.9∙800∙2902=3.71 MPa

m=20.58

ρ=1m1-1-2Rnmfy =120.581-1-2∙3.71∙20.58420 =0.00982

As=ρbd=0.00982∙800∙290=2279.9 mm2

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙800∙290=676.5 mm2

As,min=1.4420 ∙800∙290=773.3  mm2 -control

As,req=2279.9 mm2>As,min= 773.3  mm2      -OK

Use 8∅20  with  As=25.133 cm2 > As,req=22.799 cm2

Check for strain:

a=Asfy0.85 fc'b=2513.3∙4200.85∙24∙800=64.7 mm

c=aβ1       ,    β1=0.85

c=64.70.85=76.1 mm

εs=0.003d-cc=0.003290-76.176.1=0.00843>0.004   -OK

Check for bar placement:

Sb=800-40×2-10×2-8×207=77.14 mm >25 mm    -OK

4-4-5 Design Beam for Shear:

The shear envelops of beam (B,B7) as shown in Figure (4-17):

Figure (4-17) Shear envelop diagram of Beam.

The maximum shear force at the distance d from the face of support  Vu=304.2 kN

For Vu = 304.2 kN

VC=16  fc'  bwd= 16 24 ∙800∙290=189.43 kN

∅VC=0.75∙189.43=142.07 kN

Check cases:

Check Case III:

∅Vc<Vu≤∅(Vc+ Vs,min)

Vs,min  is the maximum of:

Vs,min= 116 ∙24  ∙800∙290=71.04 kN

Vs,min= 13∙ 800 ∙290=77.33 kN        -controled

Vu=304.2 kN>∅Vc+ Vs,min=0.75 189.43+77.33=200.07  kN

Case III not satisfied.

Check Case IV:

∅Vc+ Vs,min<Vu≤ ∅Vc+ Vs'

VC=189.43 kN

Vs'=13 fc' bw d=13×24  ×800× 290=378.9 kN

∅Vc+ Vs,min= 200.07 kN<Vu=304.2 kN≤ ∅Vc+ Vs'=426.25     -OK

Shear reinforcement are required

Vs'= Vu- ∅ Vc∅= 304.2-142.070.75=216.2 kN

Use ∅10  2 legs with Av=157.1 mm2

Sreq=420×157.1×290216.2×103=88.5 mm

But:

Smax≤ d2      or     Smax≤ 600 mm

Select 4∅10 @10 cm  stirups

4.6 Design of  Column :

4-6-1 Design Data:

The following table and figures gives the design parameters of column group G:

| Dead load (service) | 4000 kN |
| --- | --- |
| Live load (service) | 1600 kN |
| Length | 4.2 m |
| k | 1 (Braced) |
| b*h | 80*65 cm |
| fy | 420 Mpa |
| f'c | 24 Mpa |
| Concrete cover | 40 mm |
| Bar size | ∅25 mm |
| Type of load | Concentrically Loaded |

Table (4-5): Design Data of column group G .

4-6-2 Factored Loads:

Pu=1.2 D+1.6L

Pu=1.2 4000+1.61600=7360 kN

4-7-3 Selecting Column Dimenssion:

Assum  Ast = 0.02Ag

∅Pn,max=∅0.80[0.85.fcAg-Ast+Ast.Fy]

7360*103 = 0.65*0.80[0.85*24*(Ag-0.02Ag)+(0.02Ag*420)]

Ag=498515.3  mm2

take h = 800
b = 498515.3/800 = 623              take b = 650 mm

Ag= 800*650 =520000 mm2

∅Pn,max=∅0.80[0.85.24240000-Ast+Ast.420]

Ast= 8873.48 mm2

Use ( 10Ǿ25 with As= 9817.47 mm2 < Ast= 8873.48 mm2)

ρ=Ast/Ag

9817.47/520000 = 0.0188

Design of ties :
use ties Ǿ10 with spacing of ties shall not exceed the smallest of :
1- 48 times tie diameter , 48ds  = 48*10 = 480 mm.
2- 16 times the longitudinal bar diameter , 16db = 16*25 = 400     - control
3- the least dimension of the column = 650 mm

use Ǿ10 @ 200 mm


Check for code req. :

1- Clear spacing between longitudinal bar :

Clear space = 400-40*2-10*2-25*54 = 106.25 mm > 40mm
                                                                                    > 1.5db = 1.5*25 = 37.5

2-   0.01 < ρg=0.0188 < 0.08        -ok

3- Number of bars 20 > 4- for rectangular section     -ok

4- Minimum tie diameter Ǿ10 for Ǿ25       - ok

5- Spacing of tie s = 200mm        -ok

6- Arrangement of ties 106.25 < 150mm       -ok

Figure (4-18): Column detailing.

4-7 | Design of Staircase:

live load of  Ll=4 kNm2, assuming rise of 150 mm  , and run of 300 mm , fc'=24 Mpa , fy=420 Mpa .
4-7-1 plan and materials of stair:

The following figure demonstrate the plan of stair that we consider to design it figure (4-19) which is carries a uniform

Figure (4-19): Stair Plan and structural system.

4-7-2 Structural system and minimum thickness:

- The structural system of this stair was taken as a simply supported (one-way solid slab) since that the flight of stair will be supported at the ends of upper and lower landings.

- Minimum Slab thickness for deflection is (for simply supported one-way solid slab) is hmin=L20=2.7+220=23.5 cm  , but  in this case presented here where the slab ends are cast white the supporting beams and additional negative reinforcement is provided , minimum thickness can be assumed to be hmin=L28=2.7+228=16.8 cm.

Take hmin=25 cm

4-7-3 Loads and Reactions calculations:

The applied live loads are based on the plan area (horizontal projection), while the dead load is based on the sloped length. To transform the dead load into horizontal projection the figure below explains how figure (4-20).

θ=tan-1riserun=tan-1150300=26.6°

Figure (4-20): Transformation of dead load into horizontal projection.

- Flight Dead Load computation:

Table (4-6) shows Dead Load calculations on Flight of stair:

| Dead Load Form | Unit weight γ (kNm3) | w (kNm) |
| --- | --- | --- |
| Tiles | 27 | 23×0.150+0.350.3×0.03×1=1.15 |
| Mortar | 22 | 22×0.150+0.350.3×0.03×1=1.1 |
| Stair steps | 25 | 250.3×0.150×0.32×1=1.875 |
| Reinforced concrete (solid slab) | 25 | 25×0.25×1cos 26.6=7 |
| plaster | 22 | 22×0.03×1cos 26.6=0.74 |
| Tota Dead loads kN/m |  | 11.9 |

Table (4-6) Dead Load calculations on flight.

- Landing Dead Load computation:

Table (4-7) shows Dead Load calculations on Landing of stair:

| Dead Load Form | Unit weight γ (kNm3) | γ×δ×1 (kNm) |
| --- | --- | --- |
| Tiles | 23 | 23×0.03×1=0.69 |
| Mortar | 22 | 22×0.03×1=0.66 |
| Reinforced concrete (solid slab) | 25 | 25×0.25×1=6.25 |
| plaster | 22 | 22×0.03×1=0.66 |
| Tota Dead loads kN/m |  | 7.6 |

Table (4-7) Dead Load calculations on Landing.

- Live Load:  Ll=4 kNm2.

- Total Factored Load:   w=1.2 DL+1.6 Ll

For flight :  w=1.2 11.9+1.64=20.7  kNm.

For Landing :  w=1.2 7.6+1.64=15.5  kNm.

4-7-4 Design of flight 1:

The support reaction of flighting is:

7.75*4+20.7*2.72=43.4 kNm. as shown in figure (4-31).

Figure (4-21): Loads and reactions on statically system of flight.

Shear and moment calculations:

- Check for shear strength:

Assume bar diameter ∅14 for main rinforcemnt.

d=h-cover-db2=250-20-142=223 mm

Assume wall width 25 cm

Vu=43.4-7.75×0.150+0.223=40.4  kN

Vc=16fc'bwd=16 ×24×1000×223=182.7 kN .. for 1 m strip

ϕ=0.75-for shear

ϕVc=0.75 ×182.7=136.55 kN ..  for 1m strip

Vu,max=40.4 kN < 12ϕVc=68.27kN

∴The thickness of the slab is adequate enough

- Calculation of maximum moment and steel reinforcement:

Mu,max=43.4×6.72-7.75∙2∙2+2.72-20.7.1.35.1.352

=74.6 kN.m / m

assume bar diameter ∅14 for main rinforcemnt with  , d=223 mm

Rn=Mu∅bd=74.6×1060.9×1000×2232=1.67 Mpa  , m=fy0.85 fc'=4200.85×(24)=20.6

ρ=1m1-1-2Rnmfy =120.61-1-2∙1.67∙20.6420 =0.00415

As=ρbd=0.00415×1000×223=925.5  mm2

As,min=0.0018bh=0.0018×1000×250=450mm2

As=925.5 mm2 >As,min=450 mm2   , use ∅14

Use  7∅14@15 cm  with  As,prov=1077 mm2>As=925.5 mm2for 1m strip

Check maximum step for main reinforcement (the smallest of):

- 3h=3×250=750 mm

- 450mm.

- S=380 280fs-2.5Cc=38028023×420-2.5×20=330mm

Smax=300 280fs=30028023×420=300mm -controled

S=15 cm < Smax=30 cm-OK

- Temperature and shrinkage reinforcement:

Astemperature and shrinkagr=0.0018bh=0.00181000250=450 mm2

Use  4∅12@20

cm  with  As,prov=452.4 mm2>As=450 mm2for 1m strip

Check maximum step for temperature and shrinkage (the smallest of):

- 5h=5×250=1250 mm

- 450mm.  -controled

S=20 cm < Smax=45 cm-OK

Figure (4-22): Detailing of flight 1.

4-7-5 Design of flight 2:

The support reaction of flighting is:

7.75*4+20.7*2.72=43.4 kNm. as shown in figure (4-23).

Figure (4-23): Loads and reactions on statically system of flight.

Shear and moment calculations:

- Check for shear strength:

Assume bar diameter ∅14 for main rinforcemnt.

d=h-cover-db2=250-20-142=223 mm

Assume wall width 30 cm

Vu=43.4-7.75×0.150+0.223=40.4  kN

Vc=16fc'bwd=16 ×24×1000×223=182.7 kN .. for 1 m strip

ϕ=0.75-for shear

ϕVc=0.75 ×182.7=136.55 kN ..  for 1m strip

Vu,max=40.4 kN < 12ϕVc=68.27kN

∴The thickness of the slab is adequate enough

- Calculation of maximum moment and steel reinforcement:

Mu,max=74.6 kN.m / m

assume bar diameter ∅14 for main rinforcemnt with  , d=223 mm

Rn=Mu∅bd=74.6×1060.9×1000×2232=1.67 Mpa  , m=fy0.85 fc'=4200.85×(24)=20.6

ρ=1m1-1-2Rnmfy =120.61-1-2∙1.67∙20.6420 =0.00415

As=ρbd=0.00415×1000×223=925.5  mm2

As,min=0.0018bh=0.0018×1000×250=450mm2

As=925.5 mm2> As,min=450 mm2   , use ∅14

Use 7∅14@15 cm  with  As,prov=1077 mm2>As,min=450  mm2for 1m strip

Check maximum step for main reinforcement (the smallest of):

- 3h=3×250=750 mm

- 450mm.

- S=380 280fs-2.5Cc=38028023×420-2.5×20=330mm

Smax=300 280fs=30028023×420=300mm -controled

S=15 cm< Smax=30 cm-OK

- Temperature and shrinkage reinforcement:

Astemperature and shrinkagr=0.0018bh=0.0018.1000250=450 mm2

Use  3∅14@30 cm  with  As,prov=461.7 mm2>As=450 mm2for 1m strip

Check maximum step for temperature and shrinkage (the smallest of):

- 5h=5×250=1250 mm

- 450mm.  -controled

S=30 cm≤ Smax=45 cm-OK

Figure (4-24): Detailing of flight 2.

4-7-6 Design of  Landing:

The support reaction of flighting is:

7.75*2.2+7.75*2.2+(15.5*0.2)2=18.6 kNm. as shown in figure (4-25).

Figure (4-25): Loads and reactions on statically system of landing.

Shear and moment calculations:

- Check for shear strength:

Assume bar diameter ∅14 for main rinforcemnt.

d=h-cover-db2=250-20-142=223 mm

Assume wall width 30 cm

Vu=18.6-7.75×0.150+0.223=15.7  kN

Vc=16fc'bwd=16 ×24×1000×223=182.7 kN .. for 1 m strip

ϕ=0.75-for shear

ϕVc=0.75 ×182.7=136.55 kN ..  for 1m strip

Vu,max=15.7 kN < 12ϕVc=68.27kN

∴The thickness of the slab is adequate enough

- Calculation of maximum moment and steel reinforcement:

Mu,max=3.5 kN.m / m

assume bar diameter ∅14 for main rinforcemnt with  , d=223 mm

Rn=Mu∅bd=3.5×1060.9×1000×2232=0.08 Mpa  , m=fy0.85 fc'=4200.85×(24)=20.6

ρ=1m1-1-2Rnmfy =120.61-1-2∙0.08∙20.6420 =0.00019

As=ρbd=0.00019×1000×223=41.6  mm2

As,min=0.0018bh=0.0018×1000×250=450mm2

As=41.6 mm2< As,min=450 mm2   , use ∅14

Use 3∅14@20 cm  with  As,prov=461.7 mm2>As,min=450  mm2for 1m strip

Check maximum step for main reinforcement (the smallest of):

- 3h=3×250=750 mm

- 450mm.

- S=380 280fs-2.5Cc=38028023×420-2.5×20=330mm

Smax=300 280fs=30028023×420=300mm -controled

S=20 cm< Smax=30 cm-OK

- Temperature and shrinkage reinforcement:

Astemperature and shrinkage=0.0018bh=0.00181000250=450 mm2

Use 4∅12@20 cm  with  As,prov=452.4 mm2>As=450 mm2for 1m strip

Check maximum step for temperature and shrinkage (the smallest of):

- 5h=5×250=1250 mm

- 450mm.  -controled

S=30 cm≤ Smax=45 cm-OK

4-8 | Design isolated Footing(F1):

4-8-1 Materials and Loads:

Isolated footing that we consider to design with materials of:

fc'=24 Mpa , fy=420 Mpa .

Dead Load (service) =1000 kN.

Live Load (service) =200 kN.

Total services load = 1000+200=1200 kN.

Total Factored load = 1.21000+1.6200=1520 kN.

Column dimension a×b=40 cm×60 cm.

Soil density = 18 (KNm3).

Allowable bearing capacity qall=400 (kNm2)

Figure (4-26): Footing Section detailing.

Assume h =40 cm.

- Area of footing:

A=ptq all-net=1200400= 3.00 m2

Assume rect. Footing

Select B=1.65 m

Select L=1.85 m

- Bearing pressure:

qu=1520 1.65×1.85=498  (kNm2)

4-8-2 Design:

- Design of one-way shear strength:

Critical Section at Distance d From The Face of Column Assume  =40 cm .

Bar diameter ∅18 for main reinforcement and 7.5 cm Cover.

d =550-75-20=307 mm

Vu =qu×B-a2-d×L=498×1.65-0.42-0.307×1.85=293 kN

Figure (4-27): one-way shear calculation.

∅VC=∅×16×fc'×b×d=0.75×16×24×1650×307=310.19 kN

∅VC=310.19kN>Vu=293 kN –Safe

- Design of Tow-way shear strength:

Vu=pu-FRb

FRb=qu×area of critical section

Vu=498×1.65*1.85-0.6+0.307(0.4+0.307)=1200.8 kN

The punching shear strength is the smallest value of the following equations:

- ∅VC=∅×161+2βc×fc'×b°×d

- ∅VC=∅×112(αsb°d+2)×fc'×b°×d

- ∅VC=∅×13×fc'×b°×d

Where:

βc=column Length (a)column width (b)=6040=1.5

b°=Perimeter of critical section taken at (d/2) from the loaded area.

=2×0.6+0.307+2×0.4+0.307=3.228 m

αs=40 for interior coulmn

Substituting values in equations:

∅VC=0.75×161+21.5×24×3228×307=1416 kN
∅VC=0.75×11240*0.3073.228+2×24×3228×307=1761.17 kN

∅VC=0.75×13×24×3228×307=1213.72 kN-CONTROL

∅VC=1213.72 kN>Vu=1200 kN

- Design Bending moment for long dirction:

Critical Section at the Face of Column

select ∅18

d = 400-75-18/2 = 316mm

Mu=498×1.85×0.73×0.732=245.5 kN.m

Rn= Mu∅bd2=245.5×1060.9×1850×3162=1.48 MPa

m=4200.85×24=20.58

ρ=1m1-1-2.m.Rn420=120.581-1-2×20.58×1.48420=0.00266

AS,req=ρ×b×d=0.00266×1850×316=1519.96 mm2

ASmin=0.0018×1850×400=1332 mm2

AS,req=1519.96  mm2>ASmin=1332  mm2 –OK

Check maximum step (S) is the smallest of:

- 3h=3×400=1200 mm

- 450 mm-control

Use 10∅14 with As,prov=1539.38 mm2>AS,req 1519.96  mm2

S=1850-75*2-10*14 / 9 = 173.33 mm

S=173.33 < Smax =450 mm, select S=150mm

Check for strain:

a=As.fy0.85b fc'=1539.38×4200.85×1850×24=17.13 mm

c=aB1=17.130.85=20.15 mm

εs=0.003d-cc=0.003316-20.1520.15=0.044>0.005 ……0k

- Design Bending moment for short dirction:

Critical Section at the Face of Column

select ∅14

d = 400 -75 - 14 -14/2 = 304mm

Mu=498×1.65×0.73×0.732=218.94 kN.m

Rn= Mu∅bd2=218.94×1060.9×1650×3042=1.59 MPa

m=4200.85×24=20.58

ρ=1m1-1-2.m.Rn420=120.581-1-2×20.58×1.59420=0.0025

AS,req=ρ×b×d=0.003×1650×304=1254 mm2

ASmin=0.0018×1650×400=1188 mm2

AS,req=1254  mm2>ASmin=1188 mm2 –OK

Check maximum step (S) is the smallest of:

- 3h=3×400=1200 mm

- 450 mm-control

Use 9∅14 with As,prov=1385.44 mm2>AS,req=1254  mm2

S=1650-75*2-9*14 / 8 = 188.625 mm

S=188.625 < Smax =450 mm, select S=150mm

Check for strain:

a=As.fy0.85b fc'=1385.44×4200.85×1650×24=17.29 mm

c=aB1=17.290.85=20.34 mm

εs=0.003d-cc=0.003304-20.3420.34=0.042>0.005 ……0k

- Development Length In Footing  :-

Tension Development Length In Footing :-

LdT req=910*Fyλfc*ψeψsψtktr+cbdb*db<  300mm

Assume Ktr=0 No stripes

cb=50+142=57mm  Or cb=1502=75 mm

ktr+cbdb=0+5714=4.07>2.5

ktr+cbdb=2.5

LdT req=910*4201*24*1*1*0.82.5*14=345.67 mm< 300mm

LdT available = 1850-6002 -75= 600 mm

LdT available = 600 mm >ldreq=345.67 mm…….. OK

Compression Development Length In Footing :-

LdCreq= 0.24*Fy*dB24< 0.043*Fy*dB  <200mm

LdCreq= 0.24*420*1424 = 288.06< 0.043*420*14 = 252.84<200mm

LdCreq= 288.06 mm

Ldcavailable = 600 – 75 – 14 – 14 = 497mm <LdCreq= 288.06 mm …….. Ok

Lap Splice of Dowels in Column:-

Lsc = 0.071fydb = 0.071420 14 = 417.48 mm > 300 mm

Select Lsc = 500 mm

Figure (4-28): Detailing of footing.

4.9 Design of  Basement Wall:

- Material :

- concrete    B300             Fc' = 24N/mm2

- Reinforcement Steel         Fy = 420 N/mm2

∅=35°                  γ=18.00KNm3

Ko=1-sin∅
       =1-sin35

=0.426

- Load on basement wall:

For 1m length of wall:

* Weight of backfill:

q1=Ko*γ*h

= 0.426*18.0*3.0 =23.0 KN/m

* Load from live load:

LL=5 KN/m2

q2=Ko*LL

= 0.426 * 5 =2.13 KN/m

q1 due to triangular loading, q2 is due to uniform loading.

By using structural analysis software ( Atir ) we obtain the shear and moment diagrams.

Figure (4-29): Moment and Shear envelops of basement.

- Design of the shear force:

Assume h = 300 mm,

d=300-20-14=266 mm

Vmax =50.6 KN

= 162.89  KN  ,  Vu  >

No shear Reinforcement is required.

- Design of bending moment:

For Mu max = 25.91  KN.m :

Mn=Mu0.9=25.910.9=28.788 KN.m

Rn=Mn*106b*d2=28.788*1061000*2662=0.407 Mpa

m=Fy0.85*fc'=4200.85*24=20.59

ρ=1m*1-1-2*Rn*mFy

=120.59*1-1-2*0.407*20.59420

=0.979*10-3

Asreq=ρ*b*d=0.979*10-3*1000*266=2.6 cm2/m

Aminfor flexture=0.25fcfybw d = 7.75 cm2/m ≥ 1.4 fy bw d = 8.86 cm2/m

⁖  Amin ≥Areq

n = 886113.1 = 7.8 , take 8∅12/m  or ∅12@125 mm

→ Select ∅12@12.5cm/m

Vertical reinforcement at compression face:

As req =As min = 3.60 cm2/m

∅12@10cm/m

- Design of the horizontal reinforcement:

Asmin =0.0012*b*h=0.002*1000*300=6 cm2/m

Select∅12@30cm/m, in two layer.

Figure (4-30): Detailing of basement.

4.10 Design of Shear Wall (SW,4)

Fig 4.31: Shear Diagram of Shear Wall.

Fig 4.32: Moment Diagram of Shear Wall.

- Material and Sections:- (From Shear Wall 4)

- concrete    B300                Fc' = 24 N/mm2

- Reinforcement Steel         Fy = 420 N/mm2

- Shear Wall Thickness       h = 25 cm

- Shear Wall Width             Lw = 4.0 m

- Shear Wall Height            Hw = 4.2 m

- Design of Horizontal Reinforcement:-

The critical Section is the smaller of:

∅Vnmax=∅56fc'hd

=0.75*0.833*24*250*3200 =2448.5 KN>Vu =1080.5KN

# Vc is the smallest of :

1- Vc=16fc'hd=1624*250*3200=653.2KN  ……..Control

2- Vc=0.27fc'hd+Nud4lw=0.2724*250*3200+0=1058.2KN

3- Vc=0.05fc+lw0.1fc'+0.2NulwhMuVu-lw2hd

=0.0524+4.000.124+01.34250*3200=1365.9 KN

⟹Mu=3605 KN.m

MuVu-lw2=36051080.5-4.002=1.34

Vc =653.2KN

Vu = 1080.5 KN > 12 * 0.75*653.2 = 244.95 KN       Needs reinforcement

∅*vc+∅vs=vu

∅*vs=vu-∅*vc

Vs=vu/∅-vc

Vs=1080.5/0.75-653.2= 787.5 KN

Avhsh=vsfyd=787.5420*3200= 0.000586 mm2/ m

- Maximum spacing is the least of:

=  = 800 mm

3*h = 3*250 = 750 mm

450 mm ……. Control

Take  = 0.0025

Try ∅10 (AS=78.5 mm2) two layers

=AvhhSh=2*78.5250Sh=0.0025

Sh= 251.2mm

→ use ∅10@200 mm in tow layer

- Design of Vertical Reinforcement:-

AvvSv=AvvSv=[0.0025+0.52.5-hwLwAvhSh*h-0.0025]*300

[0.0025+0.52.5-214.00157200*250-0.0025]*300

AvvSv=0.486

Try ∅14 (AS=154 mm2) two layers

2*154Sv=0.486

Sv= 634 mm

- Maximum spacing is the least of :

=  = 1333.3 mm

3*h = 3*250 = 750mm

450 mm ……. Control

→ use ∅14@200 mm in tow layer

- Design of Bending Moment:-

# Ast=4000200*2*154=6160 mm2

# w=AstLwhfyfc'=61604000*25042024=0.1078

α=Pulwhfc'=0

Clw=w+α2w+0.85β1=0.1078+02*0.1078+0.85*0.85=0.115

∅Mn=∅0.5Astfylw(1+PuAstfy)(1-clw)

=0.90.5*6160*420*4000(1+0)(1-0.115)=4121.4 KN≧3605 KN.m

Mub = Mu-∅Mn = 3605 - 4121.4 = -516.4 KN.m

X≥lw600*∆hhw=4000600*.1=6.67 mm

Lb≥X2=3.34 mm

Since Smallest value of Lb & Mub not requires Boundary.

- Detaling of shear wall (SW4):-

Fig 4.33: Detailing of Shear Wall.

4.11 Design of  Retaining Wall:

- Material :

- concrete    B300             Fc' = 24N/mm2

- Reinforcement Steel         Fy = 420 N/mm2

∅=35°                  γ soil=18.00KNm3              γ conc=25KNm3

coefficient of friction between concrete and soil is μ = 0.5

allowable soil presssure=400 KN/m3

surcharge=5 KN/m2

- Size of  retaining  wall:

Fig 4.34: Section plan of retaining wall.

Ca=1-sin∅1+sin∅=1-sin351+sin35=0.271

Pa1 = Caωh = 0.271× 18 × 3.5 = 17.07 KN/m2

Pa2 = Cap = 0.271 × 5 = 1.36 KN/m2

Ha1=Cawh22=Pa1h2=17.07*3.52=30

Ha2 = Caph = Pa2h = 1.36 × 3.5 = 4.76 KN

Ha = Ha1 + Ha2 = 30 + 4.76 = 34.76 KN

Cp=1Ca=3.7

Hp = Cpwh22 = 3.718*0.622 = 12 KN/m2

Fig 4.35: loads of Retaining Wall on stem .

| Weight [KN] | Arm [m] | Moment [KN ∙ m] |  |
| --- | --- | --- | --- |
| ω1 = 0.25 × 3 × 25 =18.75 | 0.252+ 0.5 = 0.625 | 11.72 |  |
| ω2 = 0.6 × 2.9 × 25 =43.5 | 1.45 | 63.1 |  |
| ω3 = 2×3 × 18 = 108 | 1.9 | 205.2 |  |
| ω4 = 2.15 × 5 = 10.75 | 1.83 | 19.6 |  |
| ω5 =0.15×32× 25= 5.625 | 13× 0.15 + 0.25 + 0.5 = 0.8 |  | 4.5 |
| ω6 =0.15×32× 18= 4.05 | 23× 0.15 + 0.25 + 0.5 = 0.85 |  | 3.44 |

Table (4-8) Moment and Loads Calculation.

∑ω = 190.7 KN

Mb = ∑M = 307.6 KN ∙m .

F = μR + Hp,= F = 0.5 ×190.7 + 12 = 107.35 KN.

factor of safety against sliding is  FHa=107.3534.76=3.09>1.5           . .ok

The overturning moment is Mo=30 3.53+4.76 3.52=43.33  KN.m

The balancing moment, Mb, taken about the toe end 0 is Mb = 307.6 KN ∙ m.

The factor of safety against overturning isMbMo=307.643.33=7.1>2          . .ok

x=Mb-Mo∑ω =307.6-43.33190.7=1.386

The eccentricity is e = 1.45 - 1.386 = 0.064 m.

L6 =2.96=0.483 m>e=0.064 m

I=1*2.9312=2.03 m4   ,     Area=2.9 m2

q max= 190.72.9+ 190.7*0.0642.03×1.45=74.5 KN/m2

q min= 190.72.9- 190.7*0.0642.03×1.45=57 KN/m2

- Design of stem:

# Shear  design

Ha1=Cawh22=0.271×18×322=22 KN

Ha2 = Caph = 0.271×5 × 3 = 4.07 KN

Ha = Ha1 + Ha2 = 22 + 4.07 = 26.07 KN

Vu = 1.6 × 26.07 = 41.7 KN

d=400-75-202=315 mm

# Vu = 41.7 KN <12ϕV=96.5 KN

# The thickness of 40 cm at the stem end is adequate enough.

# flexural  design

Mu = 1.6 × [22 ×33+ 4.07×32 ] =45 KN ∙ m

Take ϕ = 0.9 for flexure

Rn=Mu*106 ϕb*d2=45*1060.9*1000*3152=0.504 Mpa

m=Fy0.85*fc'=4200.85*24=20.59

ρ=1m*1-1-2*Rn*mFy

=120.59*1-1-2*0.504*20.59420

=1.22*10-3

Asreq=ρ*b*d=1.22*10-3*1000*315=383 mm2/m

Try  12 @ 200 mm with As =565.5 mm2/m

Vertical    As,min = 0.0012bh = 0.0012 × 1000 × 400 = 480 mm2/m

As = 565.5 mm2/m > As,min,Vertical = 480 mm2/m      OK

# → Select ∅12@20cm/m

Temperature and shrinkage reinforcement: The minimum horizontal reinforcement at the

base of the wall according to ACI Code, Section 14.3, is Horizontal

Horizontal    As,min = 0.002bh = 0.002 × 1000 × 400 = 800 mm2/m

assuming a ∅ 16  bars or smaller.

Because the front face of the wall is mostly exposed to temperature changes,

use one-half to two-thirds of the horizontal bars at the external face of the wall

and place the balance at the internal face.

0.5As = 0.5 × 800 = 400 mm2/m

Use ∅ 12 horizontal bars spaced at 200 mm ,with As =565.5 mm2/m

# → Select ∅12@20cm/m

- Design of toe:

Fig 4.36: Loads of Retaining Wall on toe.

The downward pressure due to self-weight of the toe slab = 0.5 × 25 = 12.5 𝐾𝑁/𝑚2

𝑇ℎ𝑒 𝑠𝑙𝑜𝑝𝑒 = 74.5-572.9=6.03

The pressure at the face (section A)

qA = 76.7 - 0.5 × 6.03 = 73.7KNm2

d = 500 - 75 -202= 415 mm

# Shear  design

The pressure at distance d from the face

qud = 74.5 - 0.085 × 6.03 = 74 KN/m2

Vud = 1.6 × 74.5 + 742× 0.085- 0.9 × 12.5 × 0.085=9.14 KN

# Vud = 9.14 KN <12ϕV=127.07 KN

# The thickness of 50 cm at the toe slab end is adequate enough.

# flexural  design

Mu = 1.6 × 74.5-73.72×0.5 ×23×0.5+ 73.7×0.522 -0.9×12.5× 0.522=13.44 KN ∙ m

Take ϕ = 0.9 for flexure

Rn=Mu*106 ϕb*d2=13.44*1060.9*1000*4152=0.088 Mpa

m=Fy0.85*fc'=4200.85*24=20.59

ρ=1m*1-1-2*Rn*mFy

=120.59*1-1-2*0.088*20.59420

=2.1*10-4

Asreq=ρ*b*d=2.1*10-4*1000*415=87.14 mm2/m

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙1000∙415=1182 mm2/m

As,min=1.4420 ∙1000∙415=1383.3  mm2/m -control

n =1383.3 314.2= 4.4,              take 5 ∅ 20/m         or   ∅ 20 @ 200 mm

# → Select ∅20@20cm/m

Minimum shrinkage Ash = 0.0018bh = 0.0018 × 1000 × 500 = 900 mm2/m

# → Select ∅14@30cm/m

- Design of heel:

Fig 4.37: Loads of Retaining Wall on Heel.

wu = 1.2 × 3× 18 + 0.5 × 25 + 1.6 × 5.0 = 87.8 KN/m2

Vu = 87.8 × 1 m × 2 = 175.6 KN

d = 500 - 75 -202= 415 mm

# 12ϕV=127.07 KN<Vu = 175.6 KN <ϕV=254.13 KN

# The thickness of 50 cm at the toe slab end is adequate enough.

# flexural  design

Mu = 87.8 × 222 =175.6 KN ∙ m

Take ϕ = 0.9 for flexure

Rn=Mu*106 ϕb*d2=175.6*1060.9*1000*4152=1.133 Mpa

m=Fy0.85*fc'=4200.85*24=20.59

ρ=1m*1-1-2*Rn*mFy

=120.59*1-1-2*1.133*20.59420

=2.78*10-3

Asreq=ρ*b*d=2.78*10-3*1000*415=1153.7 mm2/m

Check for As,min :

As,min=0.25  fc'fy ∙bwd ≥ 1.4fy bwd

As,min=0.25 24420 ∙1000∙415=1182 mm2/m

As,min=1.4420 ∙1000∙415=1383.3  mm2/m -control

As,req=1153.7<As,min=1383.3          , Take  As,min=1383.3

n =1383.3 314.2= 4.4,              take 5 ∅ 20/m         or   ∅ 20 @ 200 mm

# → Select ∅20@20cm/m

Minimum shrinkage Ash = 0.0018bh = 0.0018 × 1000 × 500 = 900 mm2/m

# → Select ∅14@30cm/m

Fig 4.38: Detailing of Retaining Wall.