Chapter 4

Structural Analysis & Design

Chapter 4

Structural Analysis & Design

Introduction

Factored Loads

Load Calculation

Design Of Topping

Design Of Rib (FF-R30)

Design Of Beam (FF-B130)

Design of Column  (C2-C3)

Design of Stair

Design of Isolated Footing  (F4)

Design of Basement Wall

Design of Shear Wall

Design of Strip Footing

- Introduction:

Concrete is the only major building material that can be delivered to the job site in a plastic state. This unique quality makes concrete desirable as a building material because it can be molded to virtually any form or shape.

Concrete used in most construction work is reinforced with steel. When concrete structure members must resist extreme tensile stresses, steel supplies the necessary strength. Steel is embedded in the concrete in the form of a mesh, or roughened or twisted bars. A bond forms between the steel and the concrete, and stresses can be transferred between both components.

In this project, all of design calculation for all structural members would be made upon the structural system which was chosen in the previous chapter.

So, in this project, there are five types of slabs: One way solid slab, one way ribbed slab, two ribbed slab, flat slab, and waffle slab. They would be analyzed and designed by using finite element method of design, with aid of a computer program called "ATIR- Soft ware " to find the internal forces, deflections and moments for ribbed slabs and by using the previous program and Etabs, Safe,And programs to find the internal forces.

- Factored loads:

# The factored loads on which the structural analysis and design is based for our project members, is determined as follows:

.

4-1 Design of Rib 11:

Fig(4-1):Moment Envelope (factored)   units:KN.m

Fig(4.2): Rib 11

Table ( 4.1) : Dead load calculation Topping

332

| Material | Quality Density KN/m3 |
| --- | --- |
| Tiles | 23 |
| mortar | 22 |
| Sand | 17 |
| Reinforced Concrete | 25 |
| Hollow Block | 10 |
| Plaster | 22 |
| Partitions 1.5KN/m2 |  |

Compressive strength of concrete fc´=30 Mpa

Yield strength of steel   ,  fy=420 Mpa

Live Load,LL=3 KN/m2

- Minimum thickness (deflection requirements):

There are two groups of ribs and beams

The thickness of the one-way ribbed slab with drop beams can be obtained according to ACI code, table 9.5 (a).

The maximum span length for one-end continuous (for ribs): l =640cm	then

h min=l18.5=64018.5=34.6 cm

The maximum span length for both-ends continuous (for ribs): l =510cm	then

h min=l21=51021=24.3cm

The minimum ribbed slab thickness will be  h min= 34.6cm

Take slab thickness    h=35cm >  h min= 34.6cm

h=35cm   ( 27cm HOLLOW BLOCK +  8cm TOPPING )

- 4-2  Topping Design:

Topping in One way ribbed slab can Be

considered as a strip of 1 meter width and span of hollow block length with both end fixed in the ribs.

Dead Load calculations:

| Dead Load from: | δ × γ × 1 | KN/m |
| --- | --- | --- |
| Tiles | 0.03×23 223 | 0.69 |
| Mortar | 0.02 ×22 | 0.44 |
| Coarse Sand | 0.07× 17 ×14 | 1.19 |
| Topping | 0.08×25 ×25 | 2 |
| Interior Partitions | 1.5 | 1.5 |
|  |  | 5.82 |

Live Load calculations:    3 ×1=3KN/m

Total Factored Load:  𝑤𝑢 =1.2*5.82+1.6*3=11.78KN/m

Mu = wul²12   = 11.78*0.4 ²12 =0.157KN.m/m of strip width

ØMn≥Mu  Strength condition, where   Ø=0.55                    for plain concrete

Mn

=0.42λfc´ Sm

where  Sm    for rectangular section of the slab:

Sm= bh² 6=1000*80² 6=1066666.67mm³

Mn

=0.42λfc´ Sm    =  0.42*1*30  * 1066666.67  × 10_₆ = 2.45 KN.m

ØMn=0.55*2.45=1.35 KN.m≫   Mu=0.157KN.m

NO Reinforcement is required by analysis. According to ACI 10.5.4., provide      ASmin     for   slabs  as shrinkage and temperature reinforcement.

ρ =0.0018

AS= ρbt =0.0018 * 1000 *80= 144mm²/m strip

Try bars Ø 8 with      AS =50.27mm²

Bar number  n = ASASØ 8 =14450.27= 2.87

Take 3 Ø 8/m  with  AS=150.8 mm²/m strip  or   Ø 8 @ 300 mm in both direction step(s) is smallest of :

- 3h = 3×80 = 240 mm       ͟       control

- 450 mm

- S= 380280fs-2.5Cc =38028023420- 2.5*20=330 mm                      but

		S  ≤     300280fs   = 300 28023420=300mm

Take Ø 8   @  200mm   in both directions   S=200mm < Smax=240 mm     ͟     ok

From practical consideration, the secondary reinforcement parallel to the ribs shall be placed in the slab and spaced at distances not more than half of the spacings between ribs (usually two bars upon each   40 cm   width block).

520mm= be

T= 80mm

H= 350

bw=120 mm

- Load Calculations for Rib 11:

T= hf= 80 mm                      bw= 120 mm         be = 520 mm          h= 350 mm

Dead Load calculations

| Dead Load from: | δ × γ × b | KN.m |
| --- | --- | --- |
| Tiles | 0.03*23*0.52= | 0.359 |
| Mortar | 0.02*22*0.52= | 0.2288 |
| Coarse Sand | 0.07*17*0.52= | 0.619 |
| Topping | 0.08*25*0.52= | 1.04 |
| RC Rib | 0.27*25*0.12= | 0.81 |
| Hollow Block | 0.27*10*0.4= | 1.08 |
| Plaster | 0.02*22*0.52= | 0.2288 |
| Interior Partitions | 1.5*0.52= | 0.78 |
|  |  | 5.146 |

Table(4-2):Dead Load Calculation of Rib (R 11)

Live Load calculations:

LL=3 * 0.52 = 1.56 KN/m

Dead Load /  rib: DL= 5.146  KN.m

Live Load /rib:  LL=1.56  KN.m

- Design of Rib 11 for positive moments:

1.Mu =+29.9KN.m

*Assume bar diameter Ø 14 for main positive reinforcement.

d = h – cover –  dstrip–  db2= 350 – 20 – 10 - 142 = 313 mm

*Check if    a > hf

Mnf = 0.85 fc´b hf  d-hf2

= 0.85 * 30*520 *80 313-802*10-6 = 289.6 KN.m

Mnf =289.6KN.m ≫ Mu Ø =  29.90.9=33.2              a<hf

*Design:

Rn = Mu bd2Ø =29.9*1060.9*520*3132=0.652 MPa

M=  fy 0.85 fc´=4200.85*30=16.47

ρ=1m 1-1-2Rn mfy = 116.47 1-1-2*0.652*16.47420 = 0.00157

As= ρ b d=0.00157 * 520 * 313 = 255.5mm²

*Cheak for Asmin :

ASmin = 0.25   fc´fy bw d≥      1.4fy bw d

ASmin=    0.25   30´420*120*313=122.46 mm2

ASmin=    1.4420*120*313=125.2 mm2  -----Control

As=255.5mm²  >Asmin=125.2  mm²      ----  OK

use  2 ∅14  with  As=3.08 cm2>AS req=2.56 cm2----  OK

*Cheak for strain (Tension-Controlled section- εs≥0.005):

a= As fy 0.85 fc b´= 3.08*4200.85*30*520= 9.76 mm

C =    aβ                      β=0.836

C = 9.760.836= 11.67 mm

εs = 0.003d-cc= 0.003313-11.6711.67=0.0775 >0.005 ----  OK

2.Mu =+10.8KN.m

*Assume bar diameter Ø 12 for main positive reinforcement:

d = h – cover –  dstrip–  db2= 350 – 20 – 10 - 122 = 314 mm

*Check if    a > hf

Mnf = 0.85 fc´b hf  d-hf2

= 0.85 * 30*520 *80 314-802*10-6 = 290.7 KN.m

Mnf =290.7KN.m ≫ Mu Ø =  10.80.9=12       a<hf

*Design:

Rn = Mu bd2Ø =10.8*1060.9*520*3142=0.234 MPa

M=  fy 0.85 fc´=4200.85*30=16.47

ρ=1m 1-1-2Rn mfy = 116.47 1-1-2*0.234*16.47420 = 0.00056

As= ρ b d=0.00056 * 520 * 314 = 91.4mm²

*Cheak for Asmin

ASmin = 0.25   fc´fy bw d≥      1.4fy bw d

ASmin=    0.25   30´420*120*314=122.8 mm2

ASmin=    1.4420*120*314=125.6 mm2       -----Control

As=91.4mm²  <Asmin=125.6  mm²

use  2 ∅10  with  As=1.57 cm2>AS min=1.26 cm2 ----  OK

*Cheak for strain (Tension-Controlled section- εs≥0.005):

a= As fy 0.85 fc b´= 1.57*4200.85*30*520= 4.97 mm

C =    aβ                      β=0.836

C = 4.970.836= 5.95 mm

εs = 0.003d-cc= 0.003315-5.955.95=0.156 >0.005 	 ----  OK

Usually, no Reinforcement less than 2∅10 can be used .So for other span with positive moment equal 8.6 KN.m, use  2∅10  for rib span .

- Design of Rib 11 for negative moments:

1.Mu = -24.8KN.m

*Assume bar diameter Ø 14 for main negative reinforcement:

d = h – cover –  dstrip–  db2= 350 – 20 – 10 - 142 = 313 mm

*Design:

Rn = Mu bd2Ø =24.8*1060.9*120*3132=2.34 MPa

M=  fy 0.85 fc´=4200.85*30=16.47

ρ=1m 1-1-2Rn mfy = 116.47 1-1-2*2.34*16.47420 = 0.00585

As= ρ b d=0.00585 * 120 * 313 = 219.86 mm²

*Cheak for Asmin

As=219.86mm²  >Asmin=125.2  mm²      ----  OK

use  2 ∅14  with  As=3.08 cm2>AS req=2.19 cm2---ok

*Cheak for strain (Tension-Controlled section- εs≥0.005):

a= As fy 0.85 fc b´= 3.08*4200.85*30*120= 42.26 mm

C =    aβ                      β=0.836

C = 42.260.836= 50.55mm

εs = 0.003d-cc= 0.003313-50.5550.55=0.0156 >0.005 	 ----  OK

2.Mu = -7.6KN.m

*Assume bar diameter Ø 12 for main negative reinforcement:

d = h – cover –  dstrip–  db2= 350 – 20 – 10 - 122 = 314 mm

*Design:

Rn = Mu bd2Ø =7.6*1060.9*120*3142=0.714 MPa

M=  fy 0.85 fc´=4200.85*30=16.47

ρ=1m 1-1-2Rn mfy = 116.47 1-1-2*0.714*16.47420 = 0.00172

As= ρ b d=0.00172 * 120 * 314 = 64.81mm²

*Cheek for Asmin

ASmin = 0.25   fc´fy bw d≥      1.4fy bw d

ASmin=    0.25   30´420*120*314=122.8 mm2

ASmin=    1.4420*120*314=125.6 mm2       -----Control

As=64.81mm²  <Asmin=125.6  mm²

use  2 ∅10  with  As=1.57 cm2>AS min=1.26 cm2 ----  OK

*Cheak for strain (Tension-Controlled section- εs≥0.005):

a= As fy 0.85 fc b´= 1.57*4200.85*30*120= 21.55 mm

C =    aβ                      β=0.836

C = 21.550.836= 25.78 mm

εs = 0.003d-cc= 0.003315-25.7825.78=0.0337 >0.005 	 ----  OK

- Design of Rib 11 for shear:

Shear Envelope (factored)   units:KN          Fig(4-3)

The maximum shear force at the distance d from the face of support Vu = 26.9 KN .

Shear strength, Vc , provided by concrete for the ribs may be taken 10% greater than that for beams. This is mainly due to the interaction between the slab and the closely spaced ribs.

Vn=Vu ∅

Vc = 1.1 * 16 λfc´bw d = 1.1 * 16*130 *120 *314 *10-³ = 37.84 KN

∅ Vc=0.75*37.84=28.38 KN

Vu = 26.9 KN< ∅ Vc=28.38 KN

Minimum Shear reinforcement is required expect for concrete joist construction . So, no shear reinforcement is provided .

4-3  Design of Beam B G 1 :

Fig(4.4): Beam B G 1

Dead load from two way ribbed slab

| Calculation |  | Parts of Flight |
| --- | --- | --- |
| 22*0.03*0.52*0.52 = 0.178 KN |  | Tiles |
| 22*0.02*0.52*0.52 = 0.119KN |  | Mortar |
| 16*0.07*0.52*0.52 = 0.303 KN |  | Sand |
| 25*0.08*0.52*0.52 = 0.541 KN |  | Topping |
| 25*0.27*0.12*(0.52+0.4) = 0.745 KN |  | Rib |
| 9*0.27*0.4*0.4 = 0.389 KN |  | Block |
| 22*0.02*0.52*0.52 = 0.119 KN |  | Plaster |
| 2.80 KN | Sum |

Table ( 4.3) : Dead load from two way ribbed slab

DL=2.8/(0.52*0.52)=10.36  KN/m3

LL=3  KN/m3

Dead Load from Two Way Ribbed Slab on Beam=2×3.37×10.36=69.83 KN/m

Live Load from Two Way Ribbed Slab on Beam=1×3.37×3=69.83 KN/m

Dead Load from One  Ribbed Slab on Beam=5.146 KN/m

Live Load from One Ribbed Slab on Beam=1.56 KN/m

Dead load of Beam:

| W(KN/m) | Material |
| --- | --- |
| 22×0.03×0.8=0.528 | Tiles |
| 22×0.02×0.8=0352 | Mortar |
| 16×0.07×0.8=0.896 | Sand |
| 22×0.02×(0.8+0.3)=0.484 | Plaster |
| 1.5×0.8=1.2 | Partitions 1.5KN/m2 |
| 3.46 | Total Dead Load ,KN |

Table ( 4.4) : Dead load of Beam

Service DD upon the Beam=3.46+0.8×0.5×25=13.46 KN/m

Service LL upon the Beam=3×0.8=2.4 KN/m

:Moment Envelope (factored)   units:KN.m   Fig(4-5)

- Design of Positive Moment :

MAX. Positive Moment=108 KN.m

= 108 KN .m

= =  = 120 KN .m

………….the larger is control

Rn =

Rn =  = 0.784 Mpa

=  = = 16.5

ρ = (1 - )

ρ = (1 - ) = 0.001896

A req = ρ * b * d = 0.001896*800 * 437.5 = 663.72 mm²

663.72<

A min = As

Use 3 Ф 25>> # of bar =  = 3

Then we select 3 bars  Ф25

Check for  yielding

Tension = compression

As * fy = 0.85 ** b * a

=186.25>108  singly reinforcement

Check for spacing between the bar

S =

S = 332.5    ≥ 25

- Design of Positive Moment :

MAX. Positive Moment=415.8 KN.m

= 415.8KN .m

= =  = 462 KN .m

………….the larger is control

Rn =

Rn =  = 3.02Mpa

=  = = 16.5

ρ = (1 - )

ρ = (1 - ) = 0.007677

A req = ρ * b * d = 0.007677*800 * 437.5 = 2686.95 mm²

2686.95>

Use 6 Ф 25>> # of bar =  = 6

Then we select 6 bars  Ф25

Check for  yielding

Tension = compression

As * fy = 0.85 ** b * a

=453.33>415.8 ….. singly reinforcement

Check for spacing between the bar

S =

S = 110    ≥ 25

MAX. Positive Moment=259.2 KN.m

= 259.2 KN .m

= =  = 288 KN .m

………….the larger is control

Rn =

Rn =  = 1.88 Mpa

=  = = 16.5

ρ = (1 - )

ρ = (1 - ) = 0.00465

A req = ρ * b * d = 0.00465*800 * 437.5 = 1629.23 mm²

1629.23>

Use 4 Ф 25>> # of bar =  = 4

Then we select 4 bars  Ф25

Check for  yielding

Tension = compression

As * fy = 0.85 ** b * a

=309.72 >259.2   singly reinforcement

Check for spacing between the bar

S =

S = 200    ≥ 25

MAX. Positive Moment=299.3 KN.m

= 299.3KN .m

= =  = 332.56 KN .m

………….the larger is control

Rn =

Rn =  = 2.17 Mpa

=  = = 16.5

ρ = (1 - )

ρ = (1 - ) = 0.00541

A req = ρ * b * d = 0.00541*800 * 437.5 = 1894.42 mm²

1894.42>

Use 4 Ф 25>> # of bar =  = 4

Then we select 4 bars  Ф25

Check for  yielding

Tension = compression

As * fy = 0.85 ** b * a

=309.72 >259.2   singly reinforcement

Check for spacing between the bar

S =

S = 200    ≥ 25

- Design of  Negative Moment :

MAX. Negative  Moment=442.9 KN.m

= 442.9KN .m

= =  = 492.11 KN .m

………….the larger is control

Rn =

Rn =  = 3.21Mpa

=  = = 16.5

ρ = (1 - )

ρ = (1 - ) = 0.008197

A req = ρ * b * d = 0.008197*800 * 437.5 = 2868.95 mm²

2868.95>

Use 6 Ф 25>> # of bar =  = 6

Then we select 6 bars  Ф2,

Check  for  yielding

Tension = compression

As * fy = 0.85 ** b * a

=453.33>442.9  singly reinforcement

Check for spacing between the bar

S =

S = 110    ≥ 25

MAX. Negative  Moment=249 KN.m

= 249 KN .m

= =  = 276.67 KN .m

………….the larger is control

Rn =

Rn =  = 1.81Mpa

=  = = 16.5

ρ = (1 - )

ρ = (1 - ) = 0.00447

A req = ρ * b * d = 0.00447*800 * 437.5 = 1563.27 mm²

1563.27>

Use 4 Ф 25>> # of bar =  = 4

Then we select 4 bars  Ф25

Check  for  yielding

Tension = compression

As * fy = 0.85 ** b * a

=309.71>249  singly reinforcement

Check for spacing between the bar

S =

S =200    ≥ 25

MAX. Negative  Moment=216.5 KN.m

= 216.5 KN .m

= =  = 240.56 KN .m

………….the larger is control

Rn =

Rn =  = 1.57Mpa

=  = = 16.5

ρ = (1 - )

ρ = (1 - ) = 0.00386

A req = ρ * b * d = 0.00386*800 * 437.5 = 1352.25 mm²

1352.25>

Use 3 Ф 25>> # of bar =  = 3

Then we select 3 bars  Ф25

Check  for  yielding

Tension = compression

As * fy = 0.85 ** b * a

=235.095>216.5  singly reinforcement

Check for spacing between the bar

S =

S =312.5    ≥ 25

- Design of shear

Shear Envelope (factored)   units:KN Fig(4-6)

Vu,max=514 KN

d= 500-40-10-12.5=437.5mm

Vc = bw * d

Vc=  *800 *437.5*10-3=319.5 KN

=  0.75 * 319.5 = 239.63KN> Vu,max=514 KN

ФVs,min=0.75 () * bw * d =0.75 () * 800 * 437.5=89. 9KN        -control-

ФVs,min =0.75 ()* bw * d =0.75 ()* 800 * 437.5=87.5kN

Ф Vc+ ФVs,min=329.53 KN > Vu,max

ФV~s =0.75()*bw * d=0.75 ()** 800 * 437.5=479.26kN

Ф Vc + ФV~s=239.63+479.26=718.89 KN< Vu,max=514 KN

-Stirrups are required-

Use 4 leg φ10

Av= 314.2 mm2

V~s =479.260.75=639.01 KN

Sreq=314.2*420*437.5639.01*103=90.35 mm<d2=218.75 mm,, S= 7.5 cm

4.7 Design Two Way Ribbed Slab

- Determination of Thickness for Two Way Ribbed Slab:

# Assume H =35cm

# Ib1=100*35312 =357291.66 cm4

# Ib2=80*35312 =285833.3 cm4

# Ib3=100*353 12=337291.66 cm4

# Ib4=80*35312 =285833.3 cm4

# Yc= 40*8*4+35*12*1640*8+35*12 =10.81 cm

# Ir=52*10.8133-40*2.8133+21*24.1933 =78219.6 cm4

# Is1=78219.6*(6002+80)52 = 571604.77 cm4

# Is2=78219.6*(5892+80)52 =  563331.54 cm4

# Is3=78219.6*(6002+100)52 = 601689.23 cm4

# Is4=78219.6*(5892+100)52 = 593416 cm4

# α1=285833.3563331.54 = 0.51

# α2=357291.66 601689.23 = 0.59

# α3=337291.66593416 0.57

# α4=285833.3 601689.23 = 0.48

# αfm=0.51+0.59+0.57+0.48 4 = 0.538 >0.2

# β =65.89 =1.02

# hmin= 6000*(0.8+4201400)36+1.02*5*(0.538-0.2) =174.9 mm

# h= 35 cm > hmin=17.49cm

- Load Calculation:

| Calculation |  | Parts of Flight |
| --- | --- | --- |
| 22*0.03*0.52*0.52 = 0.178 KN |  | Tiles |
| 22*0.02*0.52*0.52 = 0.119KN |  | Mortar |
| 16*0.07*0.52*0.52 = 0.303 KN |  | Sand |
| 25*0.08*0.52*0.52 = 0.541 KN |  | Topping |
| 25*0.27*0.12*(0.52+0.4) = 0.745 KN |  | Rib |
| 9*0.27*0.4*0.4 = 0.389 KN |  | Block |
| 22*0.02*0.52*0.52 = 0.119 KN |  | Plaster |
| 2.80 KN | Sum |

Table ( 4.3) : Dead load from two way ribbed slab

# Dead Load of slab:

# DL = 2.800.52*0.52 =10.36 KN/m2

# WD =1.2*10.36 =12.432 KN/m2

# LL=3  KN/m2

# WL =1.6*3=4.8 KN/m2

# W = 12.432+4.8 =17.232 KN/ m2

- Moments Calculations:

# From tables use Case (9 ):

# Ca. neg = 0.0626  , ,  Cb. neg = 0.0314

# Ma,neg = Ca,neg  Wu   ( La)2 = 0.0626 17.232  (5.89)2  0.52= 20.1 KN .m /  rib.

# Mb,neg = Cb,neg  Wu   ( Lb)2 = 0.0314 17.232  (6)2  0.52= 10.13 KN .m /  rib.

# Ca+,dL =0.0234  , ,  Cb+,dL =0.019

# M+a,d = Ca,dl  Wud   ( La)2 = 0.0234 12.432  (5.89)2  0.52= 5.25 KN .m /  rib.

# M+b,d = Cb,dl  Wud   ( Lb)2 = 0.019 12.432  (6)2  0.52= 4.42 KN .m /  rib.

# Ca+,LL =0.031  , ,  Cb+,LL =0.027

M+a,L = Ca,ll   Wul  ( La )2  = 0.031  4.8  (5.89)2  0.52= 2.68 KN .m /  rib.

M+b,L = Cb,ll   Wul  ( Lb )2  = 0.027  4.8  (6)2  0.52= 2.43 KN .m /  rib.

Ma positive = 5.25 + 2.68= 7.93KN .m / rib.

Mb positive = 4.42+2.43 = 6.85KN .m / rib.

Negative moment at discontinuous edges = 1/3 positive.

Mb neg = (1/3)  6.85 = 2.28 KN.m / rib.

- Design of positive moments:

Design of Positive Moment :- (Ma=7.39KN.m)

Assume bar diameter ø 14 for main positive reinforcement

d =h- cover - dstirrups-db2=350-20-8-142=315mm

Rn=Mu∅bd2=7.39×1060.9×520×3152=0.16Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.16420=0.000382

As, req = ρ.b.d = 0.000382×520×315= 62.65 mm2

Check for As min:-

A=ACI-318 (10.5.1)

A=

A=

A=…..controls

Asreq = 62.65mm2 <Asmin= 126 mm2….OK

Use 2 ø10, As, provided= 157.08 mm2>As, required= 126mm2…  Ok

Check for strain:-

a =As.fy0.85b fc'=157.08×4200.85×520×24=6.22 mm

x=aβ1=6.220.85=7.32mm

εs=0.003d-xx=0.003315-7.327.32=0.126>0.005                0k

Design of Positive Moment:- (Mb=6.85KN.m)

d =h- cover - dstirrups-db2=350-20-8-142=315 mm

Rn=Mu∅bd2=6.85×1060.9×520×3152=0.148Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.148420=0.000354

As, req = ρ.b.d = 0.000354 ×520×315 = 57.99 mm2

Check for As min:-

A=ACI-318 (10.5.1)

A=

A=

A=…..controls

Asreq = 57.99mm2 <Asmin= 126 mm2 …..OK

Use 2 ø10, As, provided= 157.08 mm2>As, required= 126mm2…  Ok

Check for strain:-

a =As.fy0.85b fc'=157.08×4200.85×520×24=6.22 mm

x=aβ1=6.220.85=7.32mm

εs=0.003d-xx=0.003315-7.327.32=0.126>0.005                0k

- Design of negative moments:

Design of Negative Moment :- (Ma=-20.1KN.m)

Assume bar diameter ø 12 for negative reinforcement

d =h- cover - dstirrups-db2=350-20-8-122=316 mm

Rn=Mu∅bd2=20.1×1060.9×120×3162=1.86 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×1.86420=0.0047

As, req = ρ.b.d = 0.0047 ×120×316 =178.22 mm2

Check for As min:-

A=ACI-318 (10.5.1)

A=

A=

A=….controls

Asreq = 178.22 > Asmin= 126.4mm2 ….OK

Use 2 ø12, As, provided= 226.2 mm2>As, required= 178.22mm2…  Ok

Check for strain:-

a =As.fy0.85b fc'=226.2×4200.85×120×24=38.81 mm

x=aβ1=38.810.85=45.66 mm

εs=0.003d-xx=0.003316-45.6645.66=0.0178>0.005                0k

- Design of Negative Moment :-

(Mb=-10.13KN.m)

Assume bar diameter ø 12 for negative reinforcement

d =h- cover - dstirrups-db2=350-20-8-122=316 mm

Rn=Mu∅bd2=10.13×1060.9×120×3162=0.94 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.94420=0.00229

As, req = ρ.b.d = 0.00229 ×120×316 = 86.92 mm2

Check for As min:-

A=ACI-318 (10.5.1)

A=

A=

A=….controls

Asreq = 86.92 < Asmin= 126.4mm2 …..OK

Use 2 ø12, As, provided= 226.2 mm2>As, required= 126.4mm2…  Ok

Check for strain:-

a =As.fy0.85b fc'=226.2×4200.85×120×24=38.81 mm

x=aβ1=38.810.85=45.66 mm

εs=0.003d-xx=0.003316-45.6645.66=0.0178>0.005                0k

- Design of shear for rib:

Maximum shear coefficient in short direction as in case (9) Wa @ m =0.98

Wa = 0.686

The total load on the panel =5.89*6*17.232= 608.98 KN

The load per rib at the face of long beam =0.686*608.98*0.52/(2*6)= 18.1 KN

Wb = 0.306

The total load on the panel =5.89*6*17.232= 608.98 KN

The load / rib at the face of long beam =0.306*608.98*0.52/(2*5.89) = 8.23KN

Vud = Vuface - Wu*bf*d = 18.1 – 17.232*0.52*0.316= 15.27 KN

The shear strength of one rib:

Vc =1.16fc'bwd=1.1624 ×120×316×10-3=34.05 KN

øVc =0.75×34.05 =25.54KN

0.5 ø Vc =0.5×25.54=12.77KN

0.5 ø Vc< Vud< ø Vc

12.77<15.27<25.54

Minimum shear reinforcement is required except for joist construction.

Vud = 17.232*0.52*(3.8 - 0.316) = 31.3 KN

# øVc =25.54 < Vud =31.3KN

Ф Vsmin≥ 0.75 () * bw * d =0 .75* ()*120*316*10-3 = 9.48 KN ….Controls

Ф Vsmin   ≥ 0.75 () * bw * d = 0.75*() * 120* 316*10-3 = 8.7KN

Ф (Vc+ Vsmin)= 35.021>Vu= 31.3 > ФVc = 25.54

Vs'= 13*bw*d  = 61.9 KN

Vs= VuФ-Vc = 12.92 KN

Vsmin =11.4 <Vs =12.92 < Vs’ =55.85 ………Case IV

Use 2 leg Φ 8

Av min =100 mm2

Av mins= vs dfyt

s= Avmin vs  dfyt

S=926.5mm

smax ≤ d2=2852= 142.5 mm                 or smax ≤600 mm

Use 4 leg Φ 8 @125 mm

4.7 Design of column (C36)

Fig.(37) :Place Of Column (C36) within the second basement floor.

Load Calculation for Column

| Column | Column Dimensions |  |  |
| --- | --- | --- | --- |
| Col. C36 | 60cm* 45cm | 24Mpa | 420Mpa |

- Load Calculation:

| Pu(KN) |  |  | a( ) | ,required |
| --- | --- | --- | --- | --- |
| 1941.6 | 0.02 |  | 222.3 |  |

- Selecting longitudinal bars:

Take  18Ф 25 As,provided = 8834.4< As,req = 8737.6

=  =

| Ф |  |  |
| --- | --- | --- |
| 0.65 | 8737.6 |  |

- Design of Ties:

- Use ties Ф10 with spacing of ties shall not exceed the smallest of

1. 48 * ds = 48 * 10 = 480mm

2. 16 * db = 16 * 25 = 400 mm - control

3. the least dimension of the column = 700 mm

Use ties Ф10 @ 200mm

| ds(mm) | db(mm) | S(mm) |
| --- | --- | --- |
| Ф10 | Ф25 | 200 |

- Check for code requirements:

1. Clear Spacing ==100 mm > 40mm>1.5db=1.5*25=37.5mm  - OK

2. 0.01< = 0.017 <0.08  - OK

3. Number of bars 18>4 for rectangular section – OK

4. Minimum tie diameter ds =Ф10 for db = Ф25 bars – OK

5. Arrangement of ties 100 mm<150mm – OK

| Clear Spacing | No. of bars |  | ds (mm) | db (mm) |
| --- | --- | --- | --- | --- |
| 100 mm | 18 | 0.017 | Ф10 | Ф25 |

- Check Slenderness Effect:

Lu: Actual unsupported (un braced) length.

K: effective length factor (K= 1 for braced frame).

R: radius of gyration = 0.3 h =

Lu = 3.5 m

M1/M2 =1   (Braced frame with M,min)

K=1 , According to ACI 318-02 The effective length factor, k, shall be permitted to be taken as 1.0.

…..short column.

Short column in both direction

| Lu (m) | M1/M2 | K |  |
| --- | --- | --- | --- |
| 3.5 | 1.0 | 1.0 | 15.55 |

Fig. (38):Section of Column (C2)