STEPS OF OPERATION:

1. Turn on Power supply for all of the system includes aurduino and the main system

2. Choose which measurement test you would like to perform (Vascular access blood flow and recirculation ratio or Hematocrite and plasma volume) by moving the switch to the right or to the left

For hematocrite measurement:

To choose hematocrite measurement move the switch to the right

1. Wait a moment until the system appears “Push your finger” on the LCD screen of the system

2. Then Push your finger into probe quickly

3. The system will appears “please wait” then the processing will start

4. Wait 4 minutes

5 .After that the system orders to enter your weight

6 .After that Enter your gender by moving the switch to the right or to the left to determine the gender

Then the system will give you hematocrite percentage and plasma volume in ml.

For vascular access blood flow monitoring:

1. To choose vascular access blood flow monitoring move the switch to the left side.

2. Set the sensors on artery tube and vein tube of the patient.

3. Enter ultrafiltration rate in ml/min.

4. Enter temperature of artery in normal positions of the tubes by pressing the enter button.

5. Reverse the position of the tubes

6. Enter temperature of vein in reverse position of the tubes by pressing the enter button.

7. Enter prestilatic pump flow rate in reverse position of the tubes

Then the system gives access flow rate and recirculation ratio.