4

Chapter Four

Structural Analysis and Design

4.1 Introduction.

4.2 Design method and requirements.

4.3 Check of minimum thicknesses of structural members.

4.4 Design of topping.

4.5 (Rib 1 or 3 , BF) Calculations.

4.6 (Beam 2, BF) Design of Beam.

4.1 Introduction:

Many structures are built of reinforced concrete: bridges, buildings, retaining walls, tunnels, and others.

Reinforced concrete is logical union of two materials: plain concrete, which possesses highcompressive strength but little tensile strength, and steel bars embedded in the concrete, whichcan provide the needed strength in tension.

Plain concrete is made by mixing cement, fine aggregate, coarse aggregate, water, and frequentlyadmixtures.

Understanding of reinforced concrete behavior is still far from complete, building codes andspecifications that give design procedures are continually changing to reflect latest knowledge.

Structural concrete can be classified into:

Lightweight concrete with unit weight from about 1350 to 1850 kg/m3.

Normal weight concrete with unit weight from about 1800 to 2400 kg/m3.

Heavyweight concrete with unit weight from about 3200 to 5600 kg/m3.

4.2 Design method and requirements:

The design strength provided by a member is calculated in accordance with the requirements andassumptions of ACI_code (318_14).

Strength design method:

In ultimate strength design method, the service loads are increased by factors to obtain the loadat which failure is considered to be occurring.

This load called factored load or factored service load. The structure or structural element is thenproportioned such that the strength is reached when factored load is acting.

The computation ofthis strength takes into account the nonlinear stress-strain behavior of concrete.

The strength design method is expressed by the following,

Strength provided ≥ strength required to carry factored loads.

NOTE:

The statically calculation and the key plans dependent on the architectural plans.

Code : ACI 2014

UBC

Material :

Concrete: B350….  ( fc'35*0.8 28MPa ) .

Reinforcement steel : The specified yield strength of the reinforcement

{fy = 420 N/mm²(MPa)}

Mild steel : A-36

Connection Type : Weld , Bolts

Factored loads:

The factored loads for members in our project are determined by:

Wu = 1.2 DL + 1.6 SL ACI-code-318-14(9.2.1).

4.3 Check of  minimum thickness of  structural member :

TABLE 9.5(a) — MINIMUM THICKNESS OF NONPRESTRESSED BEAMS OR ONE-WAY

SLABS UNLESS DEFLECTIONS ARE CALCULATED. (ACI 318M-14)

For rib :

hmin=L/18.5=6.13/18.5=33.14 cm
select : 35cm thickness with 27 cm block and 8 toping .

For beam :

hminfor(one end continuous )=L/18.5=6.93/18.5=37.46 cm

select h=(27+8)=35 cm for rib slab with drop beam h=50 cm (deflection control).

# 4.4  Design of topping:

Statically system for topping :

Consider the topping as strip of (1m) width, and span of mold length with both end fixed in the ribs

Fig 4.1: topping load and moment diagram.

For the topping , the total dead load to be used in the analysis and design is calculated as follows:

Table (4 – 2) Dead load calculation for topping

| Calculation |  |  | Quality Density KN/m3 |  | Partsof Rib | No. |
| --- | --- | --- | --- | --- | --- | --- |
| 0.08×25×1 |  |  | 25 |  | Reinforced Concrete Topping | 1 |
| 0.07×16×1 |  |  | 16 |  | Sand | 2 |
| 0.02×22×1 |  |  | 22 |  | Mortar | 3 |
| 0.03×23×1 |  |  | 23 |  | Tile | 4 |
| 0×1 |  |  | 0 |  | Partition | 5 |
| KN/m | 4.25 | ∑ = |  |

Nominal total dead load = 4.25 KN/m2.

Nominal total live load = 5KN/m2.

Design of topping for ribbed slab as a plain concrete section :-

Fig. (4-2) : Topping of one way rib slab

qu = 1.2×D+1.6×L

13.1KN/m.   (total factored load)

Max Mu= Wu*l212= 0.175 KN.m

Max Vu= Wu*l2=2.62 KN.m

Design of shear:

Used  = 420 MPa&= 24MPa

* Vc =

No shear reinforcement is required.

Design of Moment:

∅Mn=0.55*0.42*28*1000*802/6=1.3 KN.m

∅Mn=1.3 KN.m>Mu=0.175 KN.m

No structural reinforcement is required.

The strength of plain concrete section > loaded section.

The plain concrete section is safe; however, minimum reinforcement for shrinkage and temperature to control the cracks should be used.

ρ=0.0018     ,ACI-318-11

As= ρ*b*h=0.0018*1000*80=144 mm2.

∴Use  8 @15 cm

4.5) Design Rib :

Design of Rib

R-12

R12

Fig. (4-3) :Ground floor Ribs

R-12

Fig. (4-4) :Rib (12) location in ground floor

For the one-way ribbed slabs, the total dead load to be used in the analysis and design is calculated as follows:

Fig. (4-5) : One way rib slab

Calculation of the total dead load for one way rib slab is shown in the following table:

Table (4 – 3) Calculation of the total dead load for one way rib slab.

| Calculation |  |  |  | Quality Density KN/m3 | Material | No. |
| --- | --- | --- | --- | --- | --- | --- |
| 0.52×0.08×25 = 1.04 |  |  |  | 25 | Topping | 1 |
| 0.27×0.12×25 = 0.81 |  |  |  | 25 | Rib | 2 |
| 0.52×0.07×16 = 0.5824 |  |  |  | 16 | Sand | 3 |
| 0.52×0.02×22 =0.2288 |  |  |  | 22 | Mortar | 4 |
| 0.52×0.03×23 =0.3588 |  |  |  | 23 | Tile | 5 |
| 0.52×0.02×22 =0.2288 |  |  |  | 22 | Plaster | 6 |
| 0.4×0.27×15 = 1.62 |  |  |  | 15 | Block | 7 |
| 0.0 |  |  |  | 0.0 | Partitions | 8 |
| KN/m | 4.87 | ∑ = |

By using ATIR program we get the envelope moment and shear force diagram as the follows:-

G e o m e t r y Units:meter,cm

A A 1 1 A A 2 2 A A 3 3 A A 4 5 4 0.4 5.55 0.8 0.8 5.3 0.8 0.8 5.3 0.8 0.8 5.56 0.4 6.15 6.1 6.1 6.16

12. 35. 8. 52. A A

Figure (4.6) Geometry of rib R12

L o a d i n g

load group no. 1 Dead load - Service Units:kN,meter 4.87 6.15 6.1 6.1 6.16 4.87 6.15 6.1 6.1 6.16 4.87 6.15 6.1 6.1 6.16 4.87 6.15 6.1 6.1 6.16

Live load - Service Load factors: 1.20,1.20/1.60,0.00 2.60 6.15 6.1 6.1 6.16 2.60 6.15 6.1 6.1 6.16 2.60 6.15 6.1 6.1 6.16 2.60 6.15 6.1 6.1 6.16

Figure (4.7) Loading of rib R12(KN/m).

M o m e n t / S h e a r E n v e l o p e (Factored) Units:kN,meter

Moments: spans 1 to 4 9.4 -42.4 -35.7 -35.1 -32. -26. -26. -42.5 -35.2 -35.8 9.4 32.6 20.3 20.2 32.7 1.78 2.79 2.15 2.16 2.8 1.77 1.05 1.21 0.81 0.81 1.21 1.05 2.46 3.69 3.36 2.74 2.74 3.36 3.7 2.46

Shear 18.4 29.4 25.9 32.6 -32.6 -25.9 -29.4 -18.5 25.5 -37.7 34.5 -31. 31. -34.5 37.7 -25.6

Reactions Factored DeadR LiveR Max R Min R 14.14 11.41 25.55 12.79 Service DeadR LiveR Max R Min R 11.78 7.13 18.91 10.94 40.97 31.2 72.17 52.74 34.14 19.5 53.64 41.5 32.95 29.02 61.97 41.9 27.46 18.13 45.6 33.05 41.01 31.23 72.24 52.77 34.18 19.52 53.69 41.53 14.16 11.42 25.59 12.82 11.8 7.14 18.94 10.97

Figure (4.8) Moment and Shear Envelop for rib R12

- Design of shear for rib R12

Categories for shear design:

Vu = 32.6KN

Use Ф8 with two legs

d=350-20-8-7=315

- Region I :

= 27.5KN< Vu = 32.6KN

Case III minimum Shear reinforcement required .So,

Use Φ 8,2 leg

Av = 100.53 mm2.

Vs = vu∅- vc=32.60.75-33.33=10.14 KN

Sreq= Avfyt dvs=100.53*420*31510.14*1000=1311.65 mm

sreq ≤  smax

smax ≤ d2 or smax ≤600 mm

d2=3152= 158 mm (control)

Use Φ8, @ 15 cm   (2Legs).

- Design of Positive Moment:

## Effective Flange width ( )                                 , ACI-318-14

For T- section is the smallest of the following:

= (2200) / 4 = 550 mm

= 120 + 16 (80) = 1400mm

= 520 mm.................. control

»Use Mu max positive for span 1 = 32.6 kN.m

» Determine whether the rib will act as rectangular or T – section:

For   hf = 0.08 m

~ Assume bar diameter Φ14 for main positive reinforcement.

d = 350 – 20 – 8–7= 315mm

*Mn =0.9*0.85* ƒc'*b*hf* (d- hf/2)

= 0.9*0.85*28*0.52*0.08*(0.315-0.08/2)=245 KN.m

*Mn= 245 KN.m>> Mu = 32.6 KN.m

The section will be designed as a rectangular section with = 520mm

A=>=A=ACI-318 -14

A=

A=~ control

A = 0.00169(520) (315) = 276.82 mm2> A = 126 mm2

# of bars = As / As bar = 276.82/154= 1.798      * Note  AΦ14= 154mm2

Select bottom bars   2Φ14

Total As (provide) =308 mm2>276.82mm2

* Check Strain for the magnitude of under strength factor Φ:

Tension = Compression

x=a0.85=10.450.85=12.29mm

Ok…….

»Use Mu max positive for span 2 = 20.3kN.m

» Determine whether the rib will act as rectangular or T – section:

For   hf = 0.08 m

~ Assume bar diameter Φ14 for main positive reinforcement.

d = 350 – 20 – 8–7= 315mm

*Mn =0.9*0.85* ƒc'*b*hf* (d- hf/2)

= 0.9*0.85*28*0.52*0.08*(0.315-0.08/2)=245 KN.m

*Mn= 245 KN.m>> Mu = 20.3 KN.m

The section will be designed as a rectangular section with = 520mm

A=>=A=ACI-318 -14

A=

A=~ control

A = 0.00106(520) (315) = 173.63mm2> A = 126 mm2

# of bars = As / As bar = 173.63/113= 1.53      * Note  AΦ12= 113mm2

Select bottom bars   2Φ12

Total As (provide) =226.19 mm2>173.63mm2

* Check Strain for the magnitude of under strength factor Φ:

Tension = Compression

x=a0.85=7.680.85=9mm

Ok…….

»Use Mu max positive for span 3 = 20.2kN.m =”span 2 Mu max=20.3KN.m”

» Determine whether the rib will act as rectangular or T – section:

For   hf = 0.08 m

~ Assume bar diameter Φ14 for main positive reinforcement.

d = 350 – 20 – 8–7= 315mm

*Mn =0.9*0.85* ƒc'*b*hf* (d- hf/2)

= 0.9*0.85*28*0.52*0.08*(0.315-0.08/2)=245 KN.m

*Mn= 245 KN.m>> Mu = 20.3 KN.m

The section will be designed as a rectangular section with = 520mm

A=>=A=ACI-318 -14

A=

A=~ control

A = 0.00106(520) (315) = 173.63 mm2> A = 126 mm2

# of bars = As / As bar = 173.63/113= 1.53      * Note  AΦ12= 113mm2

Select bottom bars   2Φ12

Total As (provide) =226.19 mm2>173.63mm2

* Check Strain for the magnitude of under strength factor Φ:

Tension = Compression

x=a0.85=7.680.85=9mm

Ok…….

»Use Mu max positive for span 4 = 32.7 kN.m =”span 1 Mu max=32.6 KN.m ”

» Determine whether the rib will act as rectangular or T – section:

For   hf = 0.08 m

~ Assume bar diameter Φ14 for main positive reinforcement.

d = 350 – 20 – 8–7= 315mm

*Mn =0.9*0.85* ƒc'*b*hf* (d- hf/2)

= 0.9*0.85*28*0.52*0.08*(0.315-0.08/2)=245 KN.m

*Mn= 245 KN.m>> Mu = 32.6 KN.m

The section will be designed as a rectangular section with = 520mm

A=>=A=ACI-318 -14

A=

A=~ control

A = 0.00169(520) (315) = 276.82 mm2> A = 126 mm2

# of bars = As / As bar = 276.82/154= 1.798      * Note  AΦ14= 154mm2

Select bottom bars   2Φ14

Total As (provide) =308 mm2>276.82mm2

* Check Strain for the magnitude of under strength factor Φ:

Tension = Compression

x=a0.85=10.450.85=12.29mm

Ok…….

- Design of Max Negative Moment for (Rib):

## »The maximum negative moment from spans with support (2) =”support(4)” is

Mu = - 35.8 kN.m

Mn = 35.8/ 0.9 = 39.78 kN.m

~ Assume bar diameter Φ16for main negative reinforcement.

d = 350 – 20 – 8–8= 314mm

*Mn =0.9*0.85* ƒc'*b*hf* (d- hf/2)

= 0.9*0.85*28*0.52*0.27*(0.314-0.27/2)=538.32KN.m

*Mn= 538.32KN.m>> Mu = 35.8 KN.m

The section will be designed as a rectangular section with = 520mm

A=>=A=ACI-318 -14

A=

A=~ control

A = 0.00189(520) (314) = 308.6 mm2> A = 126 mm2

# of bars = As / As bar = 308.6/201.1= 1.53      * Note  AΦ16= 201.1mm2

Select top bars   2Φ16

Total As (provide) =402.12 mm2>308.6mm2

* Check Strain for the magnitude of under strength factor Φ:

Tension = Compression

x=a0.85=13.650.85=16.06mm

Ok…….

## »The maximum negative moment from spans with support (3) is

Mu = 26 kN.m

Mn = 26/ 0.9 = 28.89kN.m

~ Assume bar diameter Φ14for main negative reinforcement.

d = 350 – 20 – 8–7= 315mm

*Mn =0.9*0.85* ƒc'*b*hf* (d- hf/2)

= 0.9*0.85*28*0.52*0.27*(0.315-0.27/2)=541.33 KN.m

*Mn= 541.33KN.m>> Mu =26  KN.m

The section will be designed as a rectangular section with = 520mm

A=>=A=ACI-318 -14

A=

A=~ control

A = 0.00135(520) (315) = 221.13mm2> A = 126 mm2

# of bars = As / As bar = 221.13/154= 1.44      * Note  AΦ14= 154mm2

Select top bars   2Φ14

Total As (provide) =308 mm2>126mm2

* Check Strain for the magnitude of under strength factor Φ:

Tension = Compression

x=a0.85=10.450.85=12.29mm

Ok…….

Figure 9 beam detaling

Design of Beam BG-26

BG-26

Figure(4.10)Beam location in ground floor slab

Load calculations for Beam:

The distributed Dead and Live loads acting upon the Beam BG-26 can be defined from the support reactions of the rib R12 AND R13.

Reactions Factored DeadR LiveR Max R Min R 14.14 11.41 25.55 12.79 Service DeadR LiveR Max R Min R 11.78 7.13 18.91 10.94 40.97 31.2 72.17 52.74 34.14 19.5 53.64 41.5 32.95 29.02 61.97 41.9 27.46 18.13 45.6 33.05 41.01 31.23 72.24 52.77 34.18 19.52 53.69 41.53 14.16 11.42 25.59 12.82 11.8 7.14 18.94 10.97

Figure (4.11) Reaction of rib R12

- The support reaction (service) from Dead loads of Rib (R12) upon beam (BG-26) is (34.14KN). The distributed Dead load  from Rib ( R12 ) on Beam(BG-26):

DL from Rib=34.140.52=65.65 KN/m

- The support reaction (service) from Live loads of Rib (R12) upon beam (BG-26) is (19.52KN). The distributed Live load  from Rib ( RG12 ) on Beam ( BG-26):

LLfrom Rib= 19.520.52=37.5 KN/m

By using ATIR program we get the envelope moment and shear diagram as the follows:-

G e o m e t r y Units:meter,cm

A A 1 1 A A 2 2 A A 3 3 A A 4 5 4 0.25 5.95 0.25 0.25 6.51 0.6 0.6 4.24 0.6 0.6 6.51 0.25 6.2 6.93 4.84 6.93

100. 50. A A

Figure (4.12) Geometry of Beam BG-26

L o a d i n g

load group no. 1 Dead load - Service Units:kN,meter 12.5 65.5 6.2 6.93 4.59 0.25 6.93 12.5 65.5 6.2 6.93 4.59 0.25 6.93 12.5 65.5 63.4 6.2 6.93 4.59 0.25 6.93 12.5 63.4 6.2 6.93 4.59 0.25 6.93

Live load - Service Load factors: 1.20,1.20/1.60,0.00 37.5 6.2 6.93 4.59 0.25 6.93 37.5 6.2 6.93 4.59 0.25 6.93 37.5 36.7 6.2 6.93 4.59 0.25 6.93 36.7 6.2 6.93 4.59 0.25 6.93

Figure (4.13) Loading of Beam BG-26 (KN/m).

57

M o m e n t / S h e a r E n v e l o p e (Factored) Units:kN,meter

Moments: spans 1 to 4 46.7 -751.9 -678.7 -678.4 -474.8 -366.1 -326.3 -672.6 -494.5 -539. 53.5 477.1 442.6 101.9 636.1 2.09 2.13 1.15 1.44 1.22 1.29 0.77 1. 1.55 1.1 2.48 3.72 3.81 3.12 2.18 2.66 4.16 2.77

Shear 296.1 508.4 271.6 505.3 -510.7 -404.3 -355. -352.5 382.8 -597.4 595.2 -517.9 385.2 -467.7 616.1 -437.1

Reactions Factored DeadR LiveR Max R Min R 217.96 164.88 382.84 192.78 Service DeadR LiveR Max R Min R 181.63 103.05 284.68 165.9 717.78 474.85 1192.63 929.2 598.15 296.78 894.93 730.29 487.77 415.41 903.18 543.08 406.48 259.63 666.11 441.04 629.27 454.57 1083.84 731.88 524.39 284.1 808.5 588.52 259.77 177.34 437.11 249.93 216.47 110.84 327.31 210.32

Figure (4.14) Moment and Shear envelop for Beam BG-26

Assume bar diameter Ф25 for main reinforcement.

Selected drop beam

- Design of shear for Beam :

ACI – 318 – Categories for shear design:

Vu critical = 510 KN

Vc = 16fc'bw d

Vc =  1628*1000*435.5

Vc= 384.1KN.

Φ Vc= 0.75*384.1=288.1 KN

vs,min= 116fc'bw d

vs,min= 11628*1000*435.5

vs,min=144 kn

vs,min= 13bw d

vs,min= 13*1000*435.5

vs,min= 145.2KN

Case III minimum Shear reinforcement required .So,

∅(vc+ vs,min⁡)<vu ≤3 ∅*vc

0.75(384.1+145)=396.83<510<3* 0.75*384.1=864.23

So, shear reinforcement are required.

Use 4 leg Φ 12.

Av =452.4 mm2.

Vs = Vn – VC = 5100.75 – 384.1= 295.9KN

S= Avfyt dvs=452.4*420*435.5384.1*1000=215.43 mm

smax ≤ d2 or smax ≤600 mm

S max=d2=435.52= 217.75 mm

Select 4 leg Φ12, @ 150 mm     (2 Legs)

- Design  of Beam  of negative moment :

- Mu =-678.7 KN.m at support (2).

Mn= Mu/0.9

= 678.7/0.9 =754.11  KN.m

~ Assume bar diameter Φ25for main negative reinforcement.

d = 500-40-12-12.5=435.5 mm

m =  = = 17.65

A=

A=~ control

A = 0.0104 (1000) (435.5) =4529.2mm2> A = 1451.67mm2

# of bars = As / As bar = 4529.2/491 = 9.22                    * Note  AΦ25 = 491 mm2

Select bar 10 Φ 25

Total As (provide) = 4910mm2>4529.2 mm2

* Check strain for the magnitude of under strength factor Φ:

Tension = Compression

X=a0.85=86.650.85=101.94mm

## Ok…….

*Check for bar distance:

- Mu =-366.1 KN.m  at support (3).

Mn= Mu/0.9

= 366.1/0.9  = 406.78 KN.m

~ Assume bar diameter Φ25for main negative reinforcement.

d = 500-40-12-12.5=435.5 mm

m =  = = 17.65

A=

A=~ control

A = 0.00535 (1000) (435.5) =2330mm2> A = 1451mm2

# of bars = As / As bar = 2330/491 = 4.75                    * Note  AΦ25 = 491 mm2

Select bar 5 Φ 25

Total As (provide) = 2455mm2>2330 mm2

* Check strain for the magnitude of under strength factor Φ:

Tension = Compression

X=a0.85=43.320.85=50.96mm

## Ok…….

*Check for bar distance:

- Mu =-539 KN.m  at support (4).

Mn= Mu/0.9

= 539/0.9  =598.89 KN.m

~Assume bar diameter Φ25for main negative reinforcement.

d = 500-40-12-12.5=435.5 mm

m =  = = 17.65

A=

A=~ control

A = 0.0081 (1000) (435.5) =3527.6 mm2> A = 1451mm2

# of bars = As / As bar = 3527.6/491 = 7.18                    * Note  AΦ25 = 491 mm2

Select bar 8 Φ 25

Total As (provide) = 3928mm2>3527.6 mm2

* Check strain for the magnitude of under strength factor Φ:

Tension = Compression

X=a0.85=69.320.85=81.55mm

## Ok…….

*Check for bar distance:

- Design  of positive moment :

*Take Mu = 477.1 KN.m at span (1&2).

~ Assume bar diameter Φ25for main negative reinforcement.

d = 500-40-12-12.5=435.5 mm

m =  = = 17.65

A=

A=~ control

A = 0.0071 (1000) (435.5) =3092.1mm2> A = 1451.67mm2

# of bars = As / As bar = 3092.1/491 = 6.3                    * Note  AΦ25 = 491 mm2

Select bar 7 Φ 25

Total As (provide) = 3437mm2>3092.1 mm2

* Check strain for the magnitude of under strength factor Φ:

Tension = Compression

X=a0.85=60.650.85=71.35mm

## Ok…….

*Check for bar distance:

*Take Mu = 102 KN.m at span (3).

~ Assume bar diameter Φ25for main negative reinforcement.

d = 500-40-12-12.5=435.5 mm

m =  = = 17.65

A=

A=~ control

A = 0.00145 (1000) (435.5) =631.5mm2< A = 1451.67mm2

# of bars = As / As bar = 1451/314 = 2.96                    * Note  AΦ25 = 314 mm2

Select bar 5 Φ 20

Total As (provide) = 1570mm2>1451.67 mm2

* Check strain for the magnitude of under strength factor Φ:

Tension = Compression

X=a0.85=27.710.85=32.6mm

## Ok…….

*Check for bar distance:

*Take Mu = 636.1KN.m at span (4).

~ Assume bar diameter Φ25for main negative reinforcement.

d = 500-40-12-12.5=435.5 mm

m =  = = 17.65

A=

A=~ control

65

A = 0.00971 (1000) (435.5) =4228.71mm2> A = 1451.67mm2

# of bars = As / As bar = 4228.71/491 = 8.61                    * Note  AΦ25 = 491 mm2

Select bar 9 Φ 25

Total As (provide) = 4419mm2>4228.71 mm2

* Check strain for the magnitude of under strength factor Φ:

Tension = Compression

X=a0.85=77.980.85=91.7mm

## Ok…….

*Check for bar distance:

Figure (4.15) reinforcement for beam BG-26

4.3.8 Design of column G2

# fc'=28 MPa

# Fy=420 MPa

The Column is an interior one.

DL = 1200KN

LL = 450KN

Pu = 1.2DL+1.6LL

Pu = 1.2(1200)+1.6(450)

Pu = 2160KN

- Check for slenderness:-

# k lnr≤34-12(M1M2)≤40

# *M1M2=1braced frame with Mmin

# K=1 for column in non-sway frames.

# k lnr≤34-12=22≤40

# k lnrx= 1*3.70.3*0.6=20.55<22 Column is short  about x-axis

# k lnry = 1*3.70.3*0.4 =30.83 > 22 Column is long about y-axis

# Nominal axial strenght of column Pn=Pnx

- Calculate the minimum eccentricity eminand the minimum moment Mmin

# exmin= (15+0.03h) = 15+0.03*600 = 33 mm

# Pufactored = 2160KN.

# Mmin= Pu*exmin= 2160* 331000 =71.28KN.

- Compute EI

# Ig= bh312= 400*600312=0.72*1010mm4

# βdns=1.2 D (sustaind)1.2D+1.6L=0.666

# EI =0.4*4750fc' Ig1+βdns=0.4*475028'*0.72*10^101+0.666=43.45MN/m2

- Determine the Euler buckling load, Pc:

# Pc=π2EI(kln)2=π2*43.45(1*3.7)2=31.32MN

- Calculate the moment magnifier factor δns :

# Cm=0.6+0.4M1M2=0.6+0.4*1=1>0.4

# δns= Cm1-Pu0.75Pc=11-2.1600.75*31.32=1.101

# 1.4>1.101>1……….ok

# The magnified eccentricity and moment:

# e =emin*δns=33*1.101=36.333 mm

# Mux=δns*Mux=1.101*71.28=78.48 KN.m

- Select column reinforcement

# We will use the tide column interaction diagrams

# eyh=36.333600=0.0605

# Compute ratio γ

# γ=d-d'h=600-2*40-2*10-18600=0.803

# ∅PnAg=PuAg=2160*10-30.6*0.4=9Mn/m^2

# ρg=0.012

# ∴select reinforcement

# Ast=ρgAg=0.012*600*400=2880mm2

# As∅18=254.47 mm2

# AsAs∅20=9.43

# Use 12∅18withAs=3052mm2 >2800mm2 ok

# *Design of ties

# Use ties ∅10 with spacing of ties shall not exceed

- 48 times the tie diameter , 48ds=48*10=480mm

- 16 times the longitudinal bar diameter 16db=16*18=288mm ……….. control

- The lest dimension of  column =40 mm

# Use ties ∅10@ 200 mm

# 1-Check for clear spacing between longitudinal bars

# Clear spacing =600-40*2-10*2-2*163=154.7>40 mm>1.5*18=27

# 80.8

(4.13) Design of footing:

Pu = 1146 KN ....    fc'=28 Mpa

# Calculate the weight of footing, soil and the surcharge floor load

# hfooting=45 cm

# Wfooting=0.45*25=11.25KN/ m2

# Weight soil = 0.45*18 = 8.1 KN/ m2

# Total surcharge load on foundation

# W=11.25+8.1+5 = 24.35 KN/ m2

- net soil pressure, qanet:

# qanet=350-24.35=325.65 KN/ m2

# Pu = 1146 KN

# qu=11462*1.8=318.33KN/m2

# qbu=1.4*q=1.4*318.33=445.662>qanet   ….. ok

# calculate d:

# Vu=qu*ba2-l2-d=445.662*1.8(22-0.552-d)

# Vu=581.59-802.19 d

# Vc=16*fc'*bw*d  =16*28*1800*d=1587.45 d

# ∅Vc= Vu ………………….∅=0.75

# 0.75*1587.45=1190.60

# 1190.60d = 518.59 -802.19 d

# d =0.260m

# assume cover 50mm and steel bar ∅14

# h =260+75+14 =349mm

# take h =450mm

# d = 450-75-14 =361mm

- Punching shear

# Let Vu=∅Vc

# Vu=445.6622*1.8-0.911*0.711

# Vu=1315.72 KN

# B = 550350=1.57

# b°=2*91.1+2*71.1= 3.244m

# αs=40-interior

# Vc=16*1+2Bfc'*b°*d             where16*1+2B=16*1+21.57=.38

# Vc= 112αsdb°+2fc'*b°*d     where 112αsdb°+2=11240*0.3863.244+2=0.54

# Vc=13*fc'*b°*d                        where 13=0.333…………control

# Take    Vc=13*fc'*b°*d =  13*28*3244*361*10-3=2065.6KN

# ∅Vc=0.75*2065.6=1549.2 KN

# ∅Vc=1549.2>Vu=1315.72KN………ok

# Design flexure in long direction

# b =2m               h =450mm                d=450-75-14/2=368

# Mu=445.662*2*0.541*0.541/2=130.44KN.m

# m = fy0.85 fc' = 4200.85*28 = 17.65  .

# Rn = Mu∅ b*d2 = 130.44*1060.9*2000* (368)2 = 0.535MPa.

# ρ = 1m(1- 1-2*Rn*mfy )

# = 117.651- 1-2*17.65*0.535420= 0.0013

# As = ρ * b *d = 0.0013 * 2000 * 368 = 956.8 mm2.

# Asmin= 0.0018*b*h= 0.0018*2000*450=1620mm2

# Asmin = 1620 mm2>Asreq = 956.8mm2.

# ∴ As = 1620 mm2.

# n= AsreqAbar∅14 = 1620 154 +1=11.52

# ∴ Use 12Ф14  with Aspro= 1848 mm2.

# S=2000-75*2-12*1411=153mm

# Step S is the smallest of

- 3h = 3*450=1350mm

- 450………control

# S = 153<Smax=450……….ok

# Check of strain

# a=AsAf0.85fc'b=1848*4200.85*28*2000=16.3

# c=a0.85=16.30.85=19.2

# ∈=0.003d-cc=0.003361-19.219.2=.0534>0.005………….ok

# 73

- Design flexure for short direction:

# Take steel bare of ∅14

# b =1800mm      h =450mm

# d =450-75-14-142=354mm

# fc'=28 MPa                fy=420 MPa

# Mu=445.662*1.8*0.548*0.548/2=120.45 KN.m

# m = fy0.85 fc' = 4200.85*28 = 17.65  .

# Rn = Mu∅ b*d2 = 120.45*1060.9*1800* (354)2 = 0.593MPa.

# ρ = 1m(1- 1-2*Rn*mfy )

# = 117.651- 1-2*17.65*0.593420= 0.00143

# As = ρ * b *d = 0.00143 * 1800 * 354= 911.65 mm2.

# Asmin= 0.0018*b*h= 0.0018*1800*450=1458mm2

# Asmin = 1458 mm2>Asreq = 911.65 mm2.

# ∴ As = 1458 mm2.

# n= AsreqAbar∅14 = 1458 154 +1=10.47

# ∴ Use 11Ф14   with As=1694 mm2

# S=1800-75*2-11*1410=149.6mm

# S = 149.6<Smax=450……….ok

Design of stairs :

Design of flight :

Calculation of load :

|  | density kn/m2 | Calculation kn/m |
| --- | --- | --- |
| flight | 25 | 25*.25/cos29.5= 7.81 |
| plaster | 22 | 22*.02/cos29.5= 0.51 |
| horizontal mortar | 22 | 22*.03*1= 0.66 |
| horizontal tiles | 23 | 23*.07*33/30 =0.76 |
| vertical tiles | 23 | 23*.07*17.5/30= 0.4 |
| vertical mortar | 22 | 22*0.03*17.5/30= 0.39 |
| triangle | 25 | 25*0.17/2= 2.1 |
| ∑=12 kn/m |

Calculation of  h min:

h  min: 4.1/20 =20.5 cm

choose h = 25 cm

Design of flight :

dead load =12 kn /m

live load   = 5 kn /m

qu=1.2*DD+ 1.6*LL

qu= 1.2*12+ 1.6*5

qu=22.4 kn /m

AU= BU = 22.4*3.6/2 =49.3KN

Max Vu =40.3*cos 29.5 = 35 kn

Max mu =40.3*2.3 – 22.4*1.8*0.9 = 56.4kn.m

Design for shear :

Max vu =35kn

d = 250-20-12/2 = 224mm

∅Vc=0.75*16*28.5*1000*224 =148 KN

∅Vc ≥ vu

No shear reinforcement required

Design moment :

Max Mu = 56.4 kn/m

# m = fy0.85 fc' = 4200.85*28 = 17.65

# Rn = Mu∅ b*d2 = 56.4*1060.9*1000* (224)2 = 1.25MPa.

# ρ = 1m(1- 1-2*Rn*mfy )= 0.00305

# As = ρ * b *d = 0.0.00305 * 100 * 22.4= 6.85 cm2.

# Asmin= 0.0018*b*h= 0.0018*100*25=4.5 cm2

# ∴ As = 6.85cm2.

# Select 1∅14 / 20cm

# As =π/4 1.4^2 *100/2 = 7.6≥ As req

# Check strain :

# As *fy = 0.85 *fc * a * b

# β 1=0.85

# 7.69*420= .85*28* a *100

# a= 13.5mm

# x = 13.5/0.85 = 15.96mm

# strain

# d-xx0.003 = 0.039 > 0.005 ok.

# ∅=0.9

# Design of landing :

# Calculation of dead load :

| material | density kn/m2 | calculation |
| --- | --- | --- |
| concrete | 25 | 25*025 =6.25 |
| sand | 16 | 16*0.07 =1.1 |
| mortar | 22 | 22*0.02 = 0.4 |
| tiles | 23 | 23*0.03 =0.7 |
| plaster | 22 | 22*.02 =0.4 |
| # ∑ =8.85 kn/m |

# Dead load = 8.85 kn/m

# Live load = 5 kn / m

# Factoral load :

qu=1.2*DD+ 1.6*LL

qu= 1.2*8.85+ 1.6*5

qu=18.62 /m

# Au=Bu from flight =  40.3

# 40.3+18.62 = 58.92 kn/m

# Au = Bu= 165 KN

# Analysis

# MAX Vu =165-58.92*.349= 144.4 kn

# MAX MU =(165*5.6/2 ) – (58.92*3.3*1.65)=223.6  KN. M

# Design of shear:

# MAX Vu = 144.4 kn

∅Vc=0.75*16*28.5*1000*224 =148 KN

# No shear reinforcement required .

# Design of moment :

# MAX MU = 223.6 KN. M

# m = fy0.85 fc' = 4200.85*28 = 17.65

# Rn = Mu∅ b*d2 = 223.6*1060.9*1000* (224)2 = 4.95MPa.

# ρ = 1m(1- 1-2*Rn*mfy )= 0.01336

# As =30 cm2

# As>As min

# Use  1∅18/ 10cm

# Check strain :

# As *fy = 0.85 *fc * a * b

# β 1=0.85

# 300*420= .85*28* a *100

# a= 52.9mm

# x = 13.5/0.85 = 62.2 mm

# strain :

# d-xx0.003 = 0.0077> 0.005 ok.

# ∅=0.9