Chapter IV

Structural analysis and design

Contents:

4-1 Introduction.

4-2 Design Method and Requirements.

4-3 Check of Minimum Thickness of Structural Member.

4-4 Design of Topping.

4-5 Design of One Way Rib Slab.

4-6 Design of Beam.

4-7 Design of One Way Solid Slab.

4-8 Design of Column(C28/GF).

4-9 Design of Isolated Footing.

4-10 Design of Stair.

4-11 Design of Basement Wall .

4-12 Design of Shear Wall(SW1,F2).

4-13 Column Coordinates.

4-1 Introduction

Many structures are built of reinforced concrete: bridges, buildings, retaining walls, tunnels and others. Reinforced concrete is logical union of two materials: plain concrete, which possesses high compressive strength but little tensile strength, and steel bars embedded in the concrete, which can provide the needed strength in tension.

Plain concrete is made by mixing cement, fine aggregate, coarse aggregate, water, and frequently admixtures. Understanding of reinforced concrete behavior is still far from complete, building codes and specifications that give design procedures are continually changing to reflect latest knowledge.

Structural concrete can be classified into:

- Lightweight concrete with unit weight from about 1350 to 1850 kg/m3.

- Normal weight concrete with unit weight from about 1800 to 2400 kg/m3.

- Heavyweight concrete with unit weight from about 3200 to 5600 kg/m3.

4-2 Design Method and Requirements

The design strength provided by a member is calculated in accordance with the requirements and assumptions of ACI_code (318_08).

Strength design method:

In ultimate strength design method, the service loads are increased by factors to obtain the load at which failure is considered to be occurring, this load called factored load or factored service load, the structure or structural element is then proportioned such that the strength is reached when factored load is acting, the computation of this strength takes into account the nonlinear stress-strain behavior of concrete.

The strength design method is expressed by the following,

Strength provided ≥ strength required to carry factored loads.

NOTE:

The statically calculation and the key plans dependent on the architectural plans.

Code: ACI  2008 UBC

Material: Concrete B300

For circular section.

But, for rectangular section ().

Reinforcement steel:

The specified yield strength of the reinforcement {fy = 420 N/mm² (MPa)}.

Factored loads:

The factored loads for members in our project are determined by:

Wu = 1.2 DL + 1.6 LL                   ACI-code-318-08(9.2.1)

4-3 Check of Minimum Thickness of Structural Member

| Minimum thickness ( h) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Member | Simply supported | One end continuous | Both end continuous | Cantilever |
| solid one way slabs | L/20 | L/24 | L/28 | L/10 |
| Beams or ribbed one way slabs | L/16 | L/18.5 | L/21 | L/8 |

Table 4-1: Check of minimum thickness of structural member.

For Rib :

hmin for(one end continuous)=L/18.5=6.16/18.5=33.3cm.

hmin for(both end continuous)=L/21=5.9/21=28.1cm.

hmin for(simply supported )=L/16=4.97/16=31cm.

Take h = 35 cm.

27 cm block + 8 cm topping = 35cm.

For Beam:

hmin for(one end continuous)=L/18.5=5.85/18.5=31.6cm.

hmin for(both end continuous)=L/21=5.95/21=28.3cm.

hmin for(cantilever )=L/8=2/8=25cm.

Take h = 35 cm, but in some regions we have a drop beam.

4-4 Design of Topping

Statically System For Topping :

Consider the topping as strip of (1m) width, and span of mold length with both end fixed in the ribs.

Figure 4-1: Topping load.

Load Calculations:

Dead Load:

| Calculation |  | Parts of Rib | No. |
| --- | --- | --- | --- |
| 0.03*23*1 = 0.69 KN/m |  | Tiles | 1 |
| 0.02*22*1 = 0.44 KN/m |  | Mortar | 2 |
| 0.07*17*1 = 1.19 KN/m |  | Coarse Sand | 3 |
| 0.08*25*1 = 2.0 KN/m |  | Topping | 4 |
| 1*1= 1 KN/m |  | partions | 5 |
| 5.32KN/m | Sum = |

Table 4-2: Dead load calculation of topping.

Live Load:

LL =5 KN/m2.

LL =5 KN/m2×1m=5KN/m.

Factored Load :

WU = 1.2 ×5.32 + 1.6×5 =14.4 KN/m.

Check the strength condition for plain concrete, øMn ≥ Mu, where ø = 0.55.

Mn = 0.42 λ fc' Sm (ACI 22.5.1, equation 22-2).

Sm=b.h26=1000.8026=1066666.67 mm2.

øMn =0.55×0.42×1×24 ×1066666.67 ×10-6 =1.21 KN.m

Mu = WuL212=0.192 KN.m                 (negative moment)

Mu = WuL224=0.96 KN.m                      (positive moment)

øMn>> Mu =  0.192 KN.m

Figure 4-2: Moment diagram.

No reinforcement is required by analysis. According to ACI 10.5.4, provide As,min for slabs as shrinkage and temperature reinforcement.

ρshrinkage= 0.0018.                                            ACI 7.12.2.1

As = ρ×b×htopping  =0.0018 ×1000×80 = 144 mm2/m

Step (s) is the smallest of:

3h = 3×80 =240 mm         control  ACI 10.5.4              450mm.

S =380280fs-2.5Cc=380 28023420-2.5 .20=330mmACI 10.6.4.

Take ø 8 @ 200 mm in both direction , S = 200 mm < Smax = 240 mm … OK

4-5 Design of One Way Rib Slab

Requirements For Ribbed Slab Floor According to  ACI- (318-08) .

bw ≥ 10cm……………………………………………ACI(8.13.2)

Select bw=12 cm

h ≤ 3.5*bw  ……………………………….…ACI(8.13.2)

Select h=35cm<3.5*12= 49 cm

tf ≥ Ln/12=600/12 ≥50mm …………………………………..ACI(8.13.6.1)

Select tf=8cm

Material :concrete    B300                Fc' = 24 N/mm2

Reinforcement Steel         fy = 420 N/mm2

Section :

B = 520 mm ,Bw= 120 mm ,h= 350 mm ,t= 80 mm , d=350-20-10-12/2= 314 m.

Statically System and Dimensions:

Figure 4-3: One way rib slab (R12).

Figure 4-4: Geometry of rib slab (R12).

Figure 4-5: Statically system and loads distribution of rib (R12).

Load Calculation:

Dead Load:

| Calculation | Parts of Rib | No. |
| --- | --- | --- |
| 0.03*23*0.52 = 0.359 KN/m/rib | Tiles | 1 |
| 0.03*22*0.52 = 0.229 KN/m/rib | Mortar | 2 |
| 0.07*17*0.52 = 0.620 KN/m/rib | Coarse Sand | 3 |
| 0.08*25*0.52 = 1.04 KN/m/rib | Topping | 4 |
| 0.27*25*0.12 = 0.81 KN/m/rib | RC. Rib | 5 |
| 0.27*10*0.4 = 1.08 KN/m/rib | Hollow Block | 6 |
| 0.02*22*.52= 0.229 KN/m/rib | plaster | 7 |
| 1*0.52= 0.52 KN/m/rib | partions | 8 |
| Sum = 5.1 KN/m/rib |

Table 4-3: Dead Load Calculation of Rib(R12).

Dead  Load /rib = 5.1 KN/m.

Live Load:

Live load = 5 KN/M2.

Live load /rib = 5 KN/m2 × 0.52m = 2.6 KN/m.

Effective Flange Width ( ):-ACI-318-11   (8.10.2)

For T- section is the smallest of the following:-

= L / 4 = 619/ 4 =154.75cm

= 12 + 16 t = 12 + 16 (8) = 140 cm

= be ≤ center to center spacing between adjacent beams = 52 cm.                  Control

For T-section = 52cm .

Figure 4-6: Shear and moment envelope diagram of rib (R12 ).

Moment Design for (R 12):

Design of Positive Moment for (Rib12 ):-(Mu=23.7 KN.m)

Assume bar diameter ø 12 for main positive reinforcement

d =h- cover - dstirrups-db2=350-20-10-122=314 mm

Check if  a>hf  to determine whether the section will act as rectangular or T- section.

Mnf =0.85. fc'.be.hf.(d-hf2)

= 0.85×24×520×80×314-802×10-6=232.5 KN.m

Mn≫Muφ=23.70.9= 26.33KN.m , the section will be designed as rectangular section                     withbe =520 mm.

Rn=Mu∅bd2=23.7×1060.9×520×3142=0.514 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.514420=0.00124

As,req = ρ.b.d = 0.00124 ×520×314 = 202.5 mm2

Check for As min:

A=ACI-318 (10.5.1)

A=

A=

A=controls

Asreq= 202.5mm2  >Asmin= 125.6 mm2           OK

Use 2 ø 12 ,As,provided= 226 mm2>As,required= 202.5mm2 ….  Ok

S =120-40-20-(2×12)1=36 mm>db=12>25  mm        OK

Check for strain:

a =As.fy0.85b fc'=226×4200.85×520×24=8.94 mm

x=aB1=8.940.85=10.53 mm

εs=0.003d-xx=0.003314 -10.5310.53=0.0864>0.005                0k

Design of Positive Moment for(Rib12 ):- (Mu=11.7 KN.m)

d =h- cover - dstirrups-db2=350-20-10-122=314 mm

Rn=Mu∅bd2=11.7×1060.9×520×3142=0.25 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.25420=0.000599

As,req = ρ.b.d = 0.000599×520×314 = 97.8 mm2

Check for As min:-

A=ACI-318 (10.5.1)

A=

A=

A=controls

As,required= 125.6  mm2.

Use 2 ø 10 ,As,provided= 157.08 mm2>As,required= 125.6  mm2  … Ok

S =120-40-20-(2×10)1=40 mm>db=10>25  mm        OK

Check for strain:

a =As.fy0.85b fc'=157×4200.85×520×24=6.22 mm

x=aB1=6.220.85=7.31mm

εs=0.003d-xx=0.003314-7.317.31=0.125>0.005                0k

Design of Positive Moment for (Rib12 ):- (Mu=12.9 KN.m)

Assume bar diameter ø 12 for main positive reinforcement

d =h- cover - dstirrups-db2=350-20-10-122=314 mm

Rn=Mu∅bd2=12.9×1060.9×520×3142=0.28 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.28420=0.000671

As,req = ρ.b.d = 0.000671×520×314 = 109.56 mm2

Check for As min:

A=ACI-318 (10.5.1)

A=

A=

A=controls

Asreq = 125.6 mm2

Use 2 ø10 ,As,provided=157.08 mm2>As,required= 125.6  mm2…  Ok

S =120-40-20-(2×10)1=40 mm>db=10>25  mm        OK

Check for strain:

a =As.fy0.85b fc'=157×4200.85×520×24=6.22 mm

x=aB1=6.220.85=7.31 mm

εs=0.003d-xx=0.003314-7.317.31=0.125>0.005                0k

Design of Positive Moment for (Rib12 ):- (Mu=33.7 KN.m)

Assume bar diameter ø 12 for main positive reinforcement

d =h- cover - dstirrups-db2=350-20-10-122=314 mm

Rn=Mu∅bd2=33.7×1060.9×520×3142=0.73 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.73420=0.00177

As,req = ρ.b.d = 0.00177×520×314 = 289 mm2

Check for As min:

A=ACI-318 (10.5.1)

A=

A=

A=controls

Asreq= 289 mm2  >Asmin= 125.6 mm2           OK

Use 2 ø14 ,As,provided=308 mm2>As,required= 289  mm2…  Ok

S =120-40-20-(2×14)1=32 mm>db=14>25  mm        OK

Check for strain:

a =As.fy0.85b fc'=308×4200.85×520×24=12.2 mm

x=aB1=12.20.85=14.35 mm

εs=0.003d-xx=0.003314-14.3514.35=0.0626>0.005                0k

Design of Negative Moment for (Rib12 ):- (Mu= -16.9 KN.m)

Assume bar diameter ø 12 for main positive reinforcement

d =h- cover - dstirrups-db2=350-20-10-122=314 mm

Rn=Mu∅bd2=16.9×1060.9×120×3142=1.59 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×1.59420=0.00395

As,req = ρ.b.d = 0.00395×120×314 = 148.8 mm2

Check for As min:

A=ACI-318 (10.5.1)

A=

A=

A=controls

Asreq = 148.8mm2  >Asmin= 125.6 mm2OK

Use 2 ø 10 ,As,provided= 157.1 mm2>As,required= 148.8  mm2… Ok

S =120-40-20-(2×10)1=40 mm>db=10>25  mm        OK

Check for strain:-

a =As.fy0.85b fc'=157.1×4200.85×120×24=26.95 mm

x=aB1=26.950.85=31.7 mm

εs=0.003d-xx=0.003314-31.731.7=0.0267>0.005                0k

Design of Negative Moment for (Rib12 ):- (Mu= -18.5 KN.m)

Assume bar diameter ø 12 for main positive reinforcement

d =h- cover - dstirrups-db2=350-20-10-122=314 mm

Rn=Mu∅bd2=18.5×1060.9×120×3142=1.74 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×1.74420=0.00434

As,req = ρ.b.d = 0.00434×120×314 = 163.5 mm2

Check for As min:-

A=ACI-318 (10.5.1)

A=

A=

A=controls

Asreq = 163.5mm2  >Asmin= 125.6 mm2OK

Use 2 ø 12 ,As,provided= 226 mm2>As,required= 163.5  mm2… Ok

S =120-40-20-(2×12)1=36 mm>db=12>25  mm        OK

Check for strain:-

a =As.fy0.85b fc'=226×4200.85×120×24=38.77 mm

x=aB1=38.770.85=45.62 mm

εs=0.003d-xx=0.003314-45.6245.62=0.0176>0.005                0k

Design of Negative Moment for (Rib12 ):- (Mu=-9.5 KN.m)

Assume bar diameter ø 12 for main positive reinforcement

d =h- cover - dstirrups-db2=350-20-10-122=314 mm

Rn=Mu∅bd2=9.5×1060.9×120×3142=0.892 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.892420=0.00217

As,req = ρ.b.d = 0.00217 ×120×314 = 81.76 mm2

Check for As min:

A=ACI-318 (10.5.1)

A=

A=

A=controls

Asreq = 125.6 mm2

Use 2 ø10 ,As,provided= 157.08 mm2>As,required= 125.6 mm2…  Ok

S =120-40-20-(2×10)1=40 mm>db=10>25  mm        OK

Check for strain:

a =As.fy0.85b fc'=157×4200.85×120×24=6.22 mm

x=aB1=6.220.85=7.31mm

εs=0.003d-xx=0.003284-7.317.31=0.125>0.005                0k

Design of Negative Moment for (Rib12 ): (Mu=-9.1 KN.m)

Assume bar diameter ø 12 for main positive reinforcement

d =h- cover - dstirrups-db2=350-20-10-122=314 mm

Rn=Mu∅bd2=9.1×1060.9×120×3142=0.855 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×0.855420=0.00208

As,req = ρ.b.d = 0.00208×120×314 = 78.37 mm2

Check for As min:

A=ACI-318 (10.5.1)

A=

A=

A=controls

Asreq = 125.6 mm2

Use 2 ø10 ,As,provided= 157.08 mm2>As,required= 125.6 mm2…  Ok .

S =120-40-20-(2×10)1=40 mm>db=10>25  mm        OK

Check for strain:

a =As.fy0.85b fc'=157×4200.85×120×24=6.22 mm

x=aB1=6.220.85=7.31mm

εs=0.003d-xx=0.003284-7.317.31=0.125>0.005                0k

Design of Negative Moment for (Rib12 ): (Mu=-27.8 KN.m)

Assume bar diameter ø 12 for main positive reinforcement

d =h- cover - dstirrups-db2=350-20-10-122=314 mm

Rn=Mu∅bd2=27.8×1060.9×120×3142=2.61 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×2.61420=0.00667

As,req = ρ.b.d = 0.00667 ×120×314 = 251.3 mm2

Check for As min:

A=ACI-318 (10.5.1)

A=

A=

A=controls

Asreq = 251.3 mm2  >Asmin= 125.6 mm2OK

Use 2 ø14 ,As,provided= 308 mm2>As,required= 251.3mm2…  Ok

S =120-40-20-(2×14)1=36 mm>db=14>25  mm        OK

Check for strain:

a =As.fy0.85b fc'=308×4200.85×120×24=12.2 mm

x=aB1=12.20.85=14.35 mm

εs=0.003d-xx=0.003284-14.3514.35=0.0626>0.005                0k

Design of Negative Moment for (Rib12 ): (Mu=-25.3KN.m)

Assume bar diameter ø 12 for main positive reinforcement

d =h- cover - dstirrups-db2=350-20-10-122=314 mm

Rn=Mu∅bd2=25.3×1060.9×120×3142=2.38 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×2.38420=0.00604

As,req = ρ.b.d = 0.00604 ×120×314 = 227.59 mm2

Check for As min:-

A=ACI-318 (10.5.1)

A=

A=

A=controls

Asreq = 227.59 mm2  >Asmin= 125.6 mm2OK

Use 2 ø14 ,As,provided= 308 mm2>As,required= 227.59 mm2…  Ok

S =120-40-20-(2×14)1=36 mm>db=14>25  mm        OK

Check for strain:-

a =As.fy0.85b fc'=308×4200.85×120×24=12.2 mm

x=aB1=12.20.85=14.35 mm

εs=0.003d-xx=0.003284-14.3514.35=0.0626>0.005                0k

Shear Design for (R 12 ):

Vu at distance d from  support= 30.9 KN

Shear strength Vc, provided by concrete for the joists may be taken 10% greater than for beams. This is mainly due to the interaction between the slab and closely spaced ribs.(ACI, 8.13.8).

Vc =1.16fc'bwd=1.1624 ×120×314×10-3=33.84 KN

øVc =0.75×33.84 =25.38 KN

0.5 ø Vc =0.5×25.38 =12.69 KN

0.5 ø Vc< Vu< ø Vc

Vu> ø Vc

For shear design, shear reinforcement is required (Av,),

Vsmin =116fc'bw d≥13  bw d

Vs min=11624*120*314=11.54kn

Vsmin = 13  bw d= 13*120*314=12.56kn

ø(Vc+Vsmin)= 0.75(33.84+12.56)=34.8kn

øVc<Vu <ø (Vc+Vsmin)

25.38 < 30.9 <34.8

For shear design, minimum shear reinforcement is required (Av,min), Reinforcement.

Use stirrups (2 leg stirrups ) ø 8@150 mm , Av = 2×  50.24 = 100.5 mm2

Avmin =116fc'bwsfyt≥13bwsfyt

Avmin=100.5 =11624120s420 →s=1.145m

100.5 =13120s420 →s=1.055m

S max→d2=157mm

S max →≤600mm

Take  (2 leg stirrups ) ø 8 @ 150 mm

Av=2*50.30.15=670.67 mm2/mstrip

Vu at distance d from  support= 24.7 KN

Shear strength Vc, provided by concrete for the joists may be taken 10% greater than for beams. This is mainly due to the interaction between the slab and closely spaced ribs.(ACI, 8.13.8).

Vc =1.16fc'bwd=1.1624 ×120×314×10-3=33.84 KN

øVc =0.75×33.84 =25.38 KN

0.5 ø Vc =0.5×25.38 =12.69 KN

Vu=24.7< 0.5 ø Vc =12.69     no

0.5 ø Vc< Vu< ø Vc

12.69< 24.7 < 33.84          ok.

Minimum shear reinforcement is required except for concrete joist construction.

So, no shear required reinforcement is provided.

Vu at distance d from  support= 20.8 KN

Shear strength Vc, provided by concrete for the joists may be taken 10% greater than for beams. This is mainly due to the interaction between the slab and closely spaced ribs.(ACI, 8.13.8).

Vc =1.16fc'bwd=1.1624 ×120×314×10-3=33.84 KN

øVc =0.75×33.84 =25.38 KN

0.5 ø Vc =0.5×25.38 =12.69 KN

Vu=20.8< 0.5 ø Vc =12.69     no

0.5 ø Vc< Vu< ø Vc

12.69< 20.8 < 33.84          ok.

Minimum shear reinforcement is required except for concrete joist construction.

So, no shear required reinforcement is provided.

Vu at distance d from  support= 24.8KN

Shear strength Vc, provided by concrete for the joists may be taken 10% greater than for beams. This is mainly due to the interaction between the slab and closely spaced ribs.(ACI, 8.13.8).

Vc =1.16fc'bwd=1.1624 ×120×314×10-3=33.84 KN

øVc =0.75×33.84 =25.38 KN

0.5 ø Vc =0.5×25.38 =12.69 KN

Vu=24.8< 0.5 ø Vc =12.69     no

0.5 ø Vc< Vu< ø Vc

12.69< 24.8 < 33.84          ok.

Minimum shear reinforcement is required except for concrete joist construction.

So, no shear required reinforcement is provided.

So we take for all rib the maximum case:

Take  (2 leg stirrups ) ø 8 @ 150 mm

Av=2*50.30.15=670.67 mm2/mstrip

4-6 Design of Beam

Material : concrete    B300                Fc' = 24 N/mm2

Reinforcement Steel         fy = 420 N/mm2

Section :

B = 60 cm

h= 35cm

d=350-40-10-18/2= 291 mm

Statically System and Dimensions:

Figure 4-7: Geometry of beam (B1).

Figure 4-8: Statically system and loads distribution of beam (B 1).

Load Calculations:

Dead Load Calculations for Beam (B1):

The distributed Dead and Live loads acting upon B1 can be defined from the support reactions of the R12.

From Rib12

The maximum support reaction from Dead Loads for R12 upon B1 is7.54 KN, The distributed Dead Load from the R12 on B1.

DL =(12.68/ 0.52) =  24.38 KN / m

Self-weight of beam = 0.35*0.6*25 = 5.25 KN / m

DL =24.38+5.25 = 18.83 KN / m

Dead Load from External wall

D = 3.45*0.3 *25 = 25.9 Kn/m

Live Load calculations for Beam (B1):

From Rib12

The maximum support reaction from Live Loads for R12upon B 1 is 6.69 KN  The distributed Live Load from the Rib 12 on B1.

LL =6.69/ 0.52=  13.38 KN/m.

Figure 4-9: Shear and moment envelope diagram of  beam (B1).

Moment Design for (B11):

Flexural Design of Positive Moment for(B1):-(Mu=203.7 KN.m)

Determine of  Mn,max

d =350 – 40 -10 – 18\2 = 291 mm

x=37d=37.291=124.7 mm

a =B.x=124.7*0.85=106 mm

Mnmax=  0.85*fc'*a *b( d -   a2  ) = 0.85*24*106*600*(291-106/2 ) *10-6=  308.8 KN.m

∅ Mnmax = 0.82* 308.8 = 253.22 KN.m >203.7 KN.m .

Design as singly reinforcement

Rn=Mu∅bd2=203.7×1060.9×600×2912=4.45Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×4.45420=0.0121

As = ρ.b.d = 0.0121×600×291 = 2112.66 mm2

Check for As,min:

Asmin =  =   = 509.14 mm2

Asmin =  = = 582 mm2Controls

As= 2112.66 mm2

Use 7ø 20 Bottom, As,provided= 2200 mm2>As,required= 2112.66 mm2…  Ok

Check spacing :

S =600-40*2-20-(7×20)6=60 mm>db=20>25 mm      OK

Check for strain:-

a =As.fy0.85b fc'=2200×4200.85×600×24=75.5 mm

x=aB1=75.50.85=88.8 mm

εs=0.003d-xx=0.003291-88.888.8=0.00683>0.005                0k

Flexural Design of Positive Moment for (B1):-(Mu=127.3 KN.m)

Rn=Mu∅bd2=127.3×1060.9×600×2912=2.78Mpa.

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×2.78420=0.00714

As = ρ.b.d = 0.00714×600×291 = 1246.6 mm2.

Check for As,min:

Asmin =  =   = 509.14 mm2

Asmin =  = = 582 mm2Controls

As= 1246.6 mm2.

Use 5ø18Bottom, As,provided= 1272.35 mm2>As,required= 1246.6 mm2 … Ok

Check spacing :

S =600-40*2-20-(5*18)4=102.5mm>db=18>25       OK

Check for strain:-

a =As.fy0.85b fc'=1272.35 ×4200.85×600×24=43.66 mm

x=aB1=43.660.85=51.36 mm

εs=0.003d-xx=0.003291-51.3651.36=0.01399>0.005                0k

Flexural Design of Positive Moment for(B1):-(Mu=157.9 KN.m)

Rn=Mu∅bd2=157.9×1060.9×600×2912=3.45 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×3.45420=0.00906

As = ρ.b.d = 0.00906 ×600×291 = 1581.9 mm2

Check for As,min:

Asmin =  =   = 509.14 mm2

Asmin =  = = 582 mm2

As = 1581.9 mm2Controls

Use 7ø 18, As,provided= 1781.3 mm2>As,required= 1581.9 mm2…  Ok

Check spacing:

S =600-40*2-20-(7*18)6=62.33 mm>db=18>25        OK

Check for strain:

a =As.fy0.85b fc'=1781.3 ×4200.85×600×24=61.12 mm

x=aB1=61.120.85=71.9mm

εs=0.003d-xx=0.003291-71.971.9=0.00914>0.005                0k

Flexural Design of Positive Moment for(B1 ):-(Mu=188.5 KN.m)

Rn=Mu∅bd2=188.5×1060.9×600×2912=4.12 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×4.12420=0.0111

As = ρ.b.d = 0.0111 ×600×291 = 1938.1 mm2

Check for As,min:

Asmin =  =   = 509.14 mm2

Asmin =  = = 582 mm2

As = 1938.1mm2Controls

Use 8 ø 18 ,As,provided= 2035.75  mm2>As,required= 1938.1 mm2…  Ok

Check spacing :

S =600-40*2-20-(8*18)7=50.85 mm>db=18>25        OK

Check for strain:-

a =As.fy0.85b fc'=2035.75 ×4200.85×600×24=70 mm

x=aB1=700.85=82.35 mm

εs=0.003d-xx=0.003291-82.3582.35=0.00758>0.005                0k

Flexural Design of Negative Moment for(B1):-(Mu=-198.3 KN.m)

Rn=Mu∅bd2=198.3×1060.9×600×2912=4.34 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×4.34420=0.0118

As = ρ.b.d = 0.0118 ×600×291 = 2060.28  mm2

Check for As,min:

Asmin =  =   = 509.14  mm2

Asmin =  = = 582 mm2

As =2060.28 mm2Controls

Use7 ø 20 ,As,provided= 2200 mm2>As,required= 2060.28mm2…  Ok

Check spacing :

S =600-40*2-20-(7*20)6=60 mm>db=20>25        OK

Check for strain:

a =As.fy0.85b fc'=2200×4200.85×600×24=75.5 mm

x=aB1=75.50.85=88.8mm

εs=0.003d-xx=0.003291-88.888.8=0.00683>0.005                0k

Flexural Design of Negative Moment for(B1):-(Mu=-205.1 KN.m)

Rn=Mu∅bd2=205.1×1060.9×600×2912=4.49 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×4.49[420=0.0122

As = ρ.b.d = 0.0122 ×600×291 = 2130.12  mm2

Check for As,min:

Asmin =  =   = 509.14  mm2

Asmin =  = = 582 mm2

As =2130.12 mm2Controls

Use7 ø 20 ,As,provided= 2200 mm2>As,required= 2130.12 mm2…  Ok

Check spacing :

S =600-40*2-20-(7*20)6=60 mm>db=20>25        OK

Check for strain:

a =As.fy0.85b fc'=2200×4200.85×600×24=75.5 mm

x=aB1=75.50.85=88.8mm

εs=0.003d-xx=0.003291-88.888.8=0.00683>0.005                0k

Flexural Design of Negative Moment for(B1 ):-(Mu=-179.1 m)

Rn=Mu∅bd2=179.1×1060.9×600×2912=3.9 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×3.9420=0.0104

As = ρ.b.d = 0.0104 ×600×291 = 1815.84  mm2

Check for As,min:

Asmin =  =   = 509.14 mm2

Asmin =  = = 582 mm2

As = 1815.84 mm2Controls

Use 6 ø 20 ,As,provided= 1885 mm2>As,required= 1815.84 mm2…  Ok

Check spacing :

S =600-40*2-20-(6*20)5=76 mm>db=20>25        OK

Check for strain:

a =As.fy0.85b fc'=1885×4200.85×600×24=64.7 mm

x=aB1=64.70.85=76.12mm

εs=0.003d-xx=0.003291-76.1276.12=0.00847>0.005                0k

Flexural Design of Negative Moment for(B1 ):-(Mu=-176.2m)

Rn=Mu∅bd2=176.2×1060.9×600×2912=3.85 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×3.85420=0.0106

As = ρ.b.d = 0.0106 ×600×291 = 1850.76 mm2

Check for As,min:

Asmin =  =   = 509.14 mm2

Asmin =  = = 582 mm2

As = 1850.76 mm2Controls

Use 6 ø 20 ,As,provided= 1885 mm2>As,required= 1850.76 mm2…  Ok

Check spacing :

S =600-40*2-20-(6*20)5=76 mm>db=20>25        OK

Check for strain:

a =As.fy0.85b fc'=1885×4200.85×600×24=64.7 mm

x=aB1=64.70.85=76.12mm

εs=0.003d-xx=0.003291-76.1276.12=0.00847>0.005                0k

Flexural Design of Negative Moment for(B1 ):-(Mu=-215.7 kn.m)

Rn=Mu∅bd2=215.7×1060.9×600×2912=4.72 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×4.72420=0.013

As = ρ.b.d = 0.013 ×600×291 = 2269.8 mm2

Check for As,min:

Asmin =  =   = 509.14 mm2

Asmin =  = = 582 mm2

As =2269.8 mm2Controls

Use 8ø 20,As,provided= 2513.3 mm2>As,required= 2269.8 mm2…  Ok

Check spacing :

S =600-40*2-20-(8*20)7=48.6 mm>db=20>25        OK

Check for strain:-

a =As.fy0.85b fc'=2513.3×4200.85×600×24=86.24 mm

x=aB1=86.240.85=101.46 mm

εs=0.003d-xx=0.003291-101.46101.46=0.0056>0.005                0k

Flexural Design of Negative Moment for(B1 ):-(Mu=-212.9 kn.m)

Rn=Mu∅bd2=212.9×1060.9×600×2912=4.66 Mpa

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420=120.61-1-2×20.6×4.66420=0.0128

As = ρ.b.d = 0.0128 ×600×291 = 2234.88 mm2

Check for As,min:

Asmin =  =   = 509.14 mm2

Asmin =  = = 582 mm2

As =2234.88 mm2Controls

Use 8ø 20,As,provided= 2513.3 mm2>As,required= 2234.88 mm2…  Ok

Check spacing :

S =600-40*2-20-(8*20)7=48.6 mm>db=20>25        OK

Check for strain:-

a =As.fy0.85b fc'=2513.3×4200.85×600×24=86.24 mm

x=aB1=86.240.85=101.46 mm

εs=0.003d-xx=0.003291-101.46101.46=0.0056>0.005                0k

Shear Design for (B 1):

Case 3 :

for shear design, minimum shear reinforcement is required (Av,min), Reinforcement.

Use stirrups (2 leg stirrups ) ø 8/ 150 mm , Av = 2×  50.24 = 100.5 mm2

Vu = 234.2 KN

Vc = 16fc'bw d =  1624*600*291=142.56 KN

Check for section dimensions:

Vs= VuΦ-Vc=234.20.75 – 142.56 = 169.71

Vs,max = 23fc'bw d= 2324*600*291=570.24 KN

Vs= 169.71< Vs,max  = 570.24  - the section is large enough .

Find the maximum stirrups spacing:

If   Vs < Vs'= 13fc'bw d         then          Smax ≤ d2      or        Smax ≤ 600 mm

Vs'= 13fc'bw d  = 1324*600*291 * 10-3 =   285.12 KN

Smax ≤ 600 mm,             Smax ≤ d2  = = 145.5 mm   Control

Check for Vs, min:

Av, min = 116fc'bw  Sfyt    but not less than

Av, min = 13bw  Sfyt    Control   (  116fc' = 4.916 < 13   )

Vs,min= 116fc'bw d = 11624*600*291 * 10-3   = 53.5 KN

Vs,min= 13bw d = 13*600*291 * 10-3   =  58.2 KN   Control

Φ Vc= 0.75*142.56 =106.92  KN

Ф Vsmin≥  0.75 () * bw * d =0 .75* ()*600*291*10-3 = 43.65 KN Controls

Ф Vsmin   ≥ 0.75 () * bw * d = 0.75*() * 600 * 291*10-3 = 40.1 KN

Ф Vc<Vu ≤ Ф Vc + Ф Vsmin

106.92<234.2 ≤ 106.92+43.65= 150.57 …… not satisfied

Cases 1&2&3 is not suitable

Case 4 :

vs'= 13fc'bw d = 1324* 600*291  = 285.12KN

∅(vc+ vs,min⁡)<vu ≤ ∅(vc+ vs')

0.75(142.56+ 43.65) < 234.2< 0.75(142.56 + 285.12)

139.66 < 234.2 <320.76

Use 2 leg Φ 10

As =158 mm2

Vs = Vn – Vc = 234.20.75 – 142.56 = 169.7KN

S= Avfyt dvs=158*420*291169.7*1000=113.95 mm

smax ≤ d2=2912= 145.5 mm                  control

or         smax ≤600 mm

Use 2 leg Φ 10  @120

4-7 Design of One Way Solid Slab.

Figure 4-10: One way solid slab(S1).

Material:-

concrete    B300                Fc' = 24 N/mm2

Reinforcement Steel         Fy = 420 N/mm2

Slab Thickness Calculation:-

The overall depth must satisfy ACI Table (9.5.a):

Min H  (deflection requirement) : “For one end continuous”

For One way solid slab, will use thickness of slab 15 cm.

Load Calculation:-

For the one-way solid slabs, the total dead load to be used in the analysis and design is calculated as follows:

-Load Calculation For the Horizontal Slab:- (For one Meter Strip)

| # | material | calculation |
| --- | --- | --- |
| 1 | Tiles | 0.03*22=0.66 |
| 2 | mortar | 0.03*22=0.66 |
| 3 | Coarse sand | 0.07*16=1.12 |
| 4 | RC concrete | 0.15*25=3.75 |
| 5 | plaster | 0.02*22=0.44 |
|  | Sum | 6.63 |

Table 4-4: Dead Load Calculation of Solid Slab.

Live load =5 KN/m

### Design of Positive Moment :

### Design of Positive Moment :-(Mu=16.5 KN.m)

Assume bar diameter Ф12 for main reinforcement .

m =  = = 20.59

Rn =

Rn =  = 1.19(Mpa)

ρ  = (1 - )

ρ = (1 - ) = 0.0029

As=ρ * b * d = 0.0029* 100 *12.4= 3.6cm²

Check for As min:

A min = =

Asreq= 3.6cm2  >Asmin= 2.7cm2          OK

Use ø 12/25cm , As,provided= 4.52cm2>As,required= 3.6cm2 ….  Ok

### Design of Positive Moment :-( Mu=12.2KN.m)

m =  = = 20.59

Rn =

Rn =  = 0.88 (Mpa)

ρ  = (1 - )

ρ = (1 - ) = 0.0021

As=ρ * b * d = 0.0021* 100 *124= 2.6cm²

Check for As min:

A min = =

Asreq= 2.6cm2  <Asmin= 2.7cm2       Not OK

Use 3ø12/1m strip , As,provided= 3.39cm2≥As,required= 2.6cm2 ….  Ok

### Design of Negative Moment:

### Design of Negative Moment:- (Mu=15.9 KN.m)

m =  = = 20.59

Rn =

Rn =  = 1.15 (Mpa)

ρ  = (1 - )

ρ = (1 - ) = 0.0028

As=ρ * b * d = 0.0028* 100 *124= 3.47 cm²

Check for As min:-

A min = =

Asreq= 3.47cm2  > Asmin= 2.7cm2           OK

Use ø12/25cm , As,provided= 4.52cm2≥As,required= 3.47cm2 ….  Ok

Shrinkage and Temperature:-

→ρ=0.0018

A min =   =       (control)

Use 3Ф12/1m strip.

Shear Design:-

Check  Whether Thickness Is Adequate For Shear:-

= 29.3 KN/ 1m strip

d = h – 15 – db = 200 – 15 – (12 /2) = 124 mm

ΦVc=

=  = 75.9 KN / 1 m strip

ΦVc/2= 37.95 KN > = 29.3 KN/ 1m strip.

The thickness of the slab is adequate enough.

4-8 Design of Column (C28/GF).

Material :

concrete    B350                Fc' = 24 N/mm2

Reinforcement Steel        Fy = 420 N/mm2

Load Calculation:-

Service Load:-

Dead Load =1553.9KN

Live Load =858.65 KN

Factored Load:-

PU = 1.2 ×1553.9+ 1.6×858.65 =3238.52KN

Dimensions of Column:-

Ag= 255605.37 mm2

Assume Rectangular Section

Try h = 600mm

b = 450mm

Figure 4-11: Column (C28) section and reinforcement.

Design of the tie reinforcement :

S ≤16  db (longitudinal bar diameter)

S≤ 48dt  (tie bar diameter).

S ≤ Least dimension.

spacing  ≤16×db=16×2.8  =44.8cm ….

spacing  ≤48×dt=48×1.0 = 48  cm

spacing  ≤ least.dim  =45cm control

Use10@40 cm

For Using Column We have using 1618.

4-9 Design of Isolated Footing.

Material :-

concrete    B350                Fc' = 24 N/mm2

Reinforcement Steel         Fy = 420 N/mm2

Load Calculations:-

Dead Load = 2207.78 Kn , Live Load = 1066.15 Kn

Total services load = 2207.78 + 1066.15 = 3273.93  Kn

Total Factored  load = 1.2*2207.78 + 1.6*1066.15 = 4355.18Kn

Column Dimensions (a*b) = 60*60 cm

Soil density = 20 Kg/cm3

Allowable Bearing Capacity = 350 Kn/m2

Figure 4-12 :Foot plan.

Assume h = 80cm

= 350 – 25*0.8 – 20*0.6 – 0.7*25 = 300.5kn/m2

Area of Footing :-

A=Pt=3273.93300.5= 10.89m2

Assume Square Footing

B required = 3.3 m

Select B = 3.3 m

Bearing Pressure :-

qu = 4355.18/3.3*3.3= 399.92 Kn/m2

Design of Footing :-

Design of One Way Shear Strength :-

Critical Section at Distance )d( From The Face of Column

Assume h = 80cm , bar diameter ø 14 for main reinforcement      and 7.5 cm Cover

d = 800 – 75 – 14 = 711 mm

Vu = qu * B-a2-d*L

Vu = 399.92 * 3.3-0.62-0.711*3.3 = 843.31Kn

Design of Two Way Shear Strength :-

The punching shear strength is the smallest value of the following equations:

Where:-

= Perimeter of critical section taken at (d/2) from the loaded area

= 40  for interior column

ФVc =4566.4 Kn<Vu=3667.8Kn

Design of Bending Moment :-

Critical Section at the Face of Column

FR = qu * B-a2*L = 399.92 * 3.3-0.62 *3.3 =1781.6Kn

Mu = 1781.6*0.675 = 1202.58 Kn.m

Rn=Mu∅bd2=1202.58×1060.9×3300×7112=0.8Mpa

m=fy0.85fc'=4200.85×24=20.59

ρ=1m1-1-2.m.Rn420=120.591-1-2×20.59×0.8420=0.00194

As,req = ρ.b.d = 0.00194×3300×711 = 4551.82mm2

As,min = 0.0018*3300*800 = 4752 mm2

As,req < As,min = 4752 mm2 ,

As,min is control.

Check for Spacing :-

S = 3h = 3*80 = 240cm

S = 380*(28023 * 420) – 2.5*75 = 192.5 cm

S = 45 cm  ……… is control

Use 35ø14 in Both Direction, As,provided= 5033.7mm2>As,required= 4752 mm2…  Ok

Check for strain:

Tension = Compression

Design of Dowels :-

A1 = 60* 60 = 0.36 m2

A2 = 3.3* 3.3 = 10.89 m2

…………….

No Need For Dowels

As,min = 0.005 * Ac =  0.005 * 600* 600 = 1800 mm2

Use 12ø14, As,provided= 1847.25 mm2>As,required= 1800mm2…  Ok

Development Length In Footing  :-

Tension Development Length In Footing :-

LdT req=910*Fyλfc*ψeψsψtktr+cbdb*db<  300mm

Ktr=0 Nostripes

ktr+cbdb=2.5

LdT req=910*4201*24*1*1*0.82.5*14=345.7mm< 300mm

LdT available = 3300-6002 -75= 1275 mm

LdT available = 1275 mm >ldreq=345.7 mm…….. OK

Compression Development Length In Footing :-

LdCreq= 0.24*Fy*dB24< 0.043*Fy*dB  <200mm

LdCreq= 0.24*420*1424 = 288.05< 0.043*420*14 = 252.84<200mm

LdCreq= 288.05mm

Ldcavailable = 800 – 75 – 14 – 14 = 697mm <LdCreq= 288.0 mm …….. Ok

Figure 4-13:Foot reinforcement details.

4-10 Design of Stair.

# Figure 4-14: Stair plan.

Material:-

Concrete    B300                Fc' = 24 N/mm2

# Reinforcement Steel         fy = 420 N/mm2

# Determination the Thickness of Slab (flight and landing):

L = 4.4m

hreq = 4.4/ 20 =0.22m

Take h= 25cm.

- Use h = 25cm.

Rise = 15cm, run = 30cm

θ =  tan-1(riserun) =   tan-1(1530) = 26.57

Cos θ = 0.894

# Load Calculations at section:

Load on Flight:

Dead Load:

For 1m strip:

Flight = (25*0.20)/(Cos26.57) = 5.59KN/m.

Horizontal Mortar = 0.03*22*1 = 0.66 KN/ m.

vertical Mortar = 0.03*22*1*(15/30) = 0.33 KN/ m.

Plaster = (0.02*22)/ (Cos 26.57) = 0.49 KN/ m.

Horizontal tiles =23*0.04*(33/30) = 1.012KN / m.

Vertical tiles =23*0.03*(15/30) =0.345 KN/m

Triangle = 25*0.15*1*0.5 = 1.875KN/m

Total dead load   = 10.564 KN/ m.

Live load:

Live load for stairs =5 KN/ m2.

Factor Loads:

Qu=1.2*10.502+1.6*5 = 20.67 KN/m.

Figure 4-15: structural system of flight.

Au = 20.67*3.6*0.5 = 37.21 KN

Max Vu =37.21

Max Mu =(37.21×(0.4+1.5))-(20.67×(1.5*1.5 /2))

=47.4 KN.m

Design of Shear:

Assume Ø 12 for main reinforcement:-

So, d = 250-20 -12\2 = 224 mm

Max Vu = 37.21 KN.

Vu = 37.21 KN>= 137.2 KN.

No shear Reinforcement is required. So the depth of the stair is OK.

# Design of Bending Moment:

Max Mu = 61.4 kN.m

=mu0.9 =47.40.9  = 52.7 KN.m.

= 0.0026*1000*224 = 682.4mm2.

=

= 450mm≤= 682.4mm2

Use Φ 12@15cm

As provided =753.9 mm2>As req.

Step(s) is the smallest of :

3h = 3*250 = 750mm.

450mm

S= 380(280fs)- 2.5cc  = 380(280280)- 2.5*20 = 330mm .

S= 150mm < Smax

Check Strain:

T=C

As*fy = 0.85**b*a

420×753.9 = 0.85×24×1000×a

a=15.5 mm

x=aβ1 = 15.50.85 = 18.3mm                     × Note: fc' = 24 MPa< 28 MPa→ β1 = 0.85

εs=(d-xx)* 0.003

= (224-18.318.3)× 0.003 = 0.0337>  0.005

∴∅ = 0.9 ....  OK.

5- Lateral reinforcement:

As min = 4.5 cm2

Use Φ10 @ 20 cm

As = 4.74cm2/m

# Design of landing:

Load on landing:

Dead Load:

Slab = 0.25 *25*1 = 6.25 KN/ m.

Tiles = 0.03*23*1 = 0.69 KN/m.

Mortar = 0.02*22*1= 0.44 KN/ m.

Plaster = 0.03*22*1 = 0.66 KN/ m.

Sand = 17*0.08*1= 1.36 KN/m

Total dead load =8.15 KN/ m.

Live load:

Live load for stairs = 5 KN/ m.

Qu=1.2*8.15+1.6*5=17.78KN/m.

Au or Bu from Analysis:

Au = 37.21 KN

W = AUB = 37.212.15

W= 17.3KN/m

Figure 4-16: Structural system of landing.

Vu = (17.78×4.6/2) + (17.3×2.15)

Vu = 78.1KN.

Mu max =(78.1×2.3)-(17.78×2.3*2.3/2)-(17.3×2.15×1.225)

Mu max= 87.04 KN/m

Design of Shear for landing:

Assume Ø12 for main reinforcement:-

So, d = 250-20 -12\2 = 224 mm

Max Vu As the support reaction = 78.1KN.

Vu = 78.1KN>= 137.2 KN.

No shear Reinforcement is required. So the depth of the stair is OK.

# Design of Bending Moment for landing :

Max Mu = 87.04 kN.m

=mu0.9 =87.040.9  = 96.71 KN.m.

= 0.0048*1000*224= 1075.2mm2.

=

= 450mm2≤= 1075.2mm2

Use Φ 12@15cm

As provided =1130 mm2>As req.

Step(s) is thr smallest of :

3h = 3*250 = 750mm .

450mm

S= 380(280fs)- 2.5cc  = 380(280280)- 2.5*20 = 330mm .

S= 150mm < Smax

Check Strain:

T=C

As×fy = 0.85××b×a

420×1130= 0.85×24×1000×a

a=23.26 mm

x=aβ1 = 23.260.85 = 27.37 mm                     × Note: fc' = 24 MPa< 28 MPa→ β1 = 0.85

εs=(d-xx) × 0.003

= (224-27.3727.37) × 0.003 = .0215> 0.005

∴∅ = 0.9....  OK.

Lateral reinforcement:

As min = 4.5 cm2

Use Φ10 @ 20 cm

As = 5.53cm2/m

Figure14-17: Reinforcement for stairs.

4-11 Design of Basement Wall .

Figure 4-18: Geometry of basement.

Material:-

Concrete    B350                Fc' = 28 N/mm2

Reinforcement Steel         fy = 420 Mpa

∅=30°    γ=19.00KNm3

- Soil at rest

Ko=1-sin∅
       =1-sin30

=0.50

Load on basement wall:

Figure 4-19: system and loads of basement.

For 1m length of wall:

- Weight of backfill:

e=Ko*γ*h

= 0.50×19.0×3.625 =34.44KN/m

q1 (Factored) = 1.6×e

q1 (Factored) = 1.6 ×34.44= 55.1KN/m

- Load from live load:

LL=5 KN/m2

q2=Ko×LL

= 0.50 * 5 =2.50 KN/m

q2 (Factored) = 1.6 *2.50 =4.0 KN/m

Design of the shear force:

- Assume Ø14 for main reinforcement.

- Assume h = 300 mm,

d=300-20-14=266 mm

By using ATIR program, we get the envelope moment and shear force diagram

Figure 4-20 shear of basement.

Max Vu = 79.5 KN.

Vu = 79.5 KN > = 162.9 KN.

No shear Reinforcement is required.

Design of bending moment:

By using ATIR program, we get the envelope moment and moment force diagram

Figure 4-21 moment of basement.

Mu max =53.8 KN.m

Mn=Mu0.9=53.80.9=59.8 KN.m

Kn=Mn×106b×d2=59.8×1061000×2662=.845 Mpa

m=Fy0.85×fc'=4200.85×24=20.6

ρ=1m×1-1-2×kn×mFy

=120.60×1-1-2×0.845×20.6420

=2.06×10-3

Asreq=ρ×b×d=2.06×10-3×1000×266=5.4674 cm2/m

Asmin =0.0012×b×h=0.0012×1000×300=3.60 cm2/m

= 3.60 cm2/m ≤= 5.4674 cm2/m

Use Φ 14@20cm

As provided =6.16 cm2/m  >As req =5.4674 cm2/m.

Step(s) is the smallest of :

- 3h = 3*300 = 900mm .

- 450mm

- S= 380(280fs)- 2.5cc  = 380(280280)- 2.5*20 = 330mm .

S= 200mm < Smax

Select ∅14@20cm/m in one direction.

With as =6.16 cm2/m

Select ∅10@20cm/m in the other direction.

With as =3.92 cm2/m

Design of the horizontal reinforcement:

Asmin =0.0012*b*h=0.0012*1000*300=360cm2/m

Select∅10@20cm/m, in two layer.

Figure 4-22: Reinforcement for basement wall.

4-12 Design of Shear Wall (SW1, F2).

Figure 4-23: Moment and shear diagram for shear wall.

Material:-

Concrete    B300                Fc' = 24 N/mm2

Reinforcement Steel         fy = 420 N/mm2

h=30cm .shear wall thickness

Lw = 6.12m .shear wall width

Hw for one wall = 3.8 m story height

# *Design of shear

Design of the Horizontal reinforcement:

The critical Section is the smaller of:

Design as rectangular section:

d=4.9m, b=h=30cm

∅Vnmax=∅56fc'hd

=0.75×0.83×24×300×4900×10-3 =4482.93KN>Vu

# Vc is the smallest of :

1- Vc=16fc'hd=1624×300×4900×10-3=1200.2KN

2- Vc=0.25fc'hd+Nud4lw=0.2524×300×4900×10-3+0=1800.4KN

3- Vc=0.5fc+lw0.1fc'+0.2NulwhMuVu-lw2hd 10    =0.524+6.120.124+03300×490010=830.4KN……cont

6172.5-4789.53.8=Mu-4789.53.8-3.06⟹Mu=5058.8KN.m

MuVu-lw2=5058.8840-6.122=3

Vu = 840KN>0.75*830.4 = 622.8KN.       ‘Horizontal reinforcement is required’

∅Vc +∅Vs=Vu

Vs = Vu∅-Vc

Vs =289.6

Avhs=Vsfy×d

Avhs=289.6*10^3420×4900=0.14

(Avhs)min= 0.0025×h=0.0025×300=0.75>0.14…cont

Maximum spacing is the least of:

lw5 =  = 1224mm

3×h = 3×300 = 900mm

450 mm ……. Control

Try ϕ12 (As = 113.09 mm2) for two layers

=  =  = 0.75

S2 = 301.57 mm   ,   select ϕ12@250 mm

→useϕ12@250 mm in two layer

# Design of uniform Vertical reinforcement:-

ρvmin>0.0025+0.52.5-hwlρt-0.0025>0.0025

For this wall with = 4.3 > 2.5

Avvsv = 0.0025+0.52.5-hwlAvhs×h-0.0025×h

Avvsv = 0.82

Select Ф 12@200mm.  In two layer

-Maximum spacing is the least of :

lw5 =  = 1224mm

3×h = 3×300 = 900mm

450 mm ……. Control

Select Ф 12@200mm In tow layer

# Design of bending moment (vertical steel in boundary) :

# Asv=6120200×226.2=6921.72mm2

# w=AstLwhfyfc'=6921.726120*30042024=0.066

α=Pulwhfc'=0

Clw=w+α2w+0.85β1=0.066+02×0.066+0.85×0.85=0.077

∅Mn=∅0.5Asvfylw(1+PuAstfy)(1-clw)

=0.90.5×6921.72×420×6120(1+0)(1-0.077)=7389.7KN.m>Mu.

Select Ф 12@200mm for vertical reinforcement.

4-13 Column Coordinates.

| Column NO. | X-AXIS | Y-AXIS |
| --- | --- | --- |
| C1 | 33.23 | 48.48 |
| C2 | 26.54 | 48.48 |
| C3 | 21.2 | 48.48 |
| C4 | 51.73 | 43.95 |
| C5 | 45.97 | 43.95 |
| C6 | 40 | 43.95 |
| C7 | 33.23 | 43.95 |
| C8 | 25.94 | 43.95 |
| C9 | 20.37 | 43.95 |
| C10 | 16.7 | 43.95 |
| C11 | 51.73 | 39.67 |
| C12 | 45.97 | 39.67 |
| C13 | 40 | 39.67 |
| C14 | 33.38 | 39.67 |
| C15 | 26.54 | 39.67 |
| C16 | 20.37 | 40.13 |
| C17 | 13.4 | 40.9 |
| C18 | 51.73 | 30 |
| C19 | 45.97 | 34.2 |
| C20 | 40 | 34.2 |
| C21 | 34.45 | 34.2 |
| C22 | 27.15 | 34.2 |
| C23 | 20.37 | 34.13 |
| C24 | 17.38 | 36.38 |
| C25 | 23.5 | 30.8 |
| C26 | 51.73 | 25.72 |
| C27 | 45.97 | 27.43 |
| C28 | 40 | 27.43 |
| C29 | 34.45 | 27.43 |
| C30 | 27.15 | 27.43 |
| C31 | 30.32 | 24 |
| C32 | 51.86 | 20.39 |
| C33 | 45.97 | 20.39 |
| C34 | 40 | 20.39 |
| C35 | 33.76 | 20.6 |
| C36 | 33.38 | 20.2 |
| C37 | 29.95 | 23.63 |
| C38 | 26.54 | 27 |
| C39 | 23.11 | 30.47 |
| C40 | 19.84 | 33.6 |
| C41 | 17.1 | 36.5 |
| C42 | 12.97 | 40.46 |
| C43 | 9.16 | 36.5 |
| C44 | 6.63 | 33.95 |
| C45 | 2.1 | 29.56 |
| C46 | 12.97 | 32.5 |
| C47 | 10.34 | 30 |
| C48 | 6.03 | 25.4 |
| C49 | 8.62 | 22.55 |
| C50 | 4.5 | 18.52 |
| C51 | 0.44 | 13.98 |
| C52 | 13.4 | 17.97 |
| C53 | 9.16 | 13.74 |
| C54 | 4.5 | 9.22 |
| C55 | 7.72 | 6.46 |
| C56 | 10.34 | 3.82 |
| C57 | 14.64 | 8.27 |
| C58 | 18.88 | 12.5 |
| C59 | 23.5 | 17.11 |
| C60 | 25.94 | 19.6 |
| C61 | 29.59 | 15.94 |
| C62 | 33.23 | 12.1 |
| C63 | 27.15 | 13.43 |
| C64 | 22.57 | 8.81 |
| C65 | 18.33 | 4.58 |
| C66 | 13.95 | 0.46 |
| C67 | 74.4 | 4.58 |
| C68 | 69.1 | 4.58 |
| C69 | 63.5 | 4.58 |
| C70 | 57.5 | 4.58 |
| C71 | 52.33 | 4.58 |
| C72 | 52.28 | 10.87 |
| C73 | 57.5 | 10.87 |
| C74 | 63.5 | 10.87 |
| C75 | 69.1 | 10.87 |
| C76 | 73.96 | 10.87 |
| C77 | 74.4 | 15.97 |
| C78 | 69.27 | 15.97 |
| C79 | 63.5 | 15.97 |
| C80 | 57.5 | 15.97 |
| C81 | 52.33 | 20.39 |
| C82 | 57.5 | 20.39 |
| C83 | 60.45 | 20.39 |
| C84 | 63.5 | 20.39 |
| C85 | 69.1 | 20.39 |
| C86 | 73.96 | 20.39 |
| C87 | 74.4 | 25.72 |
| C88 | 70.51 | 25.72 |
| C89 | 66.2 | 25.72 |
| C90 | 61.8 | 25.72 |
| C91 | 57.5 | 25.72 |
| C92 | 52.33 | 25.72 |
| C93 | 61.95 | 30 |
| C94 | 58.23 | 33.8 |
| C95 | 55.95 | 36 |
| C96 | 52.28 | 39.67 |
| C97 | 52.28 | 30 |
| C98 | 70.51 | 30.23 |
| C99 | 66.55 | 34.32 |
| C100 | 62.6 | 38.16 |
| C101 | 59.69 | 40.9 |
| C102 | 56.7 | 43.95 |
| C103 | 52.18 | 43.95 |
| C104 | 52.18 | 48.48 |
| C105 | 52.28 | 52.24 |
| C106 | 56.5 | 52.24 |
| C107 | 60.96 | 47.96 |
| C108 | 63.64 | 45 |
| C109 | 66.55 | 42.1 |
| C110 | 70.27 | 38.38 |
| C111 | 74.56 | 30.8 |
| C112 | 73.96 | 34.32 |
| C113 | 73.96 | 42.6 |
| C114 | 70.76 | 46.3 |
| C115 | 67.84 | 49.32 |
| C116 | 64.9 | 52.24 |
| C117 | 64.9 | 52.24 |
| C118 | 69.65 | 52.24 |
| C119 | 74.56 | 47.34 |
| C120 | 69.65 | 56.91 |
| C121 | 73.96 | 60.89 |
| C122 | 76.57 | 57.95 |
| C123 | 97.52 | 55.1 |
| C124 | 83.2 | 51.32 |
| C125 | 87.41 | 47.18 |
| C126 | 83.43 | 43.13 |
| C127 | 78.67 | 38.38 |
| C128 | 79.22 | 47.34 |
| C129 | 75.85 | 51 |
| C130 | 51.73 | 34.32 |
| C131 | 52.18 | 34.32 |

Table 4-5: Column coordinates.