Chapter 4

Structural Analysis And Design

4.1 Introduction.

4.2 Design method and requirements.

4.3 Comparison Between the thickness of one way Rib slab and Two Rib slab

4.4 Design of topping.

4.5 Pos (R 1): Design of one way Rib slab.

4.6 Pos. (R 9): Design of two way Rib slab.

4.7 Pos. (B 3): Design of Beam.

4.1 Introduction:

Many structures are built of reinforced concrete: bridges, buildings, retaining walls, tunnels, and others.

Reinforced concrete is logical union of two materials: plain concrete, which possesses high compressive strength but little tensile strength, and steel bars embedded in the concrete, which can provide the needed strength in tension.

Plain concrete is made by mixing cement, fine aggregate, coarse aggregate, water, and frequently admixtures.

Understanding of reinforced concrete behavior is still far from complete, building codes and specifications that give design procedures are continually changing to reflect latest knowledge.

Structural concrete can be classified into:

- Lightweight concrete with unit weight from about 1350 to 1850 kg/m3.

- Normal weight concrete with unit weight from about 1800 to 2400 kg/m3.

- Heavyweight concrete with unit weight from about 3200 to 5600 kg/m3.

4.2 Design method and requirements:

The design strength provided by a member is calculated in accordance with the requirements and assumptions of ACI_code (318_08).

- Strength design method:

In ultimate strength design method, the service loads are increased by factors to obtain the load at which failure is considered to be occurring.

This load called factored load or factored service load. The structure or structural element is then proportioned such that the strength is reached when factored load is acting. The computation of this strength takes into account the nonlinear stress-strain behavior of concrete.

The strength design method is expressed by the following,

Strength provided ≥ strength required to carry factored loads.

NOTE:

The statically calculation and the key plans dependent on the architectural plans.

- Code :    ACI 2008

UBC

- Material :

Concrete:    B300…. For circular section

but for rectangular section () .

Reinforcement steel :   The specified yield strength of the reinforcement {fy = 420 N/mm² (MPa)}

Mild steel :    A-36

Connection Type :     Weld , Bolts

- Factored loads:

The factored loads for members in our project are determined by:

Wu = 1.2 DL + 1.6 LL                   ACI-code-318-08(9.2.1).

4.3 Comparison between the thickness of one way rib slab and two way rib slab :

*Check Thickness of one way rib slab

Fig 4.1: One Way Rib slab (R 1)

- Statically system for (R 1) :

The minimum required thickness is:

L16= 4000 16 = 250 mm                       for simply supported.

*Check Thickness of two way rib slab:

Fig 4.2: Two way Rib slab (R 9)

- Statically system for (R 9) :

Rib slab (R 9) :

Yc =0.55*0.08*0.04+0.15*0.24*0.20.55*0.08 +  0.15*0.24  = 11.2 cm

Irib =0.55*0.08312     +0.55*0.08*0.0722 +   0.15*0.24312   + 0.15*0.24*0.0882 = 7.032 * 104  cm4

T Beam (B,G28 / B,G31):

Yc =32*80*16+28*50*4680*32 +  28*50  = 26.61 cm

Ib = 80*32312 + 80*32*10.612 +  50*28312   + 50*28*19.392 = 112.447 *10 4  cm4

Rectangular Beam (B,G49) :

Ib = 30*603/ 12  =   54 *10 4  cm4

T Beam (B,G43) :

Yc =32*80*16+28*50*4680*32 +  28*50  = 26.61 cm

Ib = 80*32312 + 80*32*10.612 +  50*28312   + 50*28*19.392 = 112.447 *10 4  cm4

External:

Long direction L = 700 cm

Is = 10252+8055*7.032 * 104  =757538.1818 cm4

Internal :

In short direction L = 7 cm

Is = 10252+712.662+8055*7.032 * 104=1213122.284 cm4

In long direction L = 10.25 cm

Is = 7002+6452+62.555*7.032 * 104=939730.9091 cm4

Is = 7002+6452+8055*7.032 * 104=962105.4545 cm4

α 1 =  54*104757538.1818    = 0.713

α2 = 112.447 *104 939730.9091  = 1.1966

α3 = 112.447 *1041213122.284   = 0.9269

α4 = 112.447 *104 962105.4545  = 1.1688

αfm = ∝1+∝2+∝3+∝44= 1.0013 < 2                                  β = 10.257 = 1.464

h=1025 (0.8+ 4201400 )36+5*1.464*(1.002-0.2) = 269.264  >  hmin =125

The thickness of one way rib slab is smaller than in two way rib slab SO ,

Take the slab thickness = 32 cm, 24 cm for concrete block , 8 cm, for topping.

4.4 Design of topping:

- Statically system for topping :

Consider the topping as strip of (1m) width, and span of mold length with both end fixed in the ribs.

Fig 4.3: topping load.

- Load calculations:

Dead load calculations:

| Dead load from: | δ×γ×1 | KN/m |
| --- | --- | --- |
| Tiles | 0.03×23×1 | 0.69 |
| Mortar | 0.03×22×1 | 0.66 |
| Coarse sand | 0.07×16×1 | 1.12 |
| Topping | 0.08×25×1 | 2 |
| Interior partitions | 1.5×1 | 1.5 |
|  | ∑ | 5.97 |

Live load :

LL =5 KN/m2 	 LL =5 KN/m2 × 1m= 5 KN/m

Factored load :

WU = 1.2 ×5.97 + 1.6×5 =15.164 KN/m.

Check the strength condition for plain concrete, øMn ≥ Mu, where ø = 0.55.

Mn = 0.42 λ fc' Sm                        (ACI 22.5.1, equation 22-2)

Sm=b.h26=1000×806=1066666.67 mm2 .

øMn =0.55×1×24 ×1066666.67 ×10-6 =1.232 KN.m

Mu = WuL212=0.202 KN.m                                   (negative moment).

Mu = WuL224=0.101 KN.m                                 (positive moment).

øMn >> Mu =  0.202 KN.m

No reinforcement is required by analysis. According ACI 10.5.4, provide As,min for slabs as shrinkage and temperature reinforcement.

ρshrinkage = 0.0018                                               ACI 7.12.2.1

As = ρ×b×htopping  =0.0018 ×1000×80 = 144 mm2/m .

Step (s) is the smallest of:

- 3h = 3×80 =240 mm.         control                                               ACI 10.5.4

- 450mm.

- S =380280fs-2.5Cc=380 28023420-2.5 .20=330mm                           but

S≤ 300280fs=30028023420=300mm                                     ACI 10.6.4

Take ø 8 @ 200 mm in both direction , S = 200 mm < Smax = 240 mm … OK

4.5 Po. (R 1): Design of one way Rib slab .

- Requirements For Ribbed Slab Floor According to      ACI- (318-08) .

bw ≥ 10cm……………………………………………ACI(8.13.2)

Select bw=15cm

h ≤ 3.5*bw  ……………………………….…….. ACI(8.13.2)

Select h=32cm<3.5*15=52.5cm

tf ≥ Ln/12≥50mm ………………………………….ACI(8.13.6.1)

Select tf=8cm

- The effective flange width (be), according to ACI 8.12.2 is the smallest of:

- be ≤ L4=4000-8004=800 mm           L, is the clear span of the rib.

- be  ≤ bw +16hf  = 150+16×80 = 1430 mm.

- be ≤ center to center spacing between adjacent beams = 550 mm.                  Control

- Statically system and Dimensions.

Fig 4.4: One way Rib slab (R 1).

- Load calculations:

Dead load calculations:

| Dead load from: | δ×γ×be | KN/m |
| --- | --- | --- |
| Tiles | 0.03×23×0.55 | 0.3795 |
| Mortar | 0.03×22×0.55 | 0.363 |
| Coarse sand | 0.07×16×0.55 | 0.616 |
| Topping | 0.08×25×0.55 | 1.1 |
| Interior partitions | 1.5×0.55 | 0.825 |
| RC rib | 0.24×25×0.15 | 0.9 |
| Hollow Block | 0.24×10×0.4 | 0.96 |
| Plaster | 0.03×22×0.55 | 0.363 |
|  | ∑ | 5.5065 |

Live load :

Live load /rib = 5 KN/m2 × 0.55m = 2.75 KN/m.

Factored load :

Du = 1.2 ×5.5065 = 6.61 KN/m.

Lu = 1.6 ×2.75 = 4.4 KN/m. m

Wu = 11.01 KN/m.

- Flexural Design for (R 1):

Moment for R1:

Mu = WuL28=22.02 KN.m

Fig 4.5: loads of rib (R 1)

Design of positive moment.

Mu = 22.02 KN.m.

Assume bar diameter ø 12 for main positive reinforcement.

d =h- cover - dstirrups -db2=320-20-10-122=284 mm.

Check if  a > hf  to determine whether the section will act as rectangular or T- section,

Mnf =0.85. fc'.be.hf.(d-hf2)

= 0.85×24×550×80×284-802×10-6=219.014 KN.m

Mnf ≫ Muφ = 24.47 KN.m, the section will be designed as rectangular section with  be = b = 550 mm.

Rn=Mu∅bd2=22.02×1060.9×550×2842=0.552 Mpa.

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420  =120.61-1-2×20.6×0.552420  =0.00133

As,req = ρ.b.d = 0.00133 ×550×284 = 207.97 mm2

*Check for As,min .

As,min is the maximum of :-

As,min =0.25f'cfy bw.d≥1.4fybw.d

- As,min =0.2524420 150×284=124.22 mm2

- As,min = 1.4420150×284=142 mm2                        Control

As,required = 207.97  mm2.

Use 3 ø 10 , As,provided = 235.5 mm2 >  As,required = 207.97  mm2.         Ok

S =150-40-20-(2×10)1=70 mm>db=10>25  mm        OK

Check for strain:

a =As.fy0.85b fc'=235.5×4200.85×550×24=8.82 mm

c=aB1=8.820.85=10.37 mm

εs=0.003d-cc=0.003284-10.3710.37=0.079>0.005                0k

For the top reinforcement use 2 ø 10 as Montage.

- Shear Design for (R 1):

Fig 4.6: loads of rib (shear R 1)

wuD = 1.2 ×5.5065 = 6.61 KN/m.                                 wuL = 1.6 ×2.75 = 4.4 KN/m

Vu at the face of support = wl2=11.01*42=22.02 KN

Vu at midspan = wuLl8=4.4*48=2.2 KN

Vu  at d = 284 mm from the face of support.

22.02-2.22.2=y2.2- 0.284               y = 17.26 KN

Vu = y+2.2=19.46 KN

Shear strength Vc, provided by concrete for the joists may be taken 10% greater that for beams. This is mainly due to the interaction between the slab and closely spaced ribs. (ACI, 8.13.8).

Vc =1.16 λ fc' bwd=1.1624 ×150×284×10-3=38.26 KN

ø Vc =0.75×38.26 =28.69 KN.

0.5 ø Vc =0.5×0.75×38.26 =14.35 KN

0.5 ø Vc < Vu < ø Vc

Case (2) for shear design, minimum shear reinforcement is required (Av,min), exception for Ribbed slab , No shear Reinforcement .

Use stirrups U-shape as montage (2 leg stirrups ) ø 8 @ 250 mm , Av = 2×  50.24 = 100.48 mm2.

Av=2*50.30.25=401.92 mm2/mstrip

4.6 Pos. (R 9): Design of two way Rib slab .

Fig. (4-5): Two way ribbed slab (R 9)

- Statically system and Dimensions.

- Load calculations:

Dead load calculations:

| Dead load from: | W = γ×V | KN |
| --- | --- | --- |
| Tiles | 0.03×23×0.552 | 0.2087 |
| Mortar | 0.03×22×0.552 | 0.1997 |
| Coarse sand | 0.07×16×0.552 | 0.3388 |
| Topping | 0.08×25×0.552 | 0.605 |
| Interior partitions | 1.5×0.552 | 0.454 |
| RC rib | 0.24×25×0.15×(0.55+0.4) | 0.855 |
| Hollow Block | 0.24×10×0.4×0.4 | 0.384 |
| Plaster | 0.03×22×0.552 | 0.19965 |
|  | ∑ | 3.245 |

Table (4.2) Calculation of two way dead load (R 9)

Nominal Total Dead Load = 3.245 KN/Rib

DL= 3.245/(0.552) = 10.73 KN/m2

Nominal Total live load = 5 KN/m2

Determination of factored dead & live load

Factored dead load = 1.2*Dead load = 1.2*10.73 = 12.87  KN/m2.

Factored Live load = 1.6*live load = 1.6*5 = 8 KN/m2.

W = 12.87 + 8 = 20.87 KN/m2

- Flexural Design for (R 9) :

Moments calculations :-

Ma = Ca wla2 brib   and     Mb = Cb wlb2 brib

La/Lb = 7/10.25 = 0.683 ………..Case 9

The moment calculation will be done for the slab middle strip.

*Negative moments at continuous edge :

Ca,neg (la/lb=0.70) = 0.081                      Ca,neg (la/lb=0.65) = 0.083

Ca,neg (la/lb=0.683) = 0.08168

Ma-ve=Ca*W*La2*brib=0.08168*20.87*7 2*0.55= 45.95 KN.m/Rib

Cb,neg (la/lb=0.70) = 0.011                      Cb,neg (la/lb=0.65) = 0.008

Cb,neg (la/lb=0.683) = 0.00998

Mb-ve=Cb*W* Lb2*brib =0.00998*20.87*10.25 2*0.55= 12.04 KN.m/Rib

*Positive moments :

Ca,D (la/lb=0.70) = 0.033                      Ca,D (la/lb=0.65) = 0.034

Ca,D (la/lb=0.683) = 0.03334

Ma+ve,D=Ca*W* La2*brib =0.03334*12.87*7 2*0.55= 11.56 KN.m/Rib

Ca,L(la/lb=0.70) = 0.05                      Ca,L (la/lb=0.65) = 0.054

Ca,L (la/lb=0.683) = 0.05136

Ma+ve,L=Ca*W* La2*brib =0.05136*8*7 2*0.55= 11.07 KN.m/Rib

Ma+ve = Ma+ve,L + Ma+ve,D = 22.634 KN.m/Rib

Cb,D (la/lb=0.70) = 0.006                      Cb,D (la/lb=0.65) = 0.005

Cb,D (la/lb=0.683) = 0.00566

Mb+ve,D=Cb*W* Lb2*brib =0.00566*12.87*10.25 2*0.55= 4.21 KN.m/Rib

Cb,L(la/lb=0.70) = 0.011                      Cb,L (la/lb=0.65) = 0.009

Cb,L (la/lb=0.683) = 0.01032

Mb+ve,L=Cb*W* Lb2*brib =0.01032*8*10.25 2*0.55= 4.77 KN.m/Rib

Mb+ve = Mb+ve,L + Mb+ve,D = 8.98 KN.m/Rib

*Negative moments at Discontinuous edge (1/3 * positive moments):

Design for Negative and Positive moment:

* Short direction

d = 320 – 20 – 8 –  = 285 mm

Positive Moment:

Midspan: (Mu = +22.634 KN.m/Rib )

Rn=Mu∅bd2=22.634×1060.9×150×2852=2.064 Mpa.

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420  =120.61-1-2×20.6×2.064420  =0.00519

As = ρ.b.d = 0.00519 ×150×285 = 221.97 mm2.                Control.

Check for As,min .

As,min =0.25f'cfy bw.d≥1.4fybw.d

As,min =0.2524420 150×285=124.66 mm2

As,min = 1.4420150×285=142.5 mm2

As,required = 221.97 mm2.

Use 2 ø 12 Bottom, As,provided = 226.08 mm2 >  As,required = 221.97 mm2.         Ok

Check spacing :

S =150-40-20-(2×12)1=66 mm>db=12>25       OK

Check for strain:

a =As.fy0.85b fc'=226.08×4200.85×150×24=31.03 mm

c=aB1=31.030.85=36.5 mm

εs=0.003d-cc=0.003285-36.536.5=0.0204>0.005                0k

Negative Moment:

Continuous edge : (Mu = - 45.95 KN.m/Rib )

Rn=Mu∅bd2=45.95×1060.9×150×2852=4.19 Mpa.

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420  =120.61-1-2×20.6×4.19420  =0.0113

As = ρ.b.d = 0.0113 ×150×285 = 482.66 mm2.                Control.

Use 2 ø 18 Top , As,provided = 508.68 mm2 >  As,required = 482.66 mm2.         Ok

Check spacing :

S =150-40-20-(2×18)1=54 mm>db=18>25       OK

Check for strain:

a =As.fy0.85b fc'=508.68×4200.85×150×24=69.82 mm

c=aB1=69.820.85=82.14 mm

εs=0.003d-cc=0.003285-82.1482.14=0.0074>0.005                0k

* Long direction

d = 320 – 20 – 8 –  = 285 mm

Positive Moment:

Midspan: (Mu = +8.98 KN.m/Rib )

Rn=Mu∅bd2=8.98×1060.9×150×2852=0.819 Mpa.

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420  =120.61-1-2×20.6×0.819420  =0.00199

As = ρ.b.d = 0.00199 ×150×285 = 85.1 mm2.

Check for As,min .

As,min =0.25f'cfy bw.d≥1.4fybw.d

As,min =0.2524420 150×285=124.66 mm2

As,min = 1.4420150×285=142.5 mm2            Control.

As,required = 142.5 mm2.

Use 2 ø 10 Bottom, As,provided = 157 mm2 >  As,required = 142.5 mm2.         Ok

Check spacing :

S =150-40-20-(2×10)1=70 mm>db=10>25       OK

Check for strain:

a =As.fy0.85b fc'=157×4200.85×150×24=21.55 mm

c=aB1=21.550.85=25.35 mm

εs=0.003d-cc=0.003285-25.3525.35=0.0307>0.005                0k

Negative Moment :

Continuous edge : (Mu = - 12.04 KN.m/Rib )

Rn=Mu∅bd2=12.04×1060.9×150×2852=1.2 Mpa.

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420  =120.61-1-2×20.6×1.2420  =0.00269

As = ρ.b.d = 0.00269 ×150×285 = 114.94 mm2.

Check for As,min .

As,min =0.25f'cfy bw.d≥1.4fybw.d

As,min =0.2524420 150×285=124.66 mm2

As,min = 1.4420150×285=142.5 mm2            Control.

As,required = 142.5 mm2.

Use 2 ø 10 Top, As,provided = 157 mm2 >  As,required = 142.5 mm2.         Ok

Check spacing :

S =150-40-20-(2×10)1=70 mm>db=10>25       OK

Check for strain:

a =As.fy0.85b fc'=157×4200.85×150×24=21.55 mm

c=aB1=21.550.85=25.35 mm

εs=0.003d-cc=0.003285-25.3525.35=0.0307>0.005                0k

- Shear Design for (R 9):

Wa (la/lb=0.70) = 0.89                      Wa (la/lb=0.65) = 0.92

Wa (la/lb=0.683) = 0.9002

- The total load on the panel being ( 7×10.25×20.87 = 1497.423 KN)

- The load per rib at face of the long beam is (0.9002×1497.423×0.55/(2×10.25)=36.165 KN)

Vud = 36.165 – 20.87×0.55×0.285 = 32.89 KN

Vc =1.16ʎfc' bwd=1.1624 ×150×285×10-3=38.4 KN

ø Vc =0.75×38.4 =28.8 KN.

0.5 ø Vc =0.5×0.75×38.4 =14.4 KN

Vs,min =13 150×285×10-3=14.25 KN           Control

Vs,min = 11624 ×150×285×10-3=13.09 KN

ø  Vc < Vu < ø ( Vc + Vs,min )

Case (3) for shear Design : Minimum shear reinforcement

Use stirrups U-shape (2 leg stirrups ) ø 10 Av = 2×  78.5 = 157 mm2.

Smax≤d2≤600 mm                                         d2=2842=142 mm         Control.

Sreq=3 Av fyt bw=157×420×3150=1318.8 mm> Smax take S=Smax=142 mm

Sreq=16 Av fyt bwfc'=157×420×16150 24=1435.73 mm> Smax take S=Smax=142 mm

Use 2-Leg ø 10 @ 140 mm , and 2-Leg ø 10 @ 200 mm in the middle space.

4.7 Pos. (B 3): Design of Beam .

- Load calculations:

Dead load calculations:

| Dead load from: | δ×γ×be | KN/m |
| --- | --- | --- |
| Tiles | 0.03×23×0.8 | 0.552 |
| Mortar | 0.03×22×0.8 | 0.528 |
| Coarse sand | 0.07×16×0.8 | 0.986 |
| Interior partitions | 1.5×0.8 | 1.2 |
| Beam 3 | ((0.32×0.8)+(0.5×0.28))×25 | 9.9 |
| Plaster | 0.03×22×1.36 | 0.8976 |
|  | ∑ | 14.06 |

Nominal Total live load = 5 * 0.8 = 4 KN/m

Dead load from two way = 10.73 KN/m2

Live Load = 5 KN/m2

- load calculations for ( R6) :

( DL =10.73 ×2.95 =  31.65 KN/m   ,  LL = 5×2.95 = 14.8 KN/m)

- load calculations for ( R7) :

( DL =10.73 ×3.4 =  36.48 KN/m   ,  LL = 5×3.4 = 17 KN/m)

- load calculations for ( R1) :

( DL =5.5 × 2.75/ (2×0.55)=  13.75 KN/m   ,  LL = 5×2.75/ 2 = 6.875 KN/m)

- load calculations for (Parallel R1) :

( DL =5.5 × 1/ 0.55 =  10 KN/m   ,  LL = 5×1 = 5 KN/m)

- Statically system and Dimensions.

- Flexural Design for (B 3) :

Determine of  Mn,max :

d = 600 – 40 -10 – 18\2 = 541 mm

a = hf = 320 mm

Mn,max= 0.85 * fc * hf * b *(d-hf/2) = 0.85* 24 * 350 *800* (541-320/2) * 10 -6 = 2176.27 KN .m

∅Mn ,max = 0.9* 2176.27= 1958.645 KN.m

Design for positive moment :

1) Mu = 416.7 KN.m < 1958.645 KN.m     (Design as rectangular)

Rn=Mu∅bd2=416.7×1060.9×800×5412=1.977 Mpa.

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420  =120.61-1-2×20.6×1.977420  =0.00496

As = ρ.b.d = 0.00496 ×800×541 = 2147.42 mm2.

Check for As,min .

As,min =0.25f'cfy bw.d≥1.4fybw.d

As,min =0.2524420 500×541=788.79 mm2

As,min = 1.4420500×541=901.667 mm2            Control.

As,min = 901.667 mm2 < As = 2147.42 mm2

Use 7 ø 20 Bottom, As,provided = 2198 mm2 >  As,required = 2147.42 mm2.         Ok

Check spacing :

S =500-40*2-20-(7×20)6=43.33 mm>db=18>25       OK

Check for strain:

a =As.fy0.85b fc'=2198 ×4200.85×800×24=56.57 mm

c=aB1=56.570.85=66.55 mm

εs=0.003d-cc=0.003541-66.5566.55=0.0214>0.005                0k

2) Mu = 294.1 KN.m < 1958.645 KN.m     (Design as rectangular)

Rn=Mu∅bd2=294.1×1060.9×800×5412=1.396 Mpa.

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420  =120.61-1-2×20.6×1.396420  =0.00345

As = ρ.b.d = 0.00345 ×800×541 = 1491.49 mm2.

Check for As,min .

As,min =0.25f'cfy bw.d≥1.4fybw.d

As,min =0.2524420 500×541=788.79 mm2

As,min = 1.4420500×541=901.667 mm2            Control.

As,min = 901.667 mm2 < As = 1491.49 mm2

Use 5 ø 20 Bottom, As,provided = 1570  mm2 >  As,required = 1491.49 mm2.         Ok

Check spacing :

S =500-40*2-20-(5×20)4=75 mm>db=18>25       OK

Check for strain:

a =As.fy0.85b fc'=1570 ×4200.85×800×24=40.41 mm

c=aB1=40.410.85=47.53 mm

εs=0.003d-cc=0.003541-47.5347.53=0.0311>0.005                0k

Design for Negative moment :

1) Mu = - 421 KN.m

Rn=Mu∅bd2=421×1060.9×800×5412=1.998 Mpa.

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420  =120.61-1-2×20.6×1.998420  =0.005

As = ρ.b.d = 0.005 ×800×541 = 2170.85 mm2.

Check for As,min .

As,min =0.25f'cfy bw.d≥1.4fybw.d

As,min =0.2524420 500×541=788.79 mm2

As,min = 1.4420500×541=901.667 mm2            Control.

As,min = 901.667 mm2 < As = 2170.85 mm2

Use 7 ø 20 Bottom, As,provided = 2198 mm2 >  As,required = 2170.85 mm2.         Ok

Check spacing :

S =500-40*2-20-(7×20)6=43.33 mm>db=18>25       OK

Check for strain:

a =As.fy0.85b fc'=2198 ×4200.85×800×24=56.57 mm

c=aB1=56.570.85=66.55 mm

εs=0.003d-cc=0.003541-66.5566.55=0.0214>0.005                0k

2) Mu = - 498.1 KN.m

Rn=Mu∅bd2=498.1×1060.9×800×5412=2.36 Mpa.

m=fy0.85fc'=4200.85×24=20.6

ρ=1m1-1-2.m.Rn420  =120.61-1-2×20.6×2.36420  =0.005998

As = ρ.b.d = 0.005998 ×800×541 = 2596.12 mm2.

Check for As,min .

As,min =0.25f'cfy bw.d≥1.4fybw.d

As,min =0.2524420 500×541=788.79 mm2

As,min = 1.4420500×541=901.667 mm2            Control.

As,min = 901.667 mm2 < As = 2596.12 mm2

Use 8 ø 20 Top, As,provided = 2512 mm2

Check spacing :

S =500-40*2-20-(8×20)7=34.29 mm>db=20>25       OK

Check for strain:

a =As.fy0.85b fc'=2512 ×4200.85×800×24=64.65 mm

c=aB1=64.650.85=76.055 mm

εs=0.003d-cc=0.003541-76.05576.055=0.0183>0.005                0k

- Shear Design for (B 3):

- Vu = 337.9 KN

Vc = 16fc'bw d= =  1624*500*541= 220.86 KN

Φ Vc= 0.75*220.86 =165.65 KN

vs,min= 116fc'bw d

vs,min= 13bw d = = 13 500* 541= 90.167 KN        control

vs'= 13fc'bw d = 1324* 500*541  = 441.72 KN

∅(vc+ vs,min⁡)<vu ≤ ∅(vc+ vs')              case 4

0.75(220.86 + 90.167)  <   337.9  <  0.75(220.86 + 441.72)

shear reinforcement are required .

Use 2 leg Φ 10 .

As =157 mm2 .

Vs = Vn – Vc = 337.90.75 – 220.86 = 229.67 KN

S= Avfyt dvs=157*420*541229.67*1000=155.3 mm             (control)

smax ≤ d2=5412= 270.5 mm                                                 or smax ≤600 mm

Use 2 leg Φ 10  @ 150 mm .

- Vu = 201.4 KN

Vc = 16fc'bw d= =  1624*500*541= 220.86 KN

Φ Vc= 0.75*220.86 =165.65 KN

vs,min= 116fc'bw d

vs,min= 13bw d = = 13 500* 541= 90.167 KN        control

∅vc<vu ≤ ∅(vc+ vs,min)              case 3

0.75 * 220.86 <   201.4  <  0.75(220.86 + 90.167)

shear reinforcement are required .

Use 2 leg Φ 10 .

As =157 mm2 .

smax ≤ d2=5412= 270.5 mm           (control)                                      or smax ≤600 mm

Sreq=3 Av fyt bw=157×420×380=2472.75 mm> Smax take S=Smax=270.5 mm

Sreq=16 Av fyt bwfc'=157×420×1680 24=2691.99 mm> Smax take S=Smax=270.5 mm

Use 2 leg Φ 10  @ 250 mm .

- Vu = 158 KN

Vc = 16fc'bw d= =  1624*500*541= 220.86 KN

Φ Vc= 0.75*220.86 =165.65 KN

0.5 ø Vc =0.5×165.65 =82.825 KN

0.5 ø Vc < Vu < ø Vc

Case (2) for shear design, minimum shear reinforcement is required (Av,min), exception for Ribbed slab , No shear Reinforcement .

Use stirrups U-shape as montage (2 leg stirrups ) ø 10 @ 250 mm , Av = 2×  78.5 = 157 mm2