| Vertical Alignment Report PVI Stations and Curves | Data generated:. 4/17/2011 9:38:42 PM. |
| --- | --- |

| Project: mohammad |  |
| --- | --- |

Units: meter

| Horizontal Alignment Information |  |
| --- | --- |
| Name: | CL |
| Station Range: | 0+000 to 2+068.385 |
| Station Equations: | None |

Curve Calculation Options 

Vertical Alignment: Center FG

| PVI | Station | Elevation | Grade Out (%) | Curve Length |
| --- | --- | --- | --- | --- |
| 1 | 0+000 | 755.598 | -0.001 |  |
| 2 | 0+280.027 | 755.595 | 3.249 | 100.000 |
|  | Vertical Curve Information: (sag curve) PVC Station:; 0+230.027; Elevation:; 755.595. PVI Station:; 0+280.027; Elevation:; 755.595. PVT Station:; 0+330.027; Elevation:; 757.219. Grade in (%):; -0.001; Grade out (%):; 3.249. Change (%):; 3.251; K:; 30.764. Curve Length:; 100.000. Low Point:; 0+230.068; Elevation:; 755.595. Headlight Distance:; 147.857. |
| 3 | 0+600 | 765.991 | 7.947 | 140.000 |
|  | Vertical Curve Information: (sag curve) PVC Station:; 0+530; Elevation:; 763.717. PVI Station:; 0+600; Elevation:; 765.991. PVT Station:; 0+670; Elevation:; 771.554. Grade in (%):; 3.249; Grade out (%):; 7.947. Change (%):; 4.698; K:; 29.801. Curve Length:; 140.000. Headlight Distance:; 131.276. |
| 4 | 0+720 | 775.528 | 4.808 | 70.000 |
|  | Vertical Curve Information: (crest curve) PVC Station:; 0+685; Elevation:; 772.746. PVI Station:; 0+720; Elevation:; 775.528. PVT Station:; 0+755; Elevation:; 777.211. Grade in (%):; 7.947; Grade out (%):; 4.808. Change (%):; 3.139; K:; 22.301. Curve Length:; 70.000. Passing Distance:; 185.655; Stopping Distance:; 99.395. |
| 5 | 0+899.990 | 784.182 | 3.759 | 50.000 |
|  | Vertical Curve Information: (crest curve) PVC Station:; 0+874.990; Elevation:; 782.980. PVI Station:; 0+899.990; Elevation:; 784.182. PVT Station:; 0+924.990; Elevation:; 785.122. Grade in (%):; 4.808; Grade out (%):; 3.759. Change (%):; 1.050; K:; 47.634. Curve Length:; 50.000. Passing Distance:; 475.505; Stopping Distance:; 217.560. |
| 6 | 1+160 | 793.955 | 1.452 | 100.000 |
|  | Vertical Curve Information: (crest curve) PVC Station:; 1+110; Elevation:; 792.075. PVI Station:; 1+160; Elevation:; 793.955. PVT Station:; 1+210; Elevation:; 794.681. Grade in (%):; 3.759; Grade out (%):; 1.452. Change (%):; 2.307; K:; 43.348. Curve Length:; 100.000. Passing Distance:; 254.985; Stopping Distance:; 137.617. |
| 7 | 1+333.993 | 796.481 | 2.954 | 50.000 |
|  | Vertical Curve Information: (sag curve) PVC Station:; 1+308.993; Elevation:; 796.118. PVI Station:; 1+333.993; Elevation:; 796.481. PVT Station:; 1+358.993; Elevation:; 797.219. Grade in (%):; 1.452; Grade out (%):; 2.954. Change (%):; 1.503; K:; 33.273. Curve Length:; 50.000. Headlight Distance:; Infinite. |
| 8 | 1+520.027 | 801.977 | 4.520 | 80.000 |
|  | Vertical Curve Information: (sag curve) PVC Station:; 1+480.027; Elevation:; 800.795. PVI Station:; 1+520.027; Elevation:; 801.977. PVT Station:; 1+560.027; Elevation:; 803.785. Grade in (%):; 2.954; Grade out (%):; 4.520. Change (%):; 1.565; K:; 51.108. Curve Length:; 80.000. Headlight Distance:; Infinite. |
| 9 | 1+780 | 813.727 | 7.354 | 150.000 |
|  | Vertical Curve Information: (sag curve) PVC Station:; 1+705; Elevation:; 810.337. PVI Station:; 1+780; Elevation:; 813.727. PVT Station:; 1+855; Elevation:; 819.242. Grade in (%):; 4.520; Grade out (%):; 7.354. Change (%):; 2.834; K:; 52.926. Curve Length:; 150.000. Headlight Distance:; 250.373. |
| 10 | 1+976.382 | 828.168 | -3.232 | 130.000 |
|  | Vertical Curve Information: (crest curve) PVC Station:; 1+911.382; Elevation:; 823.388. PVI Station:; 1+976.382; Elevation:; 828.168. PVT Station:; 2+041.382; Elevation:; 826.068. Grade in (%):; 7.354; Grade out (%):; -3.232. Change (%):; 10.585; K:; 12.281. Curve Length:; 130.000. High Point:; 2+001.695; Elevation:; 826.709. Passing Distance:; 107.773; Stopping Distance:; 70.460. |
| 11 | 2+068.385 | 825.195 |  |  |