| Vertical Alignment Report Vertical Curves | Data generated:. 4/17/2011 9:38:42 PM. |
| --- | --- |

| Project: mohammad |  |
| --- | --- |

Units: meter

| Horizontal Alignment Information |  |
| --- | --- |
| Name: | CL |
| Station Range: | 0+000 to 2+068.385 |
| Station Equations: | None |

Curve Calculation Options 

Vertical Alignment: Center FG

Vertical Curve Information: (sag curve) PVC Station:; 0+230.027; Elevation:; 755.595. PVI Station:; 0+280.027; Elevation:; 755.595. PVT Station:; 0+330.027; Elevation:; 757.219. Grade in (%):; -0.001; Grade out (%):; 3.249. Change (%):; 3.251; K:; 30.764. Curve Length:; 100.000. Low Point:; 0+230.068; Elevation:; 755.595. Headlight Distance:; 147.857.

Vertical Curve Information: (sag curve) PVC Station:; 0+530; Elevation:; 763.717. PVI Station:; 0+600; Elevation:; 765.991. PVT Station:; 0+670; Elevation:; 771.554. Grade in (%):; 3.249; Grade out (%):; 7.947. Change (%):; 4.698; K:; 29.801. Curve Length:; 140.000. Headlight Distance:; 131.276.

Vertical Curve Information: (crest curve) PVC Station:; 0+685; Elevation:; 772.746. PVI Station:; 0+720; Elevation:; 775.528. PVT Station:; 0+755; Elevation:; 777.211. Grade in (%):; 7.947; Grade out (%):; 4.808. Change (%):; 3.139; K:; 22.301. Curve Length:; 70.000. Passing Distance:; 185.655; Stopping Distance:; 99.395.

Vertical Curve Information: (crest curve) PVC Station:; 0+874.990; Elevation:; 782.980. PVI Station:; 0+899.990; Elevation:; 784.182. PVT Station:; 0+924.990; Elevation:; 785.122. Grade in (%):; 4.808; Grade out (%):; 3.759. Change (%):; 1.050; K:; 47.634. Curve Length:; 50.000. Passing Distance:; 475.505; Stopping Distance:; 217.560.

Vertical Curve Information: (crest curve) PVC Station:; 1+110; Elevation:; 792.075. PVI Station:; 1+160; Elevation:; 793.955. PVT Station:; 1+210; Elevation:; 794.681. Grade in (%):; 3.759; Grade out (%):; 1.452. Change (%):; 2.307; K:; 43.348. Curve Length:; 100.000. Passing Distance:; 254.985; Stopping Distance:; 137.617.

Vertical Curve Information: (sag curve) PVC Station:; 1+308.993; Elevation:; 796.118. PVI Station:; 1+333.993; Elevation:; 796.481. PVT Station:; 1+358.993; Elevation:; 797.219. Grade in (%):; 1.452; Grade out (%):; 2.954. Change (%):; 1.503; K:; 33.273. Curve Length:; 50.000. Headlight Distance:; Infinite.

Vertical Curve Information: (sag curve) PVC Station:; 1+480.027; Elevation:; 800.795. PVI Station:; 1+520.027; Elevation:; 801.977. PVT Station:; 1+560.027; Elevation:; 803.785. Grade in (%):; 2.954; Grade out (%):; 4.520. Change (%):; 1.565; K:; 51.108. Curve Length:; 80.000. Headlight Distance:; Infinite.

Vertical Curve Information: (sag curve) PVC Station:; 1+705; Elevation:; 810.337. PVI Station:; 1+780; Elevation:; 813.727. PVT Station:; 1+855; Elevation:; 819.242. Grade in (%):; 4.520; Grade out (%):; 7.354. Change (%):; 2.834; K:; 52.926. Curve Length:; 150.000. Headlight Distance:; 250.373.

Vertical Curve Information: (crest curve) PVC Station:; 1+911.382; Elevation:; 823.388. PVI Station:; 1+976.382; Elevation:; 828.168. PVT Station:; 2+041.382; Elevation:; 826.068. Grade in (%):; 7.354; Grade out (%):; -3.232. Change (%):; 10.585; K:; 12.281. Curve Length:; 130.000. High Point:; 2+001.695; Elevation:; 826.709. Passing Distance:; 107.773; Stopping Distance:; 70.460.