Chapter five

Fieldwork

# 5.90998

5.1 Introduction

5.2 Instrumentation

5.3 Methodology of point's selection

5.4 Selection of Points

## Introduction

As mentioned in the project timeline, a field work is to be performed in order to get precise GNSS point readings. This work is divided into many stages. First, exploration of points on the specified area is to be done before the observing them, also these points are located on some areas of personal property or it may be doesn’t existing due to the early time of 1932 when the point where installed. Then, existing points are to be chosen according to point's quality and good distribution. Finally, the selected points are going to observed and evaluated.

## Instrumentation

Using GNSS based devices is roughly known for any researcher or a student in the field surveying and Geometric hence, the use of GNSS static method that was described before is to be practiced in order to master the task needed to get the necessary data.

The initial stage is practical training for using the GNSS devices. Two types of GNSS apparatus are used in this project they instrument are:

1.  Leica 900.

2.  Trimble R8.

Ultimately, the main structure of the devices is familiar, also using them became familiar and team is ready for the stage of point observations, as shown in figure (5-1) & (5-2).

Figure (5-1): Leica 900 GNSS components

Table (5- 1): Technical Specifications for Leica GPS 900.

| Networking Technology | Smart Track + Double frequency |
| --- | --- |
| GNSS antenna, ATX 900 GG | Built-in ground plane with Smart Check + and Bluetooth ® wireless technology, |
| Accuracy | Horizontal accuracy: + 10MM 1 ppm, the kinetic Vertical accuracy: 20mm + 1 ppm, the kinetic |
| Refresh rate | Position update rate: 1Hz standard 0.2 Hz - Option 5 Hertz |
| Range | RTK range: 2500m standard, option 5000m |
| Temperature | Operation: -30 ° C to +65 ° C Storage: -30 ° C to +80 ° C |

Figure ‎ (5-2): Trimble R8 GNSS components.

Table (5- 2): Technical Specifications for Trimble R8.

| Interface | Console TSC2 |
| --- | --- |
| Communication | Bluetooth. Internal and external Radio. GSM module |
| Accuracy | Horizontal accuracy: + 5MM 1 ppm, the kinetic Vertical accuracy: +20MM + 1 ppm, the kinetic |
| Ports | (RS-232 port),( Limo Port) |
| Temperature | Process: between -40° to 65 ° C Storage: between -40 ° to 75 ° C |

## Methodology of point's selection

# List of points and their coordinates are obtained from the Palestinian department of land survey.

# Benefiting from a GIS ortho photo of the West Bank, and by conversion of the txt file of trigs into a shapefile in a GIS Software. The location of points can be easily identified.

# The availability of points is not guaranteed by a theoretical database, so exploration of the points is the only way to get into points.

# By using a hand held GPS, the application of stakeout method is used to explore the points and to reach there position.

## Selection of Points

Points that are needed is ones of known and briefly precise 3-dimensional coordinates, these are the geodetic Triangulation points fixed during the British mandate on Palestine.

The actual preparations for setting up a triangulation system commenced only in February 1921, after the Survey Department moved to its new home in Jaffa. The survey began in May 1921.The first step was for the survey teams to layout geodetic points throughout the entire country, measure their values, and provide mathematical bases for the survey networks. The geodetic points required for mapping are classed in three categories:

1) Fixed points, or trigonometric stations, were determined by trigonometric methods and must be in sight of each other for the surveying observations. These imaginary lines form the sides of the triangles of the observation net. The data obtained were the position of the points in planmetric coordinates. The elevation of the points was determined above the datum (reference) level, which is the mean sea level (MSL).

2) Spot heights were determined by precise leveling and not necessarily in relation to the trigonometric network. The topographic heights were measured above the MSL along fixed runs in the field.

3) Gravimetric points, for the determination of the figure of the Earth.

As the points of trigonometric stations were mostly available with their names and position values, the points of project network were chosen from these points suitable coverage of the areas Hebron, Tulkarem and Qlqelya districts is chosen according to the availability of points and ease of accessibility to them. In the next stage of this project more point in the West Bank are going to be observed. Figure (5-3) & (5-4) illustrates the distribution of these points within the area of study.

The Registered points are explained in the tables (5-3) and (5-24).

Table (5.3) also shows the information about points’ names and coordinates also it provides the location of the point according to the nearest town or city (Tulkarem and Qlqelya).

Table (5-3): Selected network points (Registered)

| Point(name) | Easting(m) | Northing(m) | Elevation(m) | Location |
| --- | --- | --- | --- | --- |
| 771R | 152428.755 | 189125.387 | 87.46 | Ertah |
| 765R | 153983.185 | 190067.438 | 141.89 | Tulkarem |
| 103P | 153226.77 | 192522.21 | 568.75 | Shwakeh |
| 805S | 160711.45 | 189707.65 | 319.55 | Kafr Al Labad |
| 717T | 167342.37 | 180965.21 | 480.48 | Jeet |
| 162F | 160688.16 | 178393.60 | 412.10 | Baqa AL-hatab |
| 367X | 155518.06 | 170527.23 | 234.39 | Senerya |
| 728T | 152720.92 | 172117.77 | 203.80 | Bat Ameen |
| 191E | 150347.36 | 173830.56 | 173.20 | Ras Atya |
| 21F | 147550.43 | 176307.09 | 73.55 | Qalqelya |
| 4E | 149095.62 | 177710.29 | 116.49 | Qalqelya |
| 722T | 153639.08 | 176230.31 | 252.33 | Ezbet AL-Tabeeb |
| 870P | 156276.64 | 176536.64 | 276.00 | Azoon |
| 821P | 154797.24 | 177543.27 | 255.53 | Azoon |
| 236B | 156596.20 | 177579.47 | 316.49 | Seer |
| 803R | 153118.58 | 181710.14 | 156.05 | Falamya |
| 816S | 159351.59 | 182755.56 | 389.22 | Jyous |
| 815S | 158978.45 | 183966.59 | 318.61 | Jyous |
| 796S | 159177.69 | 192258.98 | 323.54 | Balaa |
| 493D | 155624.81 | 199032.28 | 103.51 | Zaita |

Figure (5-3): distribution of points within study area in Tulkarem and Qlqelya districts

Table (5-4): Details of Trig (771R)

| Date:- 3/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 771R |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 033 |  |
| Obs.Time:- 14 : 40 : 03 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 152430.13 | 189125.78 | 86.20 |
| Observe | 152430.3 | 189125.76 | 86.02 |
| WGS1984 | 35° 01' 25.23243" E | 32° 17' 44.60026" N | 106.57 |
| Description | This trig is located in the village of Ertah on the rock |  |  |

Table (5-5): Details of Trig (765R)

| Date:- 3/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 765R |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 032 |  |
| Obs.Time:- 14 : 24 : 14 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 153983.12 | 190068.37 | 140.30 |
| Observe | 153983.19 | 190067.91 | 140.23 |
| WGS1984 | 35° 02' 24.52971" E | 32° 18' 15.26666" N | 160.77 |
| Description | This trig is located in Tulkarem on the rock |  |  |

Table (5-6): Details of Trig (103P)

| Date:- 3/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 103P |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 029 |  |
| Obs.Time:- 14 : 01 : 12 |  | Accuracy:- 0.03 m |  |
| Coordinates | E | N | H |
| Registered | 153226.77 | 192522.21 | 86.35 |
| Observe | 153226.94 | 192521.90 | 86.32 |
| WGS1984 | 35° 01' 55.47163" E | 32° 19' 34.89309" N | 106.87 |
| Description | This trig is located in the village of Shwakeh on the rock |  |  |

Table (5-7): Details of Trig (805S)

| Date:- 21/8/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 805S |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Saleh VRS |  | Point Number:- 007 |  |
| Obs.Time:- 11 : 38 : 00 |  | Accuracy:- 0.011 m |  |
| Coordinates | E | N | H |
| Registered | 160711.45 | 189707.65. | 319.55 |
| Observe | 160711.22 | 189707.39 | 319.40 |
| WGS1984 | 35° 6 ' 41.71596" E | 32° 18' 3.82533" N | 339.91 |
| Description | This trig is located in the village of Kafr Al Labad on the rock |  |  |

Table (5-8): Details of Trig (717T)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 717T |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 027 |  |
| Obs.Time:- 16 : 27 : 58 |  | Accuracy:- 0.01 m |  |
| Coordinates | E | N | H |
| Registered | 167342.37 | 180965.21 | 480.48 |
| Observe | 167342.04 | 180964.88 | 480.54 |
| WGS1984 | 35° 10' 55.23534" E | 32° 13' 20.12599" N | 501.01 |
| Description | This trig is located in the village of Jeet on the rock |  |  |

Table (5-9): Details of Trig (162F)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 162F |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 026 |  |
| Obs.Time:- 16 : 03 : 22 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 160688.16 | 178393.60 | 412.10 |
| Observe | 160687.45 | 178392.98 | 412.03 |
| WGS1984 | 35° 06' 41.18190" E | 32° 11' 56.51568" N | 432.51 |
| Description | This trig is located in the village of Baqa AL-hatab on the rock |  |  |

Table (5-10): Details of Trig (367X)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 367X |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 024 |  |
| Obs.Time:- 15 : 09 : 30 |  | Accuracy:- 0.01 m |  |
| Coordinates | E | N | H |
| Registered | 155518.06 | 170527.23 | 234.39 |
| Observe | 155517.97 | 170527.11 | 234.45 |
| WGS1984 | 35° 03' 24.22248" E | 32° 07' 40.96853" N | 254.95 |
| Description | This trig is located in the village of Senerya on the rock |  |  |

Table (5-11): Details of Trig (728T)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 728T |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 023 |  |
| Obs.Time:- 14 : 54 : 10 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 152720.92 | 172117.77 | 203.80 |
| Observe | 152720.82 | 172117.78 | 203.72 |
| WGS1984 | 35° 01' 37.40965" E | 32° 08' 32.47135" N | 224.22 |
| Description | This trig is located in the village of Bat Ameen on the rock |  |  |

Table (5-12): Details of Trig (191E)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 191E |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 022 |  |
| Obs.Time:- 14 : 35 : 52 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 150347.36 | 173830.56 | 173.20 |
| Observe | 150347.40 | 173830.62 | 173.16 |
| WGS1984 | 35° 00' 06.72506" E | 32° 09' 27.94125" N | 193.68 |
| Description | This trig is located in the village of Ras Atya on the rock |  |  |

Table (5-13): Details of Trig (21F)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 21F |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 020 |  |
| Obs.Time:- 14 : 19 : 56 |  | Accuracy:- 0.03 m |  |
| Coordinates | E | N | H |
| Registered | 147550.43 | 176307.09 | 73.55 |
| Observe | 147550.28 | 176307.14 | 73.49 |
| WGS1984 | 34° 58' 19.77452" E | 32° 10' 48.15557" N | 94.03 |
| Description | This trig is located in Qalqelya on the rock |  |  |

Table (5-14): Details of Trig (4E)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 4E |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 019 |  |
| Obs.Time:- 14 : 04 : 59 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 149095.62 | 177710.29 | 116.49 |
| Observe | 149095.64 | 177710.43 | 116.52 |
| WGS1984 | 34° 59' 18.65699" E | 32° 11' 33.81648" N | 137.05 |
| Description | This trig is located in Qalqelya on the rock |  |  |

Table (5-15): Details of Trig (722T)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 722T |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 018 |  |
| Obs.Time:- 13 : 49 : 05 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 153639.08 | 176230.31 | 252.33 |
| Observe | 153639.03 | 176230.17 | 252.29 |
| WGS1984 | 35° 02' 12.20067" E | 32° 10' 46.02441" N | 272.8 |
| Description | This trig is located in the village of Ezbet AL-Tabeeb on the rock |  |  |

Table (5-16): Details of Trig (870P)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 870P |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 017 |  |
| Obs.Time:- 13 : 29 : 54 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 156276.64 | 176536.64 | 276.00 |
| Observe | 156276.59 | 176536.64 | 275.98 |
| WGS1984 | 35° 03' 52.86639" E | 32° 10' 56.09613" N | 296.49 |
| Description | This trig is located in the village of Azoon on the rock |  |  |

Table (5-17): Details of Trig (821P)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 821P |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 016 |  |
| Obs.Time:- 13 : 16 : 26 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 154797.24 | 177543.27 | 255.53 |
| Observe | 154797.40 | 177543.00 | 255.72 |
| WGS1984 | 35° 02' 56.34554" E | 32° 11' 28.70065" N | 276.23 |
| Description | This trig is located in the village of Azoon on the rock |  |  |

Table (5-18): Details of Trig (236B)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 236B |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 015 |  |
| Obs.Time:- 12 : 56 : 40 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 156596.20 | 177579.47 | 316.49 |
| Observe | 156596.26 | 177579.19 | 316.44 |
| WGS1984 | 35° 04' 05.01872" E | 32° 11' 29.95476" N | 336.95 |
| Description | This trig is located in the village of Seer on the rock |  |  |

Table (5-19): Details of Trig (803R)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 803R |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 013 |  |
| Obs.Time:- 12 : 23 : 53 |  | Accuracy:- 0.03 m |  |
| Coordinates | E | N | H |
| Registered | 153118.58 | 181710.14 | 156.05 |
| Observe | 153118.71 | 181709.99 | 156.14 |
| WGS1984 | 35° 01' 52.00159" E | 32° 13' 43.89428" N | 176.67 |
| Description | This trig is located in the village of Falamya on the rock |  |  |

Table (5-20): Details of Trig (816S)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 816S |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 011 |  |
| Obs.Time:- 12 : 02 : 49 |  | Accuracy:- 0.01 m |  |
| Coordinates | E | N | H |
| Registered | 159351.59 | 182755.56 | 389.22 |
| Observe | 159351.53 | 182755.44 | 389.17 |
| WGS1984 | 35° 05' 50.00996" E | 32° 14' 18.09803" N | 409.68 |
| Description | This trig is located in the village of Jyous on the rock |  |  |

Table (5-21): Details of Trig (815S)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 815S |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 008 |  |
| Obs.Time:- 11 : 38 : 46 |  | Accuracy:- 0.01 m |  |
| Coordinates | E | N | H |
| Registered | 158978.45 | 183966.59 | 318.61 |
| Observe | 158978.29 | 183966.49 | 318.72 |
| WGS1984 | 35° 05' 35.70547" E | 32° 14' 57.40085" N | 339.23 |
| Description | This trig is located in the village of Jyous on the rock |  |  |

Table (5-22): Details of Trig (796S)

| Date:- 1/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 796S |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 005 |  |
| Obs.Time:- 16 : 32 : 03 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 159177.69 | 192258.98 | 323.54 |
| Observe | 159177.15 | 192259.41 | 323.62 |
| WGS1984 | 35° 05' 42.97984" E | 32° 19' 26.62575" N | 344.15 |
| Description | This trig is located in the village of Balaa on the rock |  |  |

Table (5-23): Details of Trig (493D)

| Date:- 1/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 493D |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 001 |  |
| Obs.Time:- 12 : 59 : 32 |  | Accuracy:- 0.01 m |  |
| Coordinates | E | N | H |
| Registered | 155624.81 | 199032.28 | 103.51 |
| Observe | 155625.26 | 199034.05 | 101.49 |
| WGS1984 | 35° 03' 26.82601" E | 32° 23' 06.41448" N | 122.05 |
| Description | This trig is located in the village of Zaita on the rock |  |  |

Table (5.24) also shows the information about points’ names and coordinates also it provides the location of the point according to the nearest town or city (Hebron).

Table (5-24): Selected network points (Registered)

| Point(name) | Easting(m) | Northing(m) | Elevation(m) | Location |
| --- | --- | --- | --- | --- |
| 585B | 160773.39 | 91851.11 | 794.29 | Yatta |
| 569B | 157249.15 | 94224.60 | 810.69 | Yatta |
| 565B | 156086.70 | 95234.67 | 774.12 | Yatta |
| 564B | 156716.18 | 95937.00 | 774.24 | Yatta |
| 537T | 166776.27 | 103869.46 | 942.61 | Bani Naeem |
| 394F | 148752.64 | 108279.93 | 805.21 | Ithna |
| 344N | 152271.78 | 108643.28 | 614.98 | Trqwmya |
| 336N | 157079.28 | 117367.82 | 638.89 | Soureef |
| 570T | 156096.76 | 117739.33 | 588.94 | Soureef |
| 552B | 155580.17 | 101424.37 | 913.81 | Dura (Sinjer) |
| 356N | 155722.87 | 107271.25 | 875.47 | Hebron (Looza) |
| 441F | 142397.9 | 91081.11 | 643.29 | Aldahriyeh(Ramadeen) |
| 520T | 157133.47 | 113959.94 | 849.42 | Halhoul (Jala) |
| 351N | 150135.28 | 103756.06 | 730.17 | Dura(Homsa) |
| 597B | 160474.73 | 100867.46 | 902.79 | Hebron(Qilqis mt.) |
| 373N | 155409.64 | 96442.86 | 739.5 | Yatta (beit Emra) |
| 347N | 152144.28 | 110606.8 | 567.75 | Beit Oula |
| 705P | 148918.70 | 92762.38 | 669.29 | Aldahriyeh |
| 402N | 158738.85 | 87520.78 | 796.08 | Alsamoua' |
| 437N | 169288.70 | 107612.62 | 824.20 | ALshyoukh(Wad AL-reem) |

Figure (5-4): distribution of points within study area in Hebron district.

Table (5-25): Details of Trig (585B)

| Date:- 12/3/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 585B |  | Instrument:- Trimble R8 |  |
| File Name:- Banan |  | Point Number:- 001 |  |
| Obs.Time:- 12 : 49 : 52 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 160773.39 | 91851.11 | 794.29 |
| Observe | 160773.55 | 91851.66 | 794.37 |
| WGS1984 | 35° 06' 47.23873" E | 31° 25' 06.85521" N | 813.313 |
| Description | This trig is located in the village of Yatta on the rock |  |  |

Table (5-26): Details of Trig (569B)

| Date:- 12/3/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 569B |  | Instrument:- Trimble R8 |  |
| File Name:- Banan |  | Point Number:- 002 |  |
| Obs.Time:- 13 : 20 : 31 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 157249.15 | 94224.60 | 810.69 |
| Observe | 157249.17 | 94224.41 | 810.53 |
| WGS1984 | 35° 04' 33.69938" E | 31° 26' 23.77949" N | 829.664 |
| Description | This trig is located in the village of Yatta on the rock |  |  |

Table (5-27): Details of Trig (565B)

| Date:- 12/3/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 565B |  | Instrument:- Trimble R8 |  |
| File Name:- Banan |  | Point Number:- 003 |  |
| Obs.Time:- 13 : 42 : 45 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 156086.70 | 95234.67 | 774.12 |
| Observe | 156086.89 | 95235.00 | 773.88 |
| WGS1984 | 35° 03' 49.63568" E | 31° 26' 56.54489" N | 793.07 |
| Description | This trig is located in the village of Yatta on the rock |  |  |

Table (5-28): Details of Trig (564B)

| Date:- 12/3/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 564B |  | Instrument:- Trimble R8 |  |
| File Name:- Banan |  | Point Number:- 004 |  |
| Obs.Time:- 13 : 55 : 39 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 156716.18 | 95937.00 | 774.24 |
| Observe | 156716.45 | 95937.29 | 773.99 |
| WGS1984 | 35° 04' 13.44649" E | 31° 27' 19.37238" N | 793.202 |
| Description | This trig is located in the village of Yatta on the rock |  |  |

Table (5-29): Details of Trig (537T)

| Date:- 30/3/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 537T |  | Instrument:- Trimble R8 |  |
| File Name:- Banan |  | Point Number:- 005 |  |
| Obs.Time:- 14 : 11 : 18 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 166776.27 | 103869.46 | 942.61 |
| Observe | 166776.34 | 103869.94 | 942.72 |
| WGS1984 | 35° 10' 34.39121" E | 31° 31' 37.16903" N | 961.906 |
| Description | This trig is located in the village of Bani Naeem on the rock |  |  |

Table (5-30): Details of Trig (394F)

| Date:- 30/3/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 394F |  | Instrument:- Trimble R8 |  |
| File Name:- Banan |  | Point Number:- 006 |  |
| Obs.Time:- 14 : 49 : 32 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 148752.64 | 108302.93 | 805.21 |
| Observe | 148751.75 | 108302.31 | 506.60 |
| WGS1984 | 34° 59'10.85108" E | 31° 34' 00.41847" N | 525.871 |
| Description | This trig is located in the village of Ithna on the rock |  |  |

Table (5-31): Details of Trig (344N)

| Date:- 30/3/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 344N |  | Instrument:- Trimble R8 |  |
| File Name:- Banan |  | Point Number:- 007 |  |
| Obs.Time:- 15 : 00 : 54 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 152271.78 | 108643.28 | 614.98 |
| Observe | 152272.21 | 108643.48 | 614.53 |
| WGS1984 | 35° 01' 24.32650" E | 31° 34' 11.70650" N | 634.002 |
| Description | This trig is located in the village of Trqwmya on the rock |  |  |

Table (5-32): Details of Trig (336N)

| Date:- 30/3/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 336N |  | Instrument:- Trimble R8 |  |
| File Name:- Banan |  | Point Number:- 008 |  |
| Obs.Time:- 15 : 35 : 29 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 157079.28 | 117367.82 | 638.89 |
| Observe | 157079.64 | 117371.15 | 638.45 |
| WGS1984 | 35° 04' 26.23579" E | 31° 38' 55.28771" N | 658.207 |
| Description | This trig is located in the village of Soureef on the rock |  |  |

Table (5-33): Details of Trig (570T)

| Date:- 30/3/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 570T |  | Instrument:- Trimble R8 |  |
| File Name:- Banan |  | Point Number:- 009 |  |
| Obs.Time:- 15 : 56 : 27 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 156096.76 | 117739.33 | 588.94 |
| Observe | 156097.26 | 117739.77 | 589.90 |
| WGS1984 | 35° 03' 48.93300" E | 31° 39' 07.21559" N | 609.623 |
| Description | This trig is located in the village of Soureef on the rock |  |  |

Table (5-34): Details of Trig (552B)

| Date:- 11/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 552B |  | Instrument:- Leica 900 |  |
| File Name:- Hebron VRS |  | Point Number:- 34 |  |
| Obs.Time:- 08 : 15 : 01 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 155580.17 | 101424.37 | 913.81 |
| Observe | 155580.41 | 101424.55 | 913.19 |
| WGS1984 | 35° 03' 30.14658" E | 31° 30' 17.48970" N | 933.5 |
| Description | This trig is located in the village of Dura (Sinjer) on the rock |  |  |

Table (5-35): Details of Trig (356N)

| Date:- 11/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 356N |  | Instrument:- Leica 900 |  |
| File Name:- Hebron VRS |  | Point Number:- 33 |  |
| Obs.Time:- 04 : 25 : 15 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 155722.87 | 107271.25 | 875.47 |
| Observe | 155723.17 | 107271.38 | 874.83 |
| WGS1984 | 35° 03' 35.26416" E | 31° 33' 27.32726" N | 895.15 |
| Description | This trig is located in the village of Hebron (Looza) on the rock |  |  |

Table (5-36): Details of Trig (441F)

| Date:- 10/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 441F |  | Instrument:- Leica 900 |  |
| File Name:- Hebron VRS |  | Point Number:- 23 |  |
| Obs.Time:- 06 : 45 : 57 |  | Accuracy:- 0.01 m |  |
| Coordinates | E | N | H |
| Registered | 142397.90 | 91081.11 | 643.29 |
| Observe | 142398.23 | 91081.13 | 641.04 |
| WGS1984 | 34° 55' 11.62075" E | 31° 24' 40.82418" N | 661.38 |
| Description | This trig is located in the village of Aldahriyeh(Ramadeen) on the rock |  |  |

Table (5-37): Details of Trig (520T)

| Date:- 10/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 520T |  | Instrument:- Leica 900 |  |
| File Name:- Hebron VRS |  | Point Number:- 31 |  |
| Obs. Time:- 02 : 21 : 25 |  | Accuracy:- 0.01 m |  |
| Coordinates | E | N | H |
| Registered | 157133.47 | 113959.94 | 849.42 |
| Observe | 157133.43 | 113960.30 | 848.41 |
| WGS1984 | 35° 04' 28.43555" E | 31° 37' 04.55495" N | 868.75 |
| Description | This trig is located in the village of Halhoul (Jala) on the rock |  |  |

Table (5-38): Details of Trig (351N)

| Date:- 10/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 351N |  | Instrument:- Leica 900 |  |
| File Name:- Hebron VRS |  | Point Number:- 35 |  |
| Obs. Time:- 08 : 42 : 41 |  | Accuracy:- 0.03 m |  |
| Coordinates | E | N | H |
| Registered | 150135.28 | 103756.06 | 730.17 |
| Observe | 150135.31 | 103756.04 | 728.56 |
| WGS1984 | 35° 00' 03.63926" E | 31° 31' 32.90565" N | 748.90 |
| Description | This trig is located in the village of Dura (Homsa) on the rock |  |  |

Table (5-39): Details of Trig (597B)

| Date:- 10/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 597B |  | Instrument:- Leica 900 |  |
| File Name:- Hebron VRS |  | Point Number:- 20 |  |
| Obs. Time:- 05 : 22 : 04 |  | Accuracy:- 0.01 m |  |
| Coordinates | E | N | H |
| Registered | 160474.73 | 100867.46 | 902.79 |
| Observe | 160474.94 | 100867.55 | 901.37 |
| WGS1984 | 35° 06' 35.64447" E | 31° 29' 59.57937" N | 921.66 |
| Description | This trig is located in the village of Hebron (Qilqis mt) on the rock |  |  |

Table (5-40): Details of Trig (373N)

| Date:- 10/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 373N |  | Instrument:- Leica 900 |  |
| File Name:- Hebron VRS |  | Point Number:- 21 |  |
| Obs. Time:- 05 : 39 : 53 |  | Accuracy:- 0.01 m |  |
| Coordinates | E | N | H |
| Registered | 155409.64 | 96442.86 | 739.5 |
| Observe | 155409.90 | 96443.01 | 738.18 |
| WGS1984 | 35° 03' 23.93917" E | 31° 27' 35.74310" N | 758.47 |
| Description | This trig is located in the village of Yatta (beit Emra) on the rock |  |  |

Table (5-41): Details of Trig (347N)

| Date:- 10/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 347N |  | Instrument:- Leica 900 |  |
| File Name:- Hebron VRS |  | Point Number:- 32 |  |
| Obs. Time:- 03 : 24 : 10 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 152144.28 | 110606.8 | 567.75 |
| Observe | 152144.45 | 110606.75 | 566.50 |
| WGS1984 | 35° 01' 19.36062" E | 31° 35' 15.44396" N | 586.85 |
| Description | This trig is located in the village of Bait Oula on the rock |  |  |

Table (5-42): Details of Trig (705P)

| Date:- 10/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 705P |  | Instrument:- Leica 900 |  |
| File Name:- Hebron VRS |  | Point Number:- 22 |  |
| Obs. Time:- 06 : 20 : 36 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 148918.70 | 92762.38 | 669.29 |
| Observe | 148919.15 | 92762.43 | 666.83 |
| WGS1984 | 35° 59' 18.36202" E | 31° 25' 35.89407" N | 687.14 |
| Description | This trig is located in the village of Aldahriyeh on the rock |  |  |

Table (5-43): Details of Trig (402N)

| Date:- 10/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 402N |  | Instrument:- Leica 900 |  |
| File Name:- Hebron VRS |  | Point Number:- 25 |  |
| Obs. Time:- 08 : 31 : 56 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 158738.85 | 87520.78 | 796.08 |
| Observe | 158739.76 | 87521.19 | 794.5 |
| WGS1984 | 35° 05' 30.41067" E | 31° 22' 46.19725" N | 814.76 |
| Description | This trig is located in the village of Alsamoua' on the rock |  |  |

Table (5-44): Details of Trig (437N)

| Date:- 10/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 437N |  | Instrument:- Leica 900 |  |
| File Name:- Hebron VRS |  | Point Number:- 30 |  |
| Obs. Time:- 12 : 40 : 17 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 169288.70 | 107612.62 | 824.20 |
| Observe | 169288.57 | 107613.03 | 822.82 |
| WGS1984 | 35° 12' 09.61841" E | 31° 33' 38.71380" N | 843.09 |
| Description | This trig is located in the village of ALshyoukh(Wad AL-reem) on the rock |  |  |

Table (5.45) also shows the information about points’ names and coordinates also it provides the location of the point according to the nearest town or city (Jenin).

Table (5-45): Selected network points (Registered)

| Point(name) | Easting(m) | Northing(m) | Elevation(m) | Location |
| --- | --- | --- | --- | --- |
| 799D | 171066.13 | 216350.70 | 108.56 | Zububa |
| 523S | 179794.28 | 210343.12 | 124.97 | Sabah Al Kheir |
| 149T | 182397.17 | 208701.37 | 158.13 | Beit Qad |
| 300T | 180244.82 | 207314.87 | 193.96 | Jenin |
| 1078S | 180824.64 | 202860.76 | 371.82 | Qabatiya |
| 1076S | 180005.87 | 203829.47 | 351.74 | Qabatiya |
| 701E | 175936.27 | 206014.34 | 305.12 | Barqin |
| 702E | 176065.94 | 205495.92 | 273.84 | Ash Shuhada |
| 132T | 168551.60 | 202361.60 | 380.48 | Arraba |
| 744E | 172917.58 | 207400.21 | 189.98 | Barqin |
| 326V | 185353.72 | 211202.81 | 309.97 | Deir Ghazala |
| 993R | 168522.92 | 213702.42 | 230.2 | At Tayba |
| 579S | 174332.52 | 208442.16 | 243.89 | Kafr Dan |
| 543W | 168772.13 | 201319.42 | 360.01 | Arraba |
| 283P | 166284.91 | 195546.68 | 332.24 | Ar Rama |

Figure (5-5): distribution of points within study area in Jenin district.

Table (5-46): Details of Trig (283P)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 283P |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 001 |  |
| Obs. Time:- 06 : 37 : 04 |  | Accuracy:- 0.023 m |  |
| Coordinates | E | N | H |
| Registered | 166284.91 | 195546.68 | 332.24 |
| Observe | 166284.839 | 195546.322 | 404.296 |
| WGS1984 | 35° 10' 14.68571" E | 32° 21' 13.48387" N | 424.798 |
| Description | This trig is located in the village of (Ar Rama) on the rock |  |  |

Table (5-47): Details of Trig (543W)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 543W |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 002 |  |
| Obs. Time:- 07 : 05 : 09 |  | Accuracy:- 0.022 m |  |
| Coordinates | E | N | H |
| Registered | 168772.13 | 201319.42 | 360.01 |
| Observe | 168771.987 | 201318.856 | 359.136 |
| WGS1984 | 35° 11' 49.79176" E | 32° 24' 20.89429" N | 379.641 |
| Description | This trig is located in the village of (Arraba) on the rock |  |  |

Table (5-48): Details of Trig (132T)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 132T |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 006 |  |
| Obs. Time:- 07 : 21 : 18 |  | Accuracy:- 0.016 m |  |
| Coordinates | E | N | H |
| Registered | 168551.60 | 202361.60 | 380.48 |
| Observe | 168551.439 | 202361.24 | 380.856 |
| WGS1984 | 35° 11' 41.34843" E | 32° 24' 54.73267" N | 401.364 |
| Description | This trig is located in the village of (Arraba) on the rock |  |  |

Table (5-49): Details of Trig (744E)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 744E |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 008 |  |
| Obs. Time:- 08 : 39 : 58 |  | Accuracy:- 0.013 m |  |
| Coordinates | E | N | H |
| Registered | 172917.58 | 207400.21 | 189.98 |
| Observe | 172917.434 | 207399.63 | 290.789 |
| WGS1984 | 35° 14' 28.50572" E | 32° 27' 38.27550" N | 311.29 |
| Description | This trig is located in the village of (Barqin) on the rock |  |  |

Table (5-50): Details of Trig (579S)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 579S |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 009 |  |
| Obs. Time:- 09 : 03 : 00 |  | Accuracy:- 0.012 m |  |
| Coordinates | E | N | H |
| Registered | 174332.52 | 208442.16 | 243.89 |
| Observe | 174332.338 | 208441.643 | 243.511 |
| WGS1984 | 35° 15' 22.70202" E | 32° 28' 12.08304" N | 264.01 |
| Description | This trig is located in the village of (Kafr Dan) on the rock |  |  |

Table (5-51): Details of Trig (701E)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 701E |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 009 |  |
| Obs. Time:- 09 : 21 : 37 |  | Accuracy:- 0.013 m |  |
| Coordinates | E | N | H |
| Registered | 175936.27 | 206014.34 | 305.12 |
| Observe | 175936.246 | 206013.527 | 305.917 |
| WGS1984 | 32° 26' 53.23028" N | 35° 16' 24.06243" E | 326.404 |
| Description | This trig is located in the village of (Barqin) on the rock |  |  |

Table (5-52): Details of Trig (702E)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 702E |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 011 |  |
| Obs. Time:- 09 : 30 : 05 |  | Accuracy:- 0.013 m |  |
| Coordinates | E | N | H |
| Registered | 176065.94 | 205495.92 | 273.84 |
| Observe | 176066.018 | 205495.173 | 274.428 |
| WGS1984 | 35° 16' 29.01769" E | 32° 26' 36.40028" N | 294.913 |
| Description | This trig is located in the village of (Ash Shuhada) on the rock |  |  |

Table (5-53): Details of Trig (300T)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 300T |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 012 |  |
| Obs. Time:- 10 : 04 : 11 |  | Accuracy:- 0.017 m |  |
| Coordinates | E | N | H |
| Registered | 180244.82 | 207314.87 | 193.96 |
| Observe | 180244.72 | 207314.38 | 193.254 |
| WGS1984 | 35° 19' 09.06541" E | 32° 27' 35.33973" N | 213.729 |
| Description | This trig is located in the village of (Jenin) on the rock |  |  |

Table (5-54): Details of Trig (523S)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 523S |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 013 |  |
| Obs. Time:- 10 : 21 : 24 |  | Accuracy:- 0.017 m |  |
| Coordinates | E | N | H |
| Registered | 179794.28 | 210343.12 | 124.97 |
| Observe | 179794.43 | 210342.72 | 124.049 |
| WGS1984 | 35° 18' 51.94351" E | 32° 29' 13.66304" N | 144.532 |
| Description | This trig is located in the village of (Sabah Al Kheir) on the rock |  |  |

Table (5-55): Details of Trig (799D)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 799D |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 014 |  |
| Obs. Time:- 11 : 00 : 29 |  | Accuracy:- 0.02 m |  |
| Coordinates | E | N | H |
| Registered | 171066.13 | 216350.70 | 108.56 |
| Observe | 171066.45 | 216350.40 | 109.214 |
| WGS1984 | 35° 13' 17.68601" E | 32° 32' 28.85588" N | 129.742 |
| Description | This trig is located in the village of (Zububa) on the rock |  |  |

Table (5-56): Details of Trig (993R)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 993R |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 015 |  |
| Obs. Time:- 11 : 24 : 36 |  | Accuracy:- 0.018 m |  |
| Coordinates | E | N | H |
| Registered | 168522.92 | 213702.42 | 230.20 |
| Observe | 168522.77 | 213701.52 | 229.43 |
| WGS1984 | 35° 11' 40.21056" E | 32° 31' 02.86951" N | 249.96 |
| Description | This trig is located in the village of (At Tayba) on the rock |  |  |

Table (5-57): Details of Trig (149T)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 149T |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 016 |  |
| Obs. Time:- 12 : 11 : 49 |  | Accuracy:- 0.016 m |  |
| Coordinates | E | N | H |
| Registered | 182397.17 | 208701.37 | 158.13 |
| Observe | 182396.81 | 208701.55 | 158.69 |
| WGS1984 | 35° 20' 31.53713" E | 32° 28' 20.28840" N | 179.16 |
| Description | This trig is located in the village of (Beit Qad) on the rock |  |  |

Table (5-58): Details of Trig (326V)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 326V |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 017 |  |
| Obs. Time:- 12 : 47 : 51 |  | Accuracy:- 0.021 m |  |
| Coordinates | E | N | H |
| Registered | 185353.72 | 211202.81 | 309.97 |
| Observe | 185353.416 | 211202.406 | 310.663 |
| WGS1984 | 35° 22' 24.91227" E | 32° 29' 41.33440" N | 331.125 |
| Description | This trig is located in the village of (Deir Ghazala) on the rock |  |  |

Table (5-59): Details of Trig (1076S)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 1076S |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 018 |  |
| Obs. Time:- 02 : 02 : 01 |  | Accuracy:- 0.013 m |  |
| Coordinates | E | N | H |
| Registered | 180005.87 | 203829.47 | 351.74 |
| Observe | 180005.76 | 203829.19 | 352.27 |
| WGS1984 | 35° 18' 59.77585" E | 32° 25' 42.20879" N | 372.73 |
| Description | This trig is located in the village of (Qabatiya) on the rock |  |  |

Table (5-60): Details of Trig (1078S)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 1078S |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 019 |  |
| Obs. Time:- 02 : 15 : 18 |  | Accuracy:- 0.014 m |  |
| Coordinates | E | N | H |
| Registered | 180824.64 | 202860.76 | 371.82 |
| Observe | 180824.42 | 202860.92 | 371.40 |
| WGS1984 | 35° 19' 31.06957" E | 32° 25' 10.74688" N | 391.86 |
| Description | This trig is located in the village of (Qabatiya) on the rock |  |  |

Table (5.61) also shows the information about points’ names and coordinates also it provides the location of the point according to the nearest town or city (Tubas).

Table (5-61): Selected network points (Registered)

| Point(name) | Easting(m) | Northing(m) | Elevation(m) | Location |
| --- | --- | --- | --- | --- |
| 615W | 186254.24 | 191429.74 | 354.74 | Tubas |
| 1109S | 185037.55 | 194360.38 | 506.21 | Tubas |

Figure (5-6): distribution of points within study area in Tubas district.

Table (5-62): Details of Trig (615W)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 615W |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 020 |  |
| Obs. Time:- 03 : 12 : 44 |  | Accuracy:- 0.017 m |  |
| Coordinates | E | N | H |
| Registered | 186254.24 | 191429.74 | 354.74 |
| Observe | 186253.91 | 191429.80 | 355.18 |
| WGS1984 | 35° 22' 58.14264" E | 32° 18' 59.41049" N | 375.604 |
| Description | This trig is located in the village of (Tubas) on the rock |  |  |

Table (5-63): Details of Trig (1109S)

| Date:- 18/7/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 1109S |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Jenin VRS |  | Point Number:- 021 |  |
| Obs. Time:- 03 : 35 : 44 |  | Accuracy:- 0.018 m |  |
| Coordinates | E | N | H |
| Registered | 185037.55 | 194360.38 | 506.21 |
| Observe | 185037.551 | 194360.303 | 506.694 |
| WGS1984 | 35° 22' 11.81472" E | 32° 20' 34.60753" N | 527.121 |
| Description | This trig is located in the village of (Tubas) on the rock |  |  |

Table (5.64) also shows the information about points’ names and coordinates also it provides the location of the point according to the nearest town or city (Nablus).

Table (5-64): Selected network points (Registered)

| Point(name) | Easting(m) | Northing(m) | Elevation(m) | Location |
| --- | --- | --- | --- | --- |
| 647W | 175126.00 | 185396.52 | 668.04 | Asira ash Shamaliya |
| 333X | 173564.54 | 183636.71 | 590.25 | Asira ash Shamaliya |
| 652W | 175284.28 | 188513.36 | 602.11 | Yasid |
| 642X | 173777.83 | 188618.92 | 548.15 | Beit Imrin |
| 152B | 176494.64 | 180216.24 | 600.78 | Nablus |
| 718T | 168441.58 | 184299.89 | 370.43 | Deir Sharaf |
| 710T | 169348.11 | 181305.79 | 568.55 | Sarra |

Figure (5-7): distribution of points within study area in Nablus district.

Table (5-65): Details of Trig (647W)

| Date:- 21/08/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 647W |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Nablus VRS |  | Point Number:- 013 |  |
| Obs. Time:- 09 : 50 : 00 |  | Accuracy:- 0.013 m |  |
| Coordinates | E | N | H |
| Registered | 175126.00 | 185396.52 | 668.04 |
| Observe | 175125.74 | 185396.53 | 668.47 |
| WGS1984 | 35° 15' 52.58937" E | 32° 15' 43.95036" N | 688.92 |
| Description | This trig is located in the village of (Asira ash Shamaliya) on the rock |  |  |

Table (5-66): Details of Trig (333X)

| Date:- 21/08/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 333X |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Nablus VRS |  | Point Number:- 014 |  |
| Obs. Time:- 09 : 24 : 00 |  | Accuracy:- 0.016 m |  |
| Coordinates | E | N | H |
| Registered | 173564.54 | 183636.71 | 590.25 |
| Observe | 173564.25 | 183636.68 | 590.77 |
| WGS1984 | 35° 14' 52.90296" E | 32° 14' 46.84350" N | 611.21 |
| Description | This trig is located in the village of (Asira ash Shamaliya) on the rock |  |  |

Table (5-67): Details of Trig (652W)

| Date:- 21/08/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 652W |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Nablus VRS |  | Point Number:- 012 |  |
| Obs. Time:- 08 : 54 : 00 |  | Accuracy:- 0.010 m |  |
| Coordinates | E | N | H |
| Registered | 175284.28 | 188513.36 | 602.11 |
| Observe | 175283.90 | 188513.41 | 602.58 |
| WGS1984 | 35° 15' 58.70084" E | 32° 17' 25.13290" N | 623.03 |
| Description | This trig is located in the village of (Yasid) on the rock |  |  |

Table (5-68): Details of Trig (642X)

| Date:- 21/08/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 642X |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Nablus VRS |  | Point Number:- 011 |  |
| Obs. Time:- 08 : 36 : 00 |  | Accuracy:- 0.011 m |  |
| Coordinates | E | N | H |
| Registered | 173777.83 | 188618.92 | 548.15 |
| Observe | 173777.63 | 188618.73 | 548.76 |
| WGS1984 | 35° 15' 1.13533" E | 32° 17' 28.57651" N | 569.22 |
| Description | This trig is located in the village of (Beit Imrin) on the rock |  |  |

Table (5-69): Details of Trig (152B)

| Date:- 21/08/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 152B |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Nablus VRS |  | Point Number:- 020 |  |
| Obs. Time:- 10 : 32 : 00 |  | Accuracy:- 0.020 m |  |
| Coordinates | E | N | H |
| Registered | 176494.64 | 180216.24 | 600.78 |
| Observe | 176494.86 | 180216.316 | 601.14 |
| WGS1984 | 35° 16' 44.76134" E | 32° 12' 55.75240" N | 621.572 |
| Description | This trig is located in the village of (Nablus) on the rock |  |  |

Table (5-70): Details of Trig (718T)

| Date:- 21/08/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 718T |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Nablus VRS |  | Point Number:- 719 |  |
| Obs. Time:- 11 : 12 : 00 |  | Accuracy:- 0.015 m |  |
| Coordinates | E | N | H |
| Registered | 168441.58 | 184299.89 | 370.43 |
| Observe | 168441.386 | 184299.91 | 370.95 |
| WGS1984 | 35° 11' 37.20657" E | 32° 15' 8.39958" N | 391.429 |
| Description | This trig is located in the village of (Deir Sharaf) on the rock |  |  |

Table (5-71): Details of Trig (710T)

| Date:- 2/5/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 710T |  | Instrument:- Leica 900 |  |
| File Name:- Saleh VRS |  | Point Number:- 028 |  |
| Obs.Time:- 16 : 42 : 21 |  | Accuracy:- 0.03 m |  |
| Coordinates | E | N | H |
| Registered | 169348.36 | 181306.00 | 568.55 |
| Observe | 169348.11 | 181305.79 | 568.71 |
| WGS1984 | 35° 12 11.84703" E | 32° 13' 31.20025" N | 589.16 |
| Description | This trig is located in the village of Sara on the rock |  |  |

Table (5.72) also shows the information about points’ names and coordinates also it provides the location of the point according to the nearest town or city (Ramallah).

Table (5-72): Selected network points (Registered)

| Point(name) | Easting(m) | Northing(m) | Elevation(m) | Location |
| --- | --- | --- | --- | --- |
| 979T | 165240.6 | 150347.93 | 751.35 | Ein Qiniya |
| 676L | 169213.18 | 148845.37 | 845.65 | Surda |
| 657L | 166751.52 | 147794.39 | 745.53 | Ramallah |
| 986S | 170186.38 | 146463.99 | 871.41 | Al Bira |
| 121Y | 171841.27 | 152650.15 | 713.1 | Ein Siniya |
| 480T | 168216.57 | 143998.5 | 848.11 | Beituniya |
| 2K | 169092.08 | 141297.74 | 810.02 | Qalandiya |
| 215F | 178483.62 | 157845 | 791.77 | Abu Falah |
| 189F | 160852.72 | 162614.21 | 477.84 | Kafr Ein |
| 974T | 166120.91 | 154854.11 | 660.89 | Burham |
| 220J | 157403.96 | 150943.13 | 423.78 | Bani Harith |
| 218J | 157300.27 | 149898.38 | 397.28 | Bani Harith |

Figure (5-8): distribution of points within study area in Ramallah district.

Table (5-73): Details of Trig (979T)

| Date:- 22/8/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 979T |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Ramallah VRS |  | Point Number:- 755 |  |
| Obs.Time:- 09 : 01 : 00 |  | Accuracy:- 0.015 m |  |
| Coordinates | E | N | H |
| Registered | 165240.6 | 150347.93 | 751.35 |
| Observe | 165240.61 | 150347.65 | 751.87 |
| WGS1984 | 35° 9' 35.46578" E | 31° 56' 46.12556" N | 772.27 |
| Description | This trig is located in the village of (Ein Qiniya) on the rock |  |  |

Table (5-74): Details of Trig (676L)

| Date:- 22/8/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 676L |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Ramallah VRS |  | Point Number:- 759 |  |
| Obs.Time:- 10 : 05 : 00 |  | Accuracy:- 0.018 m |  |
| Coordinates | E | N | H |
| Registered | 169213.18 | 148845.37 | 845.65 |
| Observe | 169213.00 | 148845.467 | 846.04 |
| WGS1984 | 35° 12' 6.71679" E | 31° 55' 57.38424" N | 866.424 |
| Description | This trig is located in the village of (Surda) on the rock |  |  |

Table (5-75): Details of Trig (657L)

| Date:- 22/8/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 657L |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Ramallah VRS |  | Point Number:- 761 |  |
| Obs.Time:- 10 : 27 : 00 |  | Accuracy:- 0.019 m |  |
| Coordinates | E | N | H |
| Registered | 166751.52 | 147794.39 | 745.53 |
| Observe | 166751.76 | 147794.26 | 746.75 |
| WGS1984 | 35° 10' 33.02870" E | 31° 55' 23.24510" N | 767.14 |
| Description | This trig is located in the village of (Ramallah) on the rock |  |  |

Table (5-76): Details of Trig (986S)

| Date:- 22/8/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 986S |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Ramallah VRS |  | Point Number:- 763 |  |
| Obs.Time:- 10 : 45 : 00 |  | Accuracy:- 0.016 m |  |
| Coordinates | E | N | H |
| Registered | 170186.38 | 146463.99 | 871.41 |
| Observe | 170186.245 | 146463.937 | 871.90 |
| WGS1984 | 35° 12' 43.76147" E | 31° 54' 40.06583" N | 892.278 |
| Description | This trig is located in the village of (Al Bira) on the rock |  |  |

Table (5-77): Details of Trig (121Y)

| Date:- 22/8/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 121Y |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Ramallah VRS |  | Point Number:- 757 |  |
| Obs.Time:- 09 : 40 : 00 |  | Accuracy:- 0.019 m |  |
| Coordinates | E | N | H |
| Registered | 171841.27 | 152650.15 | 713.10 |
| Observe | 171841.13 | 152650.17 | 713.61 |
| WGS1984 | 35° 13' 46.80306" E | 31° 58' 0.89675" N | 733.99 |
| Description | This trig is located in the village of (Ein Siniya) on the rock |  |  |

Table (5-78): Details of Trig (480T)

| Date:- 22/8/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 480T |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Ramallah VRS |  | Point Number:- 765 |  |
| Obs.Time:- 11 : 12 : 00 |  | Accuracy:- 0.015 m |  |
| Coordinates | E | N | H |
| Registered | 168216.57 | 143998.5 | 848.11 |
| Observe | 168216.369 | 143998.476 | 848.58 |
| WGS1984 | 35° 11' 28.79757" E | 31° 53' 20.02144" N | 868.958 |
| Description | This trig is located in the village of (Beituniya) on the rock |  |  |

Table (5-79): Details of Trig (2K)

| Date:- 22/8/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 2K |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Ramallah VRS |  | Point Number:- 772 |  |
| Obs.Time:- 01 : 35 : 00 |  | Accuracy:- 0.021 m |  |
| Coordinates | E | N | H |
| Registered | 169092.08 | 141297.74 | 810.02 |
| Observe | 169092.05 | 141297.88 | 810.51 |
| WGS1984 | 35° 12' 2.12272" E | 31° 51' 52.34671" N | 830.87 |
| Description | This trig is located in the village of (Qalandiya) on the rock |  |  |

Table (5-80): Details of Trig (215F)

| Date:- 22/8/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 215F |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Ramallah VRS |  | Point Number:- 775 |  |
| Obs.Time:- 04 : 17 : 00 |  | Accuracy:- 0.012 m |  |
| Coordinates | E | N | H |
| Registered | 178483.62 | 157845.00 | 791.77 |
| Observe | 178483.57 | 157845.13 | 792.24 |
| WGS1984 | 35° 17' 59.94743" E | 32° 0' 49.43148" N | 812.60 |
| Description | This trig is located in the village of (Abu Falah) on the rock |  |  |

Table (5-81): Details of Trig (189F)

| Date:- 22/8/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 189F |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Ramallah VRS |  | Point Number:- 751 |  |
| Obs.Time:- 07:19 : 00 |  | Accuracy:- 0.018 m |  |
| Coordinates | E | N | H |
| Registered | 160852.72 | 162614.21 | 477.84 |
| Observe | 160852.128 | 162613.61 | 477.98 |
| WGS1984 | 35° 6' 47.97912" E | 32° 3' 24.25162" N | 498.43 |
| Description | This trig is located in the village of (Kafr Ein) on the rock |  |  |

Table (5-82): Details of Trig (974T)

| Date:- 22/8/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 974T |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Ramallah VRS |  | Point Number:- 753 |  |
| Obs.Time:- 08 : 29 : 00 |  | Accuracy:- 0.016 m |  |
| Coordinates | E | N | H |
| Registered | 166120.91 | 154854.11 | 660.89 |
| Observe | 166120.814 | 154854.02 | 661.42 |
| WGS1984 | 35° 10' 8.92356" E | 31° 59' 12.43650" N | 681.832 |
| Description | This trig is located in the village of (Burham) on the rock |  |  |

Table (5-83): Details of Trig (220J)

| Date:- 22/8/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 220J |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Ramallah VRS |  | Point Number:- 769 |  |
| Obs.Time:- 12 : 40 : 00 |  | Accuracy:- 0.020 m |  |
| Coordinates | E | N | H |
| Registered | 157403.96 | 150943.13 | 423.78 |
| Observe | 157403.951 | 150942.834 | 424.24 |
| WGS1984 | 35° 4' 37.05571" E | 31° 57' 5.24605" N | 444.68 |
| Description | This trig is located in the village of (Bani Harith) on the rock |  |  |

Table (5-84): Details of Trig (218J)

| Date:- 22/8/2014 |  | Obs.Team:- Saleh natour, Abedalhafez almuhtaseb, Ala'a rjob, Samer Serhan |  |
| --- | --- | --- | --- |
| Point ID:- 218J |  | Instrument:- Leica Viva GS08 |  |
| File Name:- Ramallah VRS |  | Point Number:- 768 |  |
| Obs.Time:- 12 : 21 : 00 |  | Accuracy:- 0.020 m |  |
| Coordinates | E | N | H |
| Registered | 157300.27 | 149898.38 | 397.28 |
| Observe | 157300.245 | 149897.999 | 397.77 |
| WGS1984 | 35° 4' 33.15409" E | 31° 56' 31.32115" N | 418.205 |
| Description | This trig is located in the village of (Bani Harith) on the rock |  |  |